TENDER SPECIFICATIONS

The “AE-PLC-APS” unit mainly consists of:

Table with four wheels with brakes mounted with aluminum profiles.

Ten pieces made of aluminum and plastic:

Five pieces with hole.

Five pieces without hole.

Unit wired and programmed for the correct operation of the system.

Light stack to indicate the state of the workstation:

Green, yellow and red led lights.

360º visualization.

Non-colored lens to avoid wrong interpretation with the lights turned off.

Electrical terminal block to connect the individually identified inputs, outputs and power signals of the unit.

Control panel:

Differential magneto-thermal switch.

Emergency pushbutton.

Start/Stop switch.

Light indicator to show the state of the workstation

Reset push-button.

Manual/automatic switch.

Next step push-button.

Two Ethernet/IP RJ45 connectors.

Power switch.

Power supply of 24 VDC.

Air treatment unit:

Filter-regulator and water trap.

Manometer with double scale indicator.

Shut-off valve.

Purge valve.

Piece feeder module:

Vertical storage for pieces.

Double acting cylinder:

Diameter: 20 mm.

Stroke: 150 mm.

Two adjustable air flow valves with bypass.

Pneumatic monostable 5/2 solenoid valve.

Capacitive sensor.

Two reed switches.

Piece verifier module:

Double acting cylinder:

Diameter: 40 mm.

Stroke: 125 mm.

Two adjustable air flow valves with bypass.

Pneumatic monostable 5/2 solenoid valve.

Capacitive sensor.

Three reed effect.

Piece displacement module:

Double acting cylinder:

Diameter: 40 mm.

Stroke: 125 mm.

Two adjustable air flow valves with bypass and piloted check valve.

Two Pneumatic monostable 3/2 solenoid valves.

Piece classification module:

Double acting cylinder:

Normally retracted rod.

Diameter: 32 mm.

Stroke: 150 mm.

Adjustable air flow valve with bypass.

Pneumatic monostable 3/2 solenoid valve.

Inductive effect sensor.

Fault generation module:

Attached to the frame of the unit.

Generates faults in the operation of different elements of the unit using switches.

Cables and accessories for a correct operation.

Manuals: This unit is supplied with the following manuals: Required services, Installation, Starting-up, Safety regulations, Maintenance and Practical exercises.

Dimensions: 1200 x 1100 x 1500 mm approx. (39.36 x 23.62 x 59.05 inches approx.). Weight: 85 kg approx. (187 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Introduction to flexible manufacturing system; identification of unit components

2.-  Configuration of a pneumatic application

3.-  Manual pieces feeding process with verifying function.

4.-  Automatic pieces feeding process with verifying function.

5.-  Optimizing the automatic pieces feeding process with verifying function.

Optional

AE-PLC-APS/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.