TENDER SPECIFICATIONS

The “AE-PLC-FMS6” unit mainly consists of:

• AE-PLC-A. Part Feeding Workstation.

Its function is to supply the base piece to the system. The “AE-PLC-A” includes a place where the pieces are stored before being supplied one by one by a pneumatic cylinder. Then, the position of the piece will be verified in order to activate the mechanism that checks whether the shape of the piece is correct. Afterwards, the piece is displaced from the station to the Linear Transport Workstation “AE-PLC-T” by a vacuum system consisting of suction pads mounted on a manipulator arm to continue with the manufacturing process.

Specifications:

Mounted with aluminum profiles.

10 pieces made of aluminum.

Vertical storage for pieces.

Table with four wheels with brake.

Control panel:

Emergency pushbutton.

Start/Stop pushbuttons.

Manual/automatic switch.

Light indicators for signaling.

Pneumatic circuit:

Air treatment unit:

Filter-regulator with water trap.

Manometer with double scale indicator.

Shut-off valve.

Cylindrical simple effect pneumatic actuator:

Material: stainless steel.

Normally retracted rod.

Diameter: 20 mm.

Stroke of 100 mm.

Cylindrical pneumatic simple effect actuator:

Material: stainless steel.

Normally retracted rod.

Diameter: 10 mm.

Stroke of 50 mm.

2 profile pneumatic double effect actuators:

Material: stainless steel and aluminum.

Normally retracted rod.

Stroke of 200 mm.

Profile pneumatic double effect actuator:

Material: stainless steel and aluminum.

Stroke of 100 mm.

2 pneumatic monostable 3/2 solenoid valves.

3 pneumatic bistable 5/2 solenoid valves.

Individually identified tubes and push in fittings.

Vacuum circuit:

Venturi effect vacuum ejectors.

4 telescopic flat suction pads made of NBR rubber mounted on an aluminum plate.

Vacuum switch.

Pneumatic monostable 3/2 solenoid valve.

Sensors:

2 inductive effect sensors to check the position of the piece.

8 reed effect limit switches to check the position of the pneumatic actuators.

Fault generation module:

Attached to the frame of the unit.

Generates faults in the operation of different elements of the unit using switches.

• AE-PLC-CF. Sorting Workstation.

Mounted with aluminum profiles.

Electrical panel:

Differential magneto-thermal switch.

Power supply of 24 Vdc.

Control panel:

Mushroom head emergency stop push button switch.

Start and Stop pushbuttons.

Switch for manual or automatic operation.

Light indicator.

Led stack light.

Terminal block to connect the individually identified inputs and outputs of the unit.

Pneumatic circuit:

Air treatment unit:

Filter-regulator with water trap.

Manometer with double scale indicator.

Shut-off valve.

Single acting spring return pneumatic actuator.

2 Double acting pneumatic actuators.

3 Pneumatic monostable 5/2 solenoid valves.

Sensors:

4 IR beam detectors.

Reflective optical sensor.

Inductive effect sensor.

8 reed effect limit switches.

Fault generation module:

Box with lock with key.

Allows to generate 20 different faults.

Each fault is generated through toggle switches.

Required elements (Required: “SAC” or “SU-P”. Of the rest of the elements, additionally, required at least one) (Not included):

• SAC. Silent Air Compressor Unit.

Designed to work with the EDIBON units.

Single-phase motor with 340 W.

Low noise level with 40 dB of maximum.

Boiler capacity: 9 l.

Maximum pressure: 8 bar or 0.8 MPa.

Maximum air flow: 50 l/min.

Safety relief valve.

Start/stop switch.

Double scale manometer psi and bar with a range from 0 – 12 bar to measure the air pressure before the air filter regulator.

Air filter regulator (FR):

Double scale manometer psi and bar with a range from 0 – 12 bar to measure the air pressure  after the air filter regulator.

Air filter with drain.

Air pressure regulator from 0 – 8 bar.

6 mm quick release connector of 6mm for pneumatic flexible tubes.

Oil level indicator:

The oil necessary to work is included with the unit.

Include the connector types to work with pneumatic applications of EDIBON.

• SU-P. Large Pneumatic Supply Unit.

• AE-PLC-PAN-UB. PANASONIC PLC Base Unit.

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CPU-PAN. PANASONIC CPU Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Overcurrent protection with fuse.

PLC device, Panasonic FP7:

High processing speed: 11 ns per basic instruction (step).

Programming capacity: 120k basic instructions (step).

Data logging capacity: 256k words.

Independent memory for comments: 3 MB.

Compatible with SD / SDHC memory cards up to 32 GB.

Sixteen digital inputs with voltage levels 0 V and 24 V.

Sixteen relay type digital outputs with voltage levels 0 V and 24 V.

Expansion module for extra channels of digital inputs and outputs.

Web server function:

HTML web server included in the PLC.

Up to sixteen sessions at the same time.

Compatible with the most common search motors.

Connector for the digital input and output signals.

Connector for the Ethernet switch module.

• N-SWT-4. 4 Ports Ethernet Switch Module.

Supply voltage (single-phase): 230 VAC.

Compact switch module:

Four Ethernet ports.

Protection fuse (at supply voltage height).

• AE-PLC-SIE-UB. SIEMENS PLC Base Unit.

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CPU-SIE. SIEMENS CPU Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Overcurrent protection with fuse.

PLC device, SIEMENS S7 – 1200:

High processing capacity: 64 bit calculation.

Program/data memory: 100 kB.

Load memory: 4 MB expandable with a Siemens memory card.

CPU processing times for bit operations: 0.085µs/instruction.

Counting frequency: 100 kHz.

Twenty-two digital inputs with voltage levels 0 V and 24 V.

Eighteen relay type digital outputs with voltage levels 0 V and 24 V.

Two analog inputs of 10 bits with voltage range from 0 V to 10 V.

Expansion module for extra digital inputs and outputs.

Web server function:

HTML web server included in the PLC.

Compatible with most common search engines.

Connector for the digital input and output signals.

Connector to the Profinet/Ethernet switch module.

• N-SWT-4. 4 Ports Ethernet Switch Module.

Supply voltage (single-phase): 230 VAC.

Compact switch module:

Four Ethernet ports.

Protection fuse (at supply voltage height).

• AE-PLC-AB-UB. ALLEN BRADLEY PLC Base Unit.

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CPU-AB. ALLEN BRADLEY CPU Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Overcurrent protection with fuse.

PLC device Allen Bradley CompactLogix 5370 L1 controller:

High speed and high performance control.

Controller tasks: 32.

Program per task: 1000.

Available user memory: 384 KB.

SD card inbox memory: 1 GB.

Two Ethernet connectors with a communication rate of 10/100 Mbps.

USB connection for firmware updates.

Sixteen digital inputs with voltage levels 0 V and 24 V.

Sixteen relay type digital outputs with voltage levels 0 V and 24 V.

Connector for the digital input and output signals.

Connector for the Ethernet switch module.

• N-SWT-4. 4 Ports Ethernet Switch Module.

Supply voltage (single-phase): 230 VAC.

Compact switch module:

Four Ethernet ports.

Protection fuse (at supply voltage height).

• AE-PLC-MIT-UB. MITSUBISHI PLC Base Unit.

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CPU-MIT. MITSUBISHI CPU Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Overcurrent protection with fuse.

PLC device, Mitsubishi FX5U:

High processing speed: 34 ns per basic instruction (step) and 34 ns or more for math instructions.

Programming capacity: 64000 steps.

SD memory card compatible.

Sixteen digital inputs with voltage levels 0 V and 24 V.

Sixteen relay type digital outputs with voltage levels 0 V and 24 V.

Two analog inputs of 12 bits with voltage range from 0 V to 10 V.

One analog output of 12 bits with voltage range from 0 V to 10 V.

Connector for the digital input and output signals.

Connector to the Ethernet switch module.

• N-SWT-4. 4 Ports Ethernet Switch Module.

Supply voltage (single-phase): 230 VAC.

Compact switch module:

Four Ethernet ports.

Protection fuse (at supply voltage height).

• AE-PLC-OMR-UB. OMRON PLC Base Unit.

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CPU-OMR. OMRON CPU Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Overcurrent protection with fuse.

PLC device, OMRON NX1:

High processing speed: 3.3 ns per basic instruction (step) and 70 ns or more for math instruction (for LONG REAL data).

Programming capacity: 1.5 MB.

SD / SDHC memory card compatible.

Eighteen digital inputs with voltage levels 0 V and 24 V.

Fourteen relay type digital outputs with voltage levels 0 V and 24 V.

Connector for the digital input and output signals.

Connector for the Ethernet switch module.

• N-SWT-4. 4 Ports Ethernet Switch Module.

Supply voltage (single-phase): 230 VAC.

Compact switch module:

Four Ethernet ports.

Protection fuse (at supply voltage height).

Additional recommended elements (Not included):

• AE-AS. Automation System Simulation Software.

2D and 3D systems simulations.

Configurable simulation speed with the modes “normal simulation”, “slow motion simulation”, “step-by-step simulation” and “pause”.

3D editor to import pieces made with formats compatible with most 3D design programs (.STEP, .STL and .IGES). Capacity to generate 2D and 3D animations associated to the results of the simulation the user is working with.

Capacity for simulating the following systems:

•Hydraulic and electrohydraulic: according to ISO 1219-1 and 1219-2 standards, with an extensive library of hydraulic and electrohydraulic components with its standardized symbol.

•Pneumatics and electro-pneumatics: with an extensive library of pneumatic, electro-pneumatic and pneumatic logic components.

Capacity to modify the most important parameters of each hydraulic and pneumatic component, such as: efficiency curves, external loads, leaks, viscosity, thermal characteristics, etc.

•Digital electronics: with an extensive library of standard electronic components (logic gates, amplifiers, transistors, displays, multiplexers, etc.).

•Electrical Single-Line Diagram: with a library that enables to create diagrams for all levels of voltage usually employed in power generation, transport and distribution networks.

•Electrical engineering: with a library that contains a great amount of components to create simple and complex electrical circuits. The models of the components included are generic and real and belong to several manufacturers.

All the libraries include the components and its standardized symbol.

Capacity to program with the following languages:

•GRAFCET: allows encapsulated stages for a better organization of the programmed control structures.

•Block Diagram: blocks included are preset but they can be completely configured by the user.

•Ladder: includes three libraries to program automata from Siemens, Allen Bradley and those fulfilling the IEC61131-3 standard, allowing the PLC to program directly. It also allows for creating and simulating the PLC program in the automated system simulated by the software.

•Digital logic: with an extensive library of logic gates and components configured by the user.

•Function blocks with configurable structured text.

Direct programming in the PLCs from the manufacturers Siemens, Allen Bradley and those fulfilling the IEC61131-3 standard of the programs simulated in the software.

Supervision, control and simulation of the manufacturing process of each station and the complete assembly by a SCADA system.

Communication with the PLCs of the unit is performed via OPC protocol.

Includes the 3D simulation of the automation system with the control panel and the visualization of the alarms generated by the system.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 2500 x 900 x 1500 mm approx. (98.42 x 35.43 x 59.05 inches approx.). - Weight: 240 Kg approx. (529 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

Practical possibilities to be done with the Flexible Manufacturing System for Feeding and Classification (AE-PLC-FMS6):

1.- Introduction to flexible manufacturing system (FMS).

2.- Introduction to pneumatics and electro-pneumatics.

3.- Introduction to vacuum technology.

4.- Study of the sensor detection.

5.- Testing the digital inputs and outputs of the automatic system through a PLC.

6.- Modify manufacturing parameters through the PLC.

7.- Configuration of a pneumatic application.

8.- Introduction to the Human-machine interface systems (HMI).

9.- Study of an automatic control of an industrial system.

10.- Control of the flexible manufacturing system through a central PLC in an Ethernet network with local PLC in each workstation.

11.- Manage the flexible manufacturing system through the HMI device.

Practical possibilities to be done with the workstation alone:

- Part Feeding Workstation (AE-PLC-A).

12.- Manual control of a Feeding system.

13.- Automatic control of a Feeding system.

14.- Change the parameters of the Feeding process.

15.- Optimize the process time.

16.- Study and analysis of the faults inserted in the process with the fault generation module.

- Sorting Workstation (AE-PLC-CF).

17.- Manual control of a sorting system.

18.- Automatic control of a sorting system.

19.- Change the parameters of the sorting process.

20.- Optimize the process time.

21.- Study and analysis of the faults inserted in the process with the fault generation module.

Practical possibilities to be done with the optional software Automation Systems Simulation Software (AE-AS):

22.- Introduction to the SCADA control system.

23.- OPC server system with Ethernet.

24.- SCADA control of an automatic industrial system through PC.

Optional

AE-PLC-FMS6/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.