TENDER SPECIFICATIONS (for main items)

1 AEDC/A. Unit:

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All metal elements are made of s316 stainless steel, including the supports.

Seals are suitable for food and can be used at temperatures of up to 300°C.

Feeding tank with vacuum connection for oil de-aeration.

Coil in the feeding tank, for preheating the oil, thus facilitating de-aeration.

Domestic hot water heater for exchange in the feeding tank coil. Water supply regulation valve to the coil, according to the desired temperature.

Feed pump to the de-aerated oil system with flow rate control, range: 8 – 20 kg/h.

Three plate heat exchangers for heating the vegetable oil to be treated:

Two counterflow exchangers with pre-deodorized oil.

Exchanger for counterflow heating the oil to be treated with thermal oil.

20 plates.

Si02-coated, suitable for food.

Distillation/desaturation column based on a column with structured packing, heat insulation, vacuum connection and equipped with:

Oil inlet distributor placed in the edible oil inlet to the column from the top. It ensures an even oil distribution when introduced into the column.

Three-tiered plate trays to promote contact between the liquid and vapor phases and to improve distillation performance.

Four hoods are the base of the stripping and allow the steam to rise through the column and a central hole, through which the oil falls to the surface of the first or upper plate.

Flexipac 500Y column packing. It is distributed in plates positioned at a 90° angle to each other, so that no preferential paths are ever formed and the even distribution of the fluid along the column is guaranteed.

Steam distributor, used as a carrier gas: five inlets for the carrier steam. There is an inlet on top of the contact plates, three others located inside the plates (one inside each plate) and the last one at the bottom of the column below the contact plates.

There are also 4 coils at the column head to help condense the extracted fatty acids. These coils can be connected the mains water so that a constant temperature is maintained.

Two demisters.

Coil at the bottom of the thermal oil circulation column to maintain the temperature in the column.

Pump with an adjustable outlet flow rate from the deodorizer.

Cooling tank for the treated oil, consisting of a cooling water coil with a transparent cover.

Thermal oil circuit:

Edible thermal oil boiler providing the energy needed to heat the oil entering the column.

Maximum thermal oil temperature: 300 °C.

Edible thermal oil.

Pump with thermal oil flow control.

Two proportional valves to control the thermal oil flow rates to the heat exchangers and/or the deodorizing column coil.

Fatty acid recirculation circuit:

Edible thermal oil boiler providing the energy needed to heat the oil entering the column.

Maximum thermal oil temperature: 300 °C.

Edible thermal oil.

Pump with thermal oil flow control.

Two proportional valves to control the thermal oil flow rates to the heat exchangers and/or the deodorizing column coil.

Fatty acid recirculation pump, to prevent them from solidifying and to maintain the temperature at the column head.

Direct extraction via manual valve or redirecting to a cooling tank up to 60 °C:

Cooling tank for cooling water coil to prevent fatty acids from condensing and thickening.

Auxiliary tank with Nitrogen intake to collect samples at any time.

Vacuum circuit:

Vacuum pump.

Vacuum trap, which ensures that the vapors from the column do not reach the pump.

It includes a coil that is part of a condensing unit, and allows water vapor from the column to be condensed, reducing its volume and enabling the vacuum pump to keep the vacuum in the system constantly.

Condensing unit filled with R-404 refrigerant.

Carrier steam generator. 3kW power.

Nitrogen circuit.

Temperature and pressure sensors at key points in the system.

Vegetable oil flow sensors.

Level switch.

Relief solenoid valves and all necessary measurements, such as alarms, thermostat, and pressure switch, etc., to work safely.

The complete unit includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Required elements (Not included):

- TGV. Steam Generator (3 kW).

2 AEDC/A/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time PID control with flexibility of modifications from the computer keyboard of the PID parameters, at any moment during the process.

Real time PID control for parameters involved in the process simultaneously.

Proportional control, integral control and derivative control, based on the real PID mathematical formula, by changing the values, at any time, of the three control constants (proportional, integral and derivative constants).

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 AEDC/A/CCSOF. PID Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control. PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*References 1 to 6 are the main items: AEDC/A + AEDC/A/CIB + DAB + AEDC/A/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Study of the operation of a small-scale model of the industrial continuous deodorization process.

2.- Study of the operation of a small-scale model of the industrial discontinuous deodorization process.

3.- Study of a structured packing column.

4.- Study of the fatty acid removal process of various vegetable oils.

5.- Study of the variation in temperature of the deodorization process.

6.- Study of the variation in pressure of the deodorization process.

7.- Study of the variation in the amount of direct steam added to the deodorization process.

8.- Study of the variation in the total processing time of the deodorization process.

9.- Study of the effect of the recirculation of the fatty acids obtained in the process.

Additional practical possibilities:

10.-Sensors calibration.

11.-Study of the impact of the type of packing in the fatty acid removal process.

Other possibilities to be done with this Unit:

12.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

13.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

12.-The Computer Control System with SCADA and PID Control allow a real industrial simulation.

13.-This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

14.-This unit can be used for doing applied research.

15.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

16.-Control of the AEDC/A unit process through the control interface box without the computer.

17.-Visualization of all the sensors values used in the AEDC/A unit process.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Technical and Vocational Education configuration

7 AEDC/A/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

8 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA and PID Control integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA and PID Control, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.