TENDER SPECIFICATIONS

The application “AEL-AD14” includes the following elements:

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single–phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose with connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-POR04. Video Camera Module.

Input voltage: two terminals of 15 VDC.

Video camera.

Main hall entrance door button.

Adjust of the kind of bell and sound.

Two output terminal L (data and audio control).

One relay output NO (2 terminals).

• N-POR05. Phone/Monitor Module.

Input voltage: 2 terminals of 15 VDC.

One input terminal L (data and audio control).

• N-POR06. Door Opener Module.

Input voltage: two terminals of 12 VAC.

It can be connected to the control module.

Main hall entrance door lock.

• N-POR08. Video-Interphone Power Supply Module.

Input voltage: 2 terminals of 230 VAC (L and N).

Input frequency: 50/60 Hz.

Output voltage: 2 terminals of 15 VDC.

Output current: 1 A.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied whit the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Verification of the operation of the main power supply.

2.-  Checking the working of the video-interphone power supply.

3.-  Wiring the supplying telephone, video entry phone and door opener.

4.-  Real application of an audio and video entry phone system.

5.-  Checking the communication between video interphone and telephone.

6.-  Verification of the operation of the door opener with the telephone.

Optional

AEL-AD14/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.