TENDER SPECIFICATIONS

The application “AEL-AD25A” includes the following elements:

The application “AEL-AD22” includes the following elements:

• N-ALI02. Domestic Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose with connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ALI03. Variable Auxiliary Voltage Supply Module (24 Vac).

Supply voltage (single-phase): 230 VAC, 1PH + N.

Output voltage:

Single-phase 24 VAC / 12 VAC.

24 VDC.

0 – 24 VDC through potentiometer.

• N-DET01. Flood Detector Module.

Voltage supply: 230 VAC.

Frequency: 50 / 60 Hz.

Power: 5 W.

Alarm LED.

One input for probe.

Two alarm outputs.

Connectable to the solenoid valve.

Status LED.

• N-DET03. Built-in Power Supply Module (gas, flood detector).

Voltage supply: 230 VAC.

Output voltage: 12 VAC.

Ground connection.

Service LED.

• N-DET04. Built-in Flood Detector Module.

Voltage supply: 12 VAC.

One input for probe.

Two alarm outputs.

Alarm and status LED.

Acoustic alarm.

• N-SEL03. Three Pilots Light Module.

Voltage supply: 24 VAC.

Colors: red, yellow and green.

• N-SEL21. Indoor Siren Module.

Volume configuration through voltage: 12 – 24 VDC.

87 – 112 dB.

12 – 30 mA.

32 tones.

The application “AEL-AD23” includes the following elements:

• N-ALI02. Domestic Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose with connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ALI03. Variable Auxiliary Voltage Supply Module (24 Vac).

Supply voltage (single-phase): 230 VAC, 1PH + N.

Output voltage:

Single-phase 24 VAC / 12 VAC.

24 VDC.

0 – 24 VDC through potentiometer.

• N-DET13. Wireless RF Intrusion Detector Module.

Vision angle: 90°.

Frequency: 433.92 MHz.

Voltage supply: battery of 3.6 VDC.

Max area: 12 x 12 m.

Status LED.

• N-DET14. Wireless RF Panic Button Module..

Frequency: 433.92 MHz.

Voltage supply: battery of 12 VDC.

Battery status LED.

Working temperature: 0 – 49 °C.

Dimensions: 32 x 53 x 17 mm.

• N-DET15. Wireless RF Single Channel Receiver Module.

Communication with 10 transmitter modules.

Frequency: 433.92 MHz.

Voltage supply: 12 VDC.

One alarm output.

One tamper output.

Dimensions: 110 x 63 x 25 mm.

• N-SEL01. Luminous Signaling Beacons Module.

Two pilots, yellow and red.

Voltage supply: 230 VAC.

• N-TIM05. Bell + Buzzer Module.

Voltage supply: 230 VAC.

Bell sound: 70 dB.

Buzzer sound: 80 dB.

The application “AEL-AD30” includes the following elements:

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose with connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ALI03. Variable Auxiliary Voltage Supply Module (24 Vac).

Supply voltage (single-phase): 230 VAC, 1PH + N.

Output voltage:

Single-phase 24 VAC / 12 VAC.

24 VDC.

0 – 24 VDC through potentiometer.

• N-DET02. Gas Detector Module.

Voltage supply: 12 VAC.

Two alarm outputs.

Acoustic alarm indicator.

Status indicator light.

Test button.

• N-DET03. Built-in Power Supply Module (gas, flood detector).

Voltage supply: 230 VAC.

Output voltage: 12 VAC.

Ground connection.

Service LED.

• N-DET12. Gas Solenoid Valve Module.

Voltage supply: 230 VAC.

Power: 14 W.

Ground connection.

Automatic reset.

Maximum pressure: 500 mBar.

Closing time: 0.1s.

• N-DET36. Home Use Smoke Detector Module.

Voltage supply: 230 VAC.

Two alarm outputs.

Alarm sound and status LED.

The application “AEL-AD40” includes the following elements:

• N-ALI02. Domestic Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose with connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CTR01. Basic Control Module.

Two voltage inputs: 230 VAC.

Frequency: 50 Hz.

Five inputs.

Five outputs per relay (20 outputs).

One telephone connection input.

One telephone connection output.

• N-VAR05. Tone Dialing Telephone Module.

Standard telephone with tones.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied whit the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

AEL-AD25A:

AEL-AD22:  - Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.)  - Weight: 60 Kg approx. (132 pounds approx.)

AEL-AD23:  - Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

AEL-AD30: - Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

AEL-AD40: - Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.)  - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Checking and measuring on the main power supply.

2.- Checking the working of the auxiliary power supply.

3.- Checking the working of pilot-lights by connecting as visual alarms.

4.- Checking the working of the built-in power supply.

5.- Checking the operation of the flooding detector.

6.- Checking the operation of the built-in flood detector.

7.- Checking the operation of the siren, used as visual and acoustic alarm.

8.- Simulate floods and evacuations with the water tank and the electrovalve.

9.- Checking the working of the bell + buzzer module using as acoustic alarm.

10.- Checking the working of the light signaling beacons module, using as visual alarm.

11.- Checking the working of  the wireless emergency pushbutton.

12.- Use of the alarms with the wireless intrusion detector.

13.- Control of gas electrovalve through the smoke sensor.

14.- Real applications of a gas leakages control system.

15.- Configuration of the Control Module.

16.- Control of the flooding detector through the Control Module.

17.- Control of the gas detector through the Control Module.

18.- Control of the motion detector through the Control Module.

Optional

AEL-AD25A/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.