TENDER SPECIFICATIONS

The “AEL-AE1A” application includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-CONT. Three- phase Commutable Capacitive Load Module.

Nominal voltage: 400 VAC.

Capacitance: 3 x 7 µF.

Manual switch to turn activate/deactivate capacitors.

Fuses: 3 x 4 A.

Terminals:

Four inputs terminals (3PH + N).

Four outputs terminals (3PH + N).

• N-REFT. Three-phase Commutable Resistor Load Module.

Nominal voltage: 400 VAC.

Resistance: 3 x 150 Ohm.

Nominal current: 1.6 A.

Manual switch to turn activate/deactivate resistors.

Fuses: 3 x 2 A.

Terminals:

Four input terminals (3PH + N).

Four output terminals (3PH + N).

• N-INDT. Three-phase Commutable Inductive Load Module.

Nominal voltage: 400 VAC.

Inductance: 1.4 H.

Nominal current: 1.6 A.

Manual switch to turn activate/deactivate inductances.

Fuses: 3 x 2 A.

Terminals:

Four input terminals (3PH + N).

Four output terminals (3PH + N).

• N-EALD. Electrical Network Analyzer Module with Oscilloscope and Data Acquisition (2 units).

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-AE1. Transmission Lines Simulation Module.

Return line of 10 Ohm.

The resistive part: two resistance values of 15 Ohm and 33 Ohm with the next possible connections:

Only one:

15 Ohm.

33 Ohm.

Series:

48 Ohm.

Parallel:

10.31 Ohm.

The inductive part:

Possibilities:

33 mH, 78 mH, 140 mH, 193 mH, 236 mH.

The capacitance between conductors:

0.5 µF and 1 µF for each conductor.

The capacitance between conductor and ground:

1 µF and 2 µF, also at the beginning and at the end of the line.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 1600 x 550 x 2000 mm approx. (62.99 x 21.65 x 78.74 inches approx.) - Weight: 100 Kg approx. (220 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Studying of the energy power losses on the transport lines.

2.- Theoretical calculation of  drops voltage on the transport  lines and comparison y networs analyzer values.

3.- Voltage increases on open-circuit lines.

4.- Studying the unbalanced power systems.

5.- Studying the balanced power systems.

6.- Directly measure of energy losses on the  electrical grid and comparison with theory  calculus.

7.- Variation of type of loads and visualizing the power factor of the grid.

8.- Study of power factor compensation and the effects in the energy losses.

- Several other exercises can be done and designed by the user.

Optional

AEL-AE1A/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.