TENDER SPECIFICATIONS

The application “AEL-AI13-E” includes the following elements:

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push button.

• N-ALI10. Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Output voltage:

15 – 0 – 15 VAC.

24 VAC.

24 VDC.

0 – 24 VDC (adjustable).

• N-LAM09. Fluorescent Lamp Module.

Nominal voltage: 230 VAC (PH + N)

Switch ON / OFF.

Fluorescent lamp.

Nominal power: 4 W.

Supply terminals (LN).

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• N-REL50. Relays Module.

Nominal voltage: 24 V.

Two relays.

Two normally open contact (NO).

Two normally close contact (NC).

• N-LAM26. Lighting Module.

Four 24 V red lamps.

Two switches.

Two simple switches.

One crossover-switch.

One push-button.

One buzzer.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 920 mm approx.(25.19 x 12.59 x 36.22 inches approx.) - Weight: 45 Kg approx. (99 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Checking the working of the main power supply.

2.- Lamp controlled by a switch or a push-buton.

3.- Lamp controlled form three points.

4.- Lamp controlled by relays.

5.- Acoustic indicator circuit.

6.- Switching a fluorescent lamp.

7.- Parallel and series association of different elements (lamps, switches, etc.).

Optional

AEL-AI13-E/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.