TENDER SPECIFICATIONS

The application “AEL-AI13” includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ALI10. Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

Output voltage:

15 – 0 – 15 VAC.

24 VAC.

24 VDC.

0 – 24 VDC (adjustable).

• N-CAR30. Inductances Module.

Nominal voltage: 24 V.

Inductances:

Two 1 mH inductances.

One 2.2 mH inductance.

Two 3.3 mH inductances.

Two 10 mH inductances.

Two 22 mH inductances.

• N-CAR31. Capacitors Module.

Nominal voltage: 24 V.

Capacitors:

One 47 µF non-polarised capacitor.

One 68 µF non-polarised capacitor.

Two 100 µF non-polarised capacitors.

One 220 µF non-polarised capacitor.

One 330 µF non-polarised capacitor.

One 470 µF non-polarised capacitor.

One 220 µF polarised capacitor.

One 470 µF polarised capacitor.

• N-CAR32. Rectifier Diodes Module.

Nominal voltage: 230 V.

Rectifiers: 6 x 40 A rectifier diodes.

• N-CAR33: Resistive Components Module.

Nominal voltage: 24 V.

Resistors:

Three 47 k Ohm resistors.

Three 51 k Ohm resistors.

Two 82 k Ohm resistors.

Two 100 k Ohm resistors.

One 120 k Ohm resistors.

Three 150 k Ohm resistors.

One 10 k Ohm linear 5 W wound potentiometer.

One 50 k Ohm linear potentiometer.

One 10 k Ohm logarithmic potentiometer.

One 50 k Ohm logarithmic potentiometer.

• N-LAM26. Lighting Module.

Four 24 V red lamps.

Two switches.

Two simple switches.

One crossover-switch.

One push-button.

One buzzer.

• N-LAM09. Fluorescent Lamp Module.

Nominal voltage: 230 VAC (PH + N)

Switch ON / OFF.

Fluorescent lamp.

Nominal power: 4 W.

Supply terminals (LN).

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• N-REL50. Relays Module.

Nominal voltage: 24 V.

Two relays.

Two normally open contact (NO).

Two normally close contact (NC).

• TRA28. Teaching Three-phase Transformer.

Nominal supply voltage: 400 and 230 VAC (3PH).

Nominal output voltage: 3 x (3 x 73 VAC).

Nominal power: 500 VA.

Free configuration (delta, star and zig-zag).

• VAR17. Dismantled Transformer Kit.

Nominal supply voltage: 230 V.

Nominal output voltage: 110 V.

Nominal power: 185 VA.

Four removable elements.

Coil of 2000 turns (0.8 A).

Coil of 1000 turns (1.6 A).

• VAR18. Electrostatic Kit.

PVC bar.

PVC tube.

Nylon bar.

Aluminium bar.

Homemade electroscope:

PMMA tube with cover, metal hook, aluminium ball and silver foil strip folded in half.

Two acetate strips.

Rabbit fur.

• N-VAR16. Electromagnetism Kit with a DC Motor/Generator Set.

Solenoids of 1000 turns and 2000 turns.

Cores of brass, stainless steel, ferrite and magnetic materials.

Motor/generator set with attached tachometer (24 VDC).

• EMT16. 1PH Squirrel-Cage Motor with Starting and Running Capacitor.

Supply voltage: 110 – 220 V.

Power: 370 W.

Speed: 2780 rpm.

Frequency: 50 / 60 Hz

Armature current: 1.85 A.

Shaft height: 71 mm.

• EMT7. 3PH Squirrel-Cage Motor.

Nominal power: 370 W.

Nominal voltage: 3 x 230 / 400 VAC Δ/Y.

Frequency: 50 / 60 Hz.

Number of poles: 2.

Speed: 2730 rpm.

Nominal current: 1.67 / 0.97 A.

Shaft height: 71 mm.

• EMT12. Universal Motor.

Supply voltage: 110 – 240 VAC/ VDC.

Power: 0.3 HP.

Speed: 9000 rpm.

Shaft height: 71 mm.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 50 Kg approx. (110 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Checking the operation of the Industrial Main Power Supply Module (N-ALI01).

2.- Checking the operation of the Power Supply Module (N-ALI10).

3.- Electrostatic demonstration on several materials.

4.- Verification of the electrotechnic effects with acetate.

5.- Verification of the electrotechnic effects with rabbit fur.

6.- Verification of the sign of the charge with the electroscope.

7.- Resistance measurement.

8.- Resistors in series association.

9.- Resistors in parallel association.

10.- Coils in series association.

11.- Coils in parallel association.

12.- Star/delta transformation.

13.- Delta/star transformation.

14.- Capacity measurement of a capacitor.

15.- Capacitors series association.

16.- Capacitors parallel association.

17.- Analysis of the charge of a capacitor.

18.- Analysis of the discharge of a capacitor.

19.- Calculation of the time constant of discharge of the capacitor.

20.- Wiring and put into operation of a Single-Phase Motor.

21.- Wiring and put into operation of a Universal Motor.

22.- Wiring and put into operation of a Squirrel-Cage Three-Phase Motor.

23.- Electric energy into mechanic energy conversion.

24.- Mechanic energy into electric energy conversion.

25.- Electric energy into magnetic energy conversion.

26.- Magnetic induction: Lenz´s Law.

27.- Assembling the transformer.

28.- Analysis of a back transformer.

29.- Wiring of a back transformer.

30.- Verification of the transformer ratio of a back transformer.

31.- Analysis of a back transformer.

32.- Wiring of a back transformer.

33.- Verification of the transformer ratio of a boost transformer.

34.- Analysis of an auto-transformer.

35.- Wiring of an auto-transformer.

36.- Connection as single-phase transformer.

37.- Direct delta/delta three-phase connection.

38.- Star/delta three-phase connection.

39.- Delta /star three-phase connection.

40.- Three-phase/six-phase connection.

41.- Transformer with coils in series in phase.

42.- Calculation of the time constant in different configurations of RLC circuits.

43.- Analysis of a RL circuit in series.

44.- Analysis of a RL circuit in parallel.

45.- Analysis of a RC circuit in series.

46.- Analysis of a RC circuit in parallel.

47.- Analysis of a RCL circuit in series.

48.- Low-pass filter.

49.- High-pass filter.

50.- Lamp controlled by a switch and a push button.

51.- Lamp controlled from two points

52.- Lamp controlled from three points.

53.- Lamp controlled by relays.

54.- Verification of the electrotechnic effects into an acoustic circuit.

55.- Verification of the electrotechnic effects into a fluorescent tube.

Optional

AEL-AI13/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.