TENDER SPECIFICATIONS

The application “AEL-CM2” includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ARR01. Star-Delta Manual Start Module.

Nominal voltage: 400 VAC.

Maximum contacts current: 10 A.

Star-delta three positions commutator:

0: Open circuit.

Y: Star connection.

Δ: Delta connection.

• N-ARR05. Manual Star-Delta Starter with Inversion Module.

Nominal voltage: 400 VAC.

Three input terminals for power supply connection.

Six output terminals for motor connection.

Maximum contacts current: 10 A.

Star-delta three positions commutator:

0: Open circuit.

Y: Star connection.

Δ: Delta connection.

Three position switch:

0: Open circuit.

1: Direct connection.

2: Reverse connection.

• N-ARR07. Manual Dahlander Switch Module, Two Speeds.

Nominal voltage: 400 VAC.

Maximum contacts current: 10 A.

Three positions commutator:

0: Open circuit.

1: Low speed.

2: High speed.

• N-ARR12. Direct Starter Module.

Nominal voltage: 400 VAC.

Three input terminals for power supply connection.

Six output terminals for motor connection.

Maximum contacts current: 10 A.

Two ON / OFF switches:

0: Open circuit.

Y: Closed circuit.

• N-ARR13. Direct Starter with Inversion Module.

Nominal voltage: 400 VAC.

Three input terminals for power supply connection.

Six output terminals for motor connection.

Maximum contacts current: 10 A.

Three position switch:

0: Open circuit.

1: Direct connection.

2: Reverse connection.

• EMT8. 3PH Wound Motor.

Nominal power: 300 W.

Nominal voltage: 3 x 230 / 400 VAC Δ/Y.

Frequency: 50 / 60 Hz.

Number of poles: 2.

Speed: 2870 rpm.

Nominal current: 1 / 0.5 A.

Shaft height: 71 mm.

• EMT9. Two-Speed Dahlander Motor.

Nominal power: 370 W.

Nominal voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Poles number: 4.

Speed: 1400 / 2800 rpm.

Nominal Current: 1.2 / 1.55 A.

Shaft height: 71 mm.

• N-TRANS03. Three-Phase Autotransformer 400/230 VAC, 1 kVA, Module.

Three-phase autotransformer.

Nominal supply voltage: 400 VAC (3PH).

Nominal output voltage: 3 x 230 VAC (3PH + N).

Nominal power: 1 kVA.

Transformer connection: YY0.

Start/stop commutator for instantaneous connection/disconnection of the grid transformer.

Fuses: 3 x 5 A.

• FLYW. Flywheel.

Weight: 2 kg.

Recommended maximum speed: 4000 rpm.

Moment of inertia: 0.0025 kgm².

• N-REFTI. Three-Phase Independent Resistor Module.

Nominal voltage: 400 VAC.

Resistor value: 3 x 150 Ohms.

Nominal power: 3 x 352 W.

Manual commutator to switch ON / OFF the resistors.

Fuses: 3 x 5 A.

Terminals:

Three input terminals (3PH).

Three output terminals (3PH).

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 920 mm approx.(25.19 x 12.59 x  36.22 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Checking the working of the main power supply.

2.- Manual star-delta starter of asynchronous three-phase induction motor with wound rotor.

3.- Manual start and reverse operation of asynchronous three-phase induction motor with wound rotor.

4.- Manual star-delta with inversion circuit of asynchronous three-phase induction motor with wound rotor.

5.- Manual soft starter with resistors of asynchronous three-phase induction motor with wound rotor.

6.- Manual start and reverse operation of Dahlander motor.

7.- Manual speed variation of a Dahlander motor.

8.- Manual speed variation of a Dahlander motor with inversion.

Optional

AEL-CM2/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.