TENDER SPECIFICATIONS

The “AEL-DLT” application includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH+N+G.

ON/OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN+E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push button.

• N-EALD. Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Single and three-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz y 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 - 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 - 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons, four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

Modbus TCP communication protocol with Ethernet interface.

• EMT7/1K. 3PH Squirrel-Cage Motor, 1 kW.

Nominal power: 1100 W.

Nominal voltage: 3 x 230/400 VAC Δ/Y.

Frequency: 50/60 Hz.

Poles number: 2.

RPM: 2730 rpm.

Nominal current: 2.52/1.45 A.

• N-ARR01. Manual Star Delta Starter.

Nominal voltage: 400 VAC.

Three input terminals for power supply connection.

Six output terminals for motor connection.

Maximum contacts current: 10 A.

Star-delta three positions commutator:

0: Open circuit.

Y: Star connection.

Δ: Delta connection.

• N-TRANS03. Three-Phase Autotransformer 400/230 VAC, 1 kVA, Module.

Three-phase autotransformer.

Input voltage: 400 VAC (3PH).

Output voltage: 3 x 230 VAC (3PH+N).

Nominal power: 1 kVA.

Three-phase ON/OFF switch.

Fuses: 3 x 5 A.

• N-MPS. Motor Protection (1.6 A - 2.5 A).

ON/OFF commutator.

Limit thermal current:

From 1.6 A - 2.5 A.

Input power terminals:

Power input connections: L1, L2, L3.

Output power terminals:

Power output connections: L1, L2, L3.

• N-PSM. Power Switch Module.

Rated voltage of power terminals: 400 VAC+N.

Frequency: 50/60 Hz.

Control voltage: 24 VDC.

Nominal current: 15 A.

Functions: two pushbutton switches and remote control for switch-off relay.

Contacts:

4 x three normally open contacts.

1 x normally open auxiliary contact.

1 x normally close auxiliary contact.

• N-CAR19T3. Three-Phase Bench of Commutable Capacitors Module (2 units).

ON/OFF switch.

3 x manual switches to turn on and off the capacitors.

4 x input voltage terminals: 400 VAC+N.

Capacitors: 3 x (3 x 7) μF.

Three LED indicators.

Required elements (Not included):

• AEL-SERIN/CA-1KW. 1 kW AC Industrial Servomotor Application.

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH+N+G.

ON/OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal, 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push button.

• N-DMC01. Double Measurement and Control Module 1.

Digital torque meter:

Torque signal connector.

Auxiliary contact NO.

Digital speed meter:

Speed signal connector.

Auxiliary contact NO.

IP66 protection.

• N-SERV1K. 1 kW Servomotor Module.

Dynamic and static four-quadrant operation.

Speed and torque signals.

Power supply terminals: 3 x 400 VAC.

Frequency: 50/60 Hz.

Nominal power: 1 kW.

Maximum power: 3 kW.

Maximum speed: 5000 rpm.

Nominal torque: 3.18 Nm.

Maximum torque: 9.55 Nm.

Temperature monitoring.

Speed control potentiometer.

Torque control potentiometer.

Mode selection switch: torque/speed control.

Motor turning reversal switch.

Additional recommended elements to increase the functionality of the “AEL-SERIN/CA-1KW” unit (Not included):

• EM-SCADA. Control and Data Acquisition System Software for Electrical Machines.

Power supply: 230 VAC.

Input signals:

Current signal (x2).

RMS current signal (x2).

Voltage signal (x2).

RMS voltage signal (x2).

Torque and speed signals.

Output signals:

Torque control signal for the servomotor.

Speed control signal for the servomotor.

The three softwares are part of the SCADA system.

Compatible with actual Windows operating systems. Graphic and intuitive simulation of the process  in screen. Compatible with the industry standards.

Registration and visualization of all process variables in an automatic and simultaneous way.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

It allows graphic representation in real time.

Comparative analysis of the obtained data, after the process and modification of the conditions during the process.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 640 x 320 x 920 mm approx. (25.19 x 12.59 x 36.22 inches approx.) - Weight: 35 Kg approx. (77 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of direct starting of induction motor and the influence on the electrical parameters of the network.

2.- Study of star/delta starting with autotransformer of the induction motor and the influence on the electrical parameters of the network.

3.- Measurement of active, reactive and apparent power using the network analyzer for the analysis of the induction motor behavior.

4.- Study of power factor compensation during induction motor operation by connecting capacitor banks.

5.- Study of the influence of the variation of the braking torque of the servomotor on the induction motor.

Practical exercises with the recommended software, “EM-SCADA”:

6.- Real-time monitoring of torque.

7.- Real-time monitoring of the speed.

8.- Real-time monitoring of current and voltage RMS values.

9.- Real-time monitoring of current and voltage waveforms.

10.- Advanced electrodynamic study of the motor.

11.- Automatic braking test and real time monitoring of the results.

12.- Obtaining of the characteristic curves of the machine (torque-speed, torque-current, etc).

13.- Storage and comparison of the obtained results.

- Several other exercises can be done and designed by the user.

Optional

AEL-DLT/ICAI. Interactive Computer Aided Instruction Software System:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.