TENDER SPECIFICATIONS

The “AEL-DMG-KIT” application includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• MED65. Digital Multimeter (2 units).

Digital multimeter with 3 ½ digits, with 4 mm double connector termination cables to facilitate interconnections.

With this digital multimeter we can measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• BRLA. Compass.

Required elements (Not included):

• CHER. Tool Box.

Crimper.

Tin.

Meter.

Insulation tape.

Heat shrink.

Voltage tester screwdriver.

Cross-head screwdriver and flat-head screwdriver.

Allen keys.

Soldering iron and soldering iron stand.

Rubber hammer.

Connection terminals.

Flanges.

Wire cutter.

Screw clamp terminals.

Wire strippers.

Required elements (at least one) (Not included):

DMG-K1: Dissectible of DC Independent Shunt-Series-Compound Excitation Motor-Generator Kit.

• EMT5-D. Dissectible DC Shunt/Series/Compound Excitation Motor-Generator.

Nominal power: 300 W.

Armature voltage: 200 VDC.

Excitation voltage: 230 VDC.

Armature current: 1.5 A.

Excitation current: 0.4 A.

Speed: 3400 / 7500 rpm.

Shaft height: 71 mm.

• N-VVCC/M. DC Motor Speed Controller Module (Intermediate option).

Supply voltage: 230 VAC.

Variable output voltage: 0 – 300 VDC.

Fuse: 2 A.

• N-REV. Single Phase Variable Resistor Module.

Variable resistor: 150 Ohm.

Maximum power: 500 W.

Potentiometer.

Terminals:

Three terminals to choose all resistance or variable resistance.

Fuse: 2 A.

DMG-K2: Dissectible of AC Synchronous Three-Phase Motor Alternator Kit.

• EMT6-D. Dissectible Independent Excitation 3PH Synchronous Motor-Generator.

Nominal power: 250 W.

Nominal output voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Speed: 3000 rpm.

Nominal output current: 1 A.

Nominal excitation current: 0.25 A.

Shaft height: 71 mm.

• N-VVCA/M. AC Motors Speed Controller (Intermediate option).

ON / OFF switch.

Supply voltage: 230 VAC.

Nominal power: 750 W.

PWM output voltage connections:

Three-phases: 230 VAC.

10 K potentiometer for the induction motor control speed.

Setting and visualization of the machine parameters.

• N-VVCC/M. DC Motor Speed Controller Module (Intermediate option).

Setting and visualization of the machine parameters.

Supply voltage: 230 VAC.

Variable output voltage: 0 – 300 VDC.

Fuse: 2 A.

DMG-K3: Dissectible of Asynchronous Three-Phase Motor of Squirrel Cage Kit.

• EMT7-D. Dissectible 3PH Squirrel-Cage Motor.

Nominal power: 370 W.

Nominal voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Speed: 900 / 1420 rpm.

Nominal current: 1.05 / 1.35 A.

Shaft height: 71 mm.

DMG-K4: Dissectible of Asynchronous Three-Phase Motor with Wound Rotor Kit.

• EMT8-D. Dissectible 3PH Wound Motor.

Nominal power: 300 W.

Nominal voltage: 3 x 230 / 400 VAC Δ / Y.

Frequency: 50 / 60 Hz.

Poles number: 2.

Speed: 2870 rpm.

Nominal current: 1 / 0.5 A.

Shaft height: 71 mm.

DMG-K5: Dissectible of Dahlander Three-Phase Motor Kit.

• EMT9-D. Dissectible Dahlander Motor, 2 speeds.

Nominal power: 370 W.

Nominal voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Poles number: 4.

Speed: 1400 / 2800 rpm.

Nominal current: 1.2 / 1.55 A.

Shaft height: 71 mm.

DMG-K6: Dissectible of Asynchronous Three-Phase Motor of Two Independent Speeds Kit.

• EMT10-D. Dissectible 3PH Squirrel-Cage Motor, 2 speeds.

Nominal power: 240 / 370 W.

Nominal voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Speed: 900 / 1420 rpm.

Nominal Current: 1.05 / 1.35 A.

Shaft height: 71 mm.

DMG-K7: Dissectible of Asynchronous Single-Phase Motor with Starting Capacitor Kit.

• EMT11-D. Dissectible 1PH Squirrel-Cage Motor with Starting Capacitor.

Nominal power: 370 W.

Nominal voltage: 3 x 230 VAC.

Frequency: 50 / 60 Hz

Speed: 2780 rpm.

Nominal current: 2.53 A.

Shaft height: 71 mm.

DMG-K8: Dissectible of Universal Motor Kit.

• EMT12-D. Dissectible Universal Motor.

Nominal power: 230 W.

Nominal voltage: 110 – 240 VAC / VDC.

Speed: 9000 rpm.

Shaft height: 71 mm.

• N-REV. Single Phase Variable Resistor Module.

Variable resistor: 150 Ohm.

Maximum power: 500 W.

Potentiometer.

Terminals:

Three terminals to choose all resistance or variable resistance.

Fuse: 2 A.

DMG-K9: Dissectible of Asynchronous Single-Phase Motor with Starting and Running Capacitor Kit.

• EMT16-D. Dissectible 1PH Squirrel-Cage Motor with Starting and Running Capacitor.

Nominal power: 370 W.

Nominal voltage: 110 / 220 VAC.

Speed: 2780 rpm.

Frequency: 50 / 60 Hz.

Nominal current: 1.85 A.

Shaft height: 71 mm.

DMG-K10: Dissectible of DC Brushless Motor Kit.

• EMT18-D. Dissectible DC Brushless Motor.

Nominal power: 80 W.

Nominal voltage: 24 VDC.

Nominal current: 3.3 A.

Speed: 3250 rpm.

Shaft height: 71 mm.

• N-ALI03. AC Auxiliary Power Supply (24 VAC) Module.

Supply voltage (single-phase): 230 VAC 1PH + N.

Output voltage:

Single-Phase 24 VAC / 12 VAC.

24 VDC.

0 – 24 VDC through potentiometer.

DMG-K11: Dissectible of Asynchronous Single-Phase Motor with split phase Kit.

• EMT20-D. Dissectible 1PH Squirrel-Cage Motor with Split Phase.

Nominal power: 370 W.

Nominal voltage: 220 VAC.

Speed: 2780 rpm.

Frequency: 50 Hz.

Armature current: 2.53 A.

Shaft height: 71 mm.

DMG-K12: Dissectible of Three-phase Reluctance Motor Kit.

• EMT21-D. Dissectible 3PH Reluctance Motor.

Nominal power: 300 W.

Nominal voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Speed: 3000 rpm.

Nominal current: 1.4 A.

Shaft height: 71 mm.

DMG-K13: Dissectible of Single-phase Shaded Pole Motor Kit.

• EMT22-D. Dissectible 1PH Shaded Pole Motor.

Nominal power: 34 W.

Nominal voltage: 230 / 240 VAC.

Frequency: 50 / 60 Hz.

Speed: 1550 rpm.

Shaft height: 71 mm.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

EMT5-D:- Dimensions: 330 x 400 x 300 mm approx.(12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT6-D: - Dimensions: 330 x 400 x 300 mm approx.(12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT7-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT8-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx.(11 pounds approx.)

EMT9-D:- Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT10-D:- Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT11-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT12-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT16-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg. approx. (11 pounds approx.)

EMT18-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT20-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT21-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EMT22-D: - Dimensions: 330 x 400 x 300 mm approx. (12.99 x 15.74 x 11.81 inches approx.) - Weight: 5 Kg approx. (11 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Complete step by step assembly of DC Independent Shunt/Series/Compound Excitation Motor/Generator.

2.- Put into operation the DC Independent Shunt/Series/Compound Excitation Motor-Generator.

3.- Complete step by step assembly of AC Synchronous Three-Phase Motor-Alternator.

4.- Put into operation the AC Synchronous Three-Phase Motor Alternator.

5.- Complete step by step assembly of Asynchronous Three-Phase Motor of Squirrel Cage.

6.- Put into operation the Asynchronous Three-Phase Motor of Squirrel Cage.

7.- Complete step by step assembly of Asynchronous Three-Phase Motor with Wound Rotor.

8.- Put into operation the Asynchronous Three-Phase Motor with Wound Rotor.

9.- Complete step by step assembly of Dahlander Three-Phase Motor.

10.- Put into operation the Dahlander Three-Phase Motor.

11.- Complete step by step assembly of Asynchronous Three-Phase Motor of Two Independent Speeds.

12.- Put into operation the Asynchronous Three-Phase Motor of Two Independent Speeds.

13.- Complete step by step assembly of Asynchronous Single-Phase Motor with Starting Capacitor.

14.- Put into operation the Asynchronous Single-Phase Motor with Starting Capacitor.

15.- Complete step by step assembly of Universal Motor.

16.- Put into operation the Universal Motor.

17.- Complete step by step assembly of Asynchronous Single-Phase Motor with Starting and Running Capacitor.

18.- Put into operation the Asynchronous Single-Phase Motor with Starting and Running Capacitor.

19.- Complete step by step assembly of DC Brushless Motor.

20.- Put into operation the DC Brushless Motor.

21.- Complete step by step assembly of Asynchronous Single-Phase Motor with split phase.

22.- Put into operation the Asynchronous Single-Phase Motor with split phase.

23.- Complete step by step assembly of Three-Phase Reluctance Motor.

24.- Put into operation the Three-Phase Reluctance Motor.

25.- Complete step by step assembly of Single-Phase Shaded Pole Motor.

26.- Put into operation the Single-Phase Shaded Pole Motor.

- Several other exercises can be done and designed by the user.

Optional

AEL-DMG-KIT/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.