TENDER SPECIFICATIONS

The application “AEL-EEEM” includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN+E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-EALD. Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-VVCA/M. AC Motors Speed Controller Module (intermediate option).

ON / OFF Switch.

Supply voltage: 230 VAC.

Nominal power: 750 W.

PWM output voltage connections:

Three-phases: 230 VAC.

10 K, potentiometer for the induction motor control speed.

Setting and visualization display of the machine parameters.

• N-VVCC/M. DC Motors Speed Controller Module (Intermediate option).

Supply voltage: 230 VAC.

Variable output voltage: 0 – 300 VDC.

Fuse: 2 A.

• FRECP. Eddy Current Brake.

Maximum supply voltage: 60 VDC.

Nominal current: 1.67 A.

Maximum current: 1.8 A.

Maximum braking torque: 1.4 Nm.

Shaft height: 71 mm.

• EMT7. 3PH Squirrel-Cage Motor.

Nominal power: 370 W.

Nominal voltage: 3 x 230 / 400 VAC Δ/Y.

Frequency: 50 / 60 Hz.

Number of poles: 2.

Speed: 2730 rpm.

Nominal current: 1.67 / 0.97 A.

Shaft height: 71 mm.

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 50 Kg approx. (44 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Wiring an asynchronous three-phase motor of squirrel cage.

2.- Start-up of the frequency controller.

3.- Forward and reverse operations with the induction motor and the frequency controller.

4.- Comparison of the energy consumption by the conventional electrical power and the frequency controller.

5.- Programming of the frequency controller according to different electrical machines operations.

6.- Checking the behavior of the electrical machine in function of the braking torque.

7.- Study the motor functioning for a demand instruction.

8.- Programming of the frequency controller for different loads.

9.- Programming of the frequency controller to get an optimal consumption.

10.- Analyze the electrical parameters.

Optional

AEL-EEEM/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.