TENDER SPECIFICATIONS

The "AEL-EGMG24" application includes the following elements:

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (Single-Phase): 230 VAC, PH+N+G.

ON-OFF removable key.

Output Voltage Connections:

Two Single-Phase: 230 VAC.

Single-Phase supply hose connecting plug.

Differential magnetothermal, 2 poles, 25 A, 30 mA AC 6 KA.

• N-VVCA/M. AC Motors Speed Controller (Intermediate option).

Supply voltage: 230 VAC.

ON/OFF Switch.

Nominal power: 0.4 kW.

PWM output voltage connections:

Three-Phases: 230 VAC.

10 K potentiometer for the induction motor control speed.

Setting and visualization display of the machine parameters.

• EMT7C. 3PH Squirrel-Cage Motor, 8 poles.

Nominal power: 550 W.

Nominal voltage: 3x 230 / 400 VCA.

Frequency: 50/60 Hz.

Poles number: 8.

RPM: 750 rpm.

Nominal current: 3.6 / 2 A.

Shaft height: 71 mm.

• EMT6C. Permanent Magnets 3PH Synchronous Motor-Generator, 8 poles, 24 Vac.

Nominal power: 0.6 kVA.

Nominal voltage: 24 VAC.

RPM: 850 rpm.

Poles number: 8.

• N-TRA30. 1 KVA Three-Phase Isolating Transformer 24VAC/400VAC.

Nominal power: 1kVA.

Input voltage: 3PH 24 VAC.

Output voltage: 3PH 400 VAC.

Ground terminal.

• N-REFT300. 300 Ohms Three-Phase Fixed Resistor Module. (2 units)

Nominal voltage: 400 VAC.

Resistor value: 3x 300 Ohms.

Nominal power: 3x 200 W.

Manual commutator to switch on/off the resistors.

Fuses: 3x 5 A.

Terminals:

Three input terminals (3PH).

Three output terminals (3PH).

Ground terminal.

• N-CONT. Three- phase Commutable Capacitive Load.

Capacitor value: 3x 7 µF.

Nominal voltage: 400 VAC.

Manual commutator to switch on/off the capacitors.

Fuses: 3x 4 A.

Terminals:

Four inputs terminals (3PH+N).

Four outputs terminals (3PH+N).

Ground terminal.

• N-INDT. Three-phase Commutable Inductive Load.

Nominal voltage: 400 VAC.

Inductance value: 1.4 H.

Nominal current: 1.6 A.

Manual commutator to switch on/off the inductances.

Fuses: 3x 2 A.

Terminals:

Four inputs terminals (3PH+N).

Four outputs terminals (3PH+N).

Ground terminal.

• N-EALD. Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Single and three-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz y 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 - 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40 th harmonic.

Voltage and current THD, TDD & K-factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage.

Harmonics analyzer:

Voltage and current THD, TDD current and K-factor, up to the 40 th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 - 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons, four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

Modbus TCP communication protocol with Ethernet interface.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 1380 x 840 x 2010 mm approx. (54.33 x 33.07 x 79.13 inches approx.). Weight: 80 Kg approx. (176 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Wiring of the motor-generator group.

2.-  Commissioning of the Frequency Controller.

3.-  Adjusting the necessary values in the frequency controller.

4.-  Adjusting the rotation direction.

5.-  Adjusting the start function.

6.-  Frequency/speed control of permanent magnet synchronous generator.

7.-  Study of voltage control generation.

8.-  Study of electrical generator with no load.

9.-  Study of drop voltage with resistive load.

10.-Study of drop voltage with inductive load.

11.-Study of drop voltage with capacitive load.

12.-Study of power factor.

13.-Study of electrical generator with different power factors.

14.-Study of the voltage versus speed of the permanent magnet synchronous generator.

Optional

AEL-EGMG24/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.