TENDER SPECIFICATIONS

The "AEL-EMRP" unit includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3 PH+N+G.

ON-OFF removable key.

Output voltage connections:

Three-Phase + Neutral: 400 VAC.

Single-Phase: 230 VAC.

Three-Phase supply hose with IP44 3PN+E 32A 400V connecting plug.

Differential magnetothermal, 4 poles, 25 A, 300 mA AC 6 KA.

Emergency stop push button.

• N-REL09. Time Electronic Relay against Overcurrents Module (0-16 A).

Nominal voltage: 230 / 24 VAC.

Time electronic relay against overcurrents.

Contacts:

Normally Open Contact (NO).

Normally Close Contact (NC).

Common point of normally close/open contacts.

• N-MPS. Motor Protection (1.6 A-2.5 A).

ON/OFF switch.

Limit thermal current:

From 1.6 A to 2.5 A.

Input power terminals:

Power input connections: L1, L2, L3.

Output power terminals:

Power output connections: L1, L2, L3.

Ground Terminal.

• EMT7/1K. 3PH Squirrel-Cage Motor, 1 kW.

Nominal power: 1100 W.

Nominal voltage: 3x 230/400 VAC Δ/Y.

Frequency: 50/60 Hz

Number of poles: 2.

Speed: 2730 rpm.

Nominal current: 2.52 / 1.45 A.

• N-SERV1K. 1 kW Servomotor Module.

Dynamic and static four-quadrant operation. Speed and torque signals.

Connection voltage: 400 V.

Frequency: 50/60 Hz.

Maximum power output: 10 kVA.

Maximum speed: 4000 rpm.

Maximum torque: 30 Nm.

Temperature monitoring.

Speed control potentiometer.

Torque control potentiometer.

Control mode selection switch torque / speed servomotor control permission switch.

Motor turning reversal switch.

GND terminal.

• MED65. Digital Multimeter.

This module has a digital multimeter of about 3 ½ digits, with double-jack ending cables of about 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• N-EALD. Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Single and three-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz y 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 - 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40 th harmonic.

Voltage and current THD, TDD & K-factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage.

Harmonics analyzer:

Voltage and current THD, TDD current and K-factor, up to the 40 th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 - 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons, four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

Modbus TCP communication protocol with Ethernet interface.

• N-PSM. Power Switch Module.

Power terminals:

Four input terminals (3 PH+N).

Four output terminals (3PH+N).

Auxiliary contacts:

One NO contact.

One NC contact.

Two push-buttons to open/close the power switch and the auxiliary contacts.

Two 24 VDC control inputs.

Two 24 VDC voltage outputs.

• N-DCPWS-24V/120W. 24VDC, 120W Power Supply Module.

DC Power supply.

Output voltage: 24 VDC.

Nominal Power: 120 W.

Additional recommended elements (Not included):

• EM-SCADA. Control and Data Acquisition System Software for Electrical Machines.

Power supply: 230 VAC.

Input Signals:

Current signal (x2).

RMS current signal (x2).

Voltage signal (x2).

RMS voltage signal (x2).

Torque and speed signals.

Output Signals:

Torque and speed control signal for the servomotor control.

Speed control signal for the servomotor.

The three softwares are part of the SCADA system.

Compatible with actual Windows operating systems. Graphic and intuitive simulation of the process in screen. Compatible with the industry standards.

Registration and visualization of all process variables in an automatic and simultaneous way.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

It allows the registration of the alarms state and the graphic representation in real time.

Comparative analysis of the obtained data, after the process and modification of the conditions during the process.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

• All necessary cables to realize the practical exercises are included.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 1380 x 840 x 2010 mm approx. (54.33 x 33.07 x 79.13 inches approx.). Weight: 80 Kg approx. (176 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Wiring and put into operation of a 3PH Squirrel-Cage Motor, 1 kW.

2.-  Studying and testing of configuration of overcurrent time electronic relay.

3.-  Studying and testing of configuration of thermal protection relay.

4.-  Studying in combination of motor protection relays.

5.-  Load protection for disconnecting the power supply to the motor.

6.-  Studying of servomotor working as load.

7.-  Studying of the joint operation of motor with servomotor.

8.-  Selectivity and coordination of the protection relays.

9.-  Analyze the network parameters with different load conditions.

10.-Configuration of the protection relays for the motor starter.

Optional

AEL-EMRP/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.