TENDER SPECIFICATIONS

The “HPSG-UB” unit includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-Phase + Neutral: 400 VAC.

Single-Phase: 230 VAC.

Three-Phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 300 mA AC 6 KA.

Emergency stop push-button.

• GEC-KIT-1. Generator Excitation Circuit Kit 1.

• N-AVR/P. Automatic Voltage Regulator Module.

Input voltage: 0 – 100 VDC.

Input for voltage regulation signal through PWM control.

Terminals for PWM signals monitoring.

Output voltage: 0 – 100 VDC.

DC ammeter for output current measurement, 0 – 4 A.

3-pin female connector for connection to the excitation input of a generator.

Fuse 5 A.

• N-TRANS04. Single-Phase Transformer Module 230 VAC/2x35 VAC, 300 VA.

230 VAC single-phase input terminals.

Terminals for a 70 VAC single-phase output.

Terminals for two 35 VAC single-phase outputs.

0 (Opened)/1 (Closed) selector switch.

Fuses:

Primary side: 5 A.

Secondary side: 2.5 A.

• N-SPAD01. Single-Phase AC/DC Converter Module 1.

0 – 230 VAC single-phase input terminals.

AC/DC rectifier.

DC output terminals.

Filter capacitor for voltage ripple reduction.

Bypass terminals for filter capacitor connection.

5 A fuse.

• N-EALD. Electrical Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-REL10. Time Analog Relay against Overcurrents Module (0 - 16 A).

Three-pole analog overcurrent relay.

Overcurrent, undercurrent and phase loss protection.

Range 0 – 16 A.

Automatic and manual reset functions.

Trip and startup temporization.

Two NO NC auxiliary contacts.

• GMG4.5K3PH. 4.5 kW Generator-Motor group.

Motor-generator group coupled in an aluminum frame with wheels.

Generator nominal power: 4.5 kVA.

Generator nominal speed: 3000 rpm.

Generator nominal output current: 6.5 A.

Generator nominal excitation current: 4 A.

Generator power factor: 0.8.

Generator nominal output voltage: 3 x 400 VAC.

Frequency: 50/60 Hz.

Motor nominal power: 5 kVA.

Motor nominal current: 7.2 A.

Motor nominal speed: 3000 rpm.

Elements required for kit generator control of the “HPSG-UB” unit (at least one) (Not included):

Manual control of the Turbine-Generator group:

• HPSG-K1. Manual Synchronization of High Power Synchronous Generators Kit.

• N-ASYB. Analog and Digital Synchronization Module.

Supply voltage: 230 VAC.

Input terminals:

Generation input terminals L1, L2, L3 and N.

Output terminals:

Network input terminals L1, L2, L3 and N.

Synchronization switch:

2 x 24 VDC Connections.

Digital synchronoscope.

Synchronization switch.

Double frequency meter.

Double voltmeter.

400 VAC Lamps.

• N-PWM01. PWM Signal Generator Module 1.

Enable/disable switch to enable the signal generation.

Potentiometer for the duty cycle control.

Display for the working and maximum duty cycle of the generated signals.

Micro-connector for the transmission of the generated signals.

2 mm terminals to monitor the generated signals through an oscilloscope or any other device.

ON / OFF power switch.

• N-VVCA5K. 5 kW Motor Speed Controller Module.

ON / OFF switch.

Micro connector of 8 pins.

Supply terminals:

Supply: 400 VAC.

Supply terminals L1, L2, L3 and N from the module to the frequency variator.

Output connector.

Motor speed potentiometer.

Automatic control of the Turbine-Generator group:

• N-ERP-PGC01. Generator Protection Relay Module.

Automatic speed and voltage controller:

It enables to connect up to 16 electric generators in parallel-island with active and reactive power distribution and start/stop depending on the load demand.

Enables to connect a generator in parallel with the grid.

It allows different switch control modes, such as opening, closing and synchronization.

Three-phase measurement of the grid and the generator voltage.

Three-phase measurement of the generator intensity and power.

Single-phase measurement of the grid intensity.

Protection system:

Generator: Maximum/minimum voltage (59/27), maximum/minimum frequency (81O/U), voltage asymmetry, detection of dead busbars, overvoltage (32), load unbalance (46), negative sequence power/reduced power (32R/F), overcurrent by defined curve (50/51), inverse time overcurrent (IEC255), measured ground fault (50N/51N), phase rotation, switches faults.

Network: Maximum/minimum voltage (59/27), maximum/minimum frequency (81O/U), vector jump, phase rotation.

Measurement terminals.

Output terminals for the connection to the lab grid.

2 mm terminals to monitor the PWM for the excitation circuit and speed signals.

Three control switches to:

Start and stop the turbine.

Give permission to synchronize the generator with the grid.

Give permission to synchronize the generator with the national grid.

Two potentiometers to:

Adjust the set point for the generated active power.

Adjust the set point for the voltage.

Emergency stop push button:

Two circuit breakers for synchronization and island mode operations with state indicator lamps.

Additional recommended elements for a greater functionality of the “HPSG-UB” unit (at least one) (Not included):

• RL-KIT-1. Resistive and Inductive Loads Kit 1.

• N-REFT/3C. 3 x 300 W Three-Phase Configurable Resistors Module.

Configurable star and delta connections.

Three banks with three three-phase resistors of 1600 Ω.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 100) W.

• N-INDT/3C. 3 x 300 VAr Three-Phase Configurable Inductances Module.

Configurable star and delta connection.

Inductance: 3 x (3 x 5) H.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 100) Var.

Supply voltage: 230 VAC.

Common point of normally close/open contacts.

• GFS-KIT-1. Study Faults in Generators Kit 1.

• N-ERP-PDF01. Differencial Protection Relay with Digital I/O Module.

Differential protection relay module.

Single-phase supply voltage: 230 VAC.

Light indicator of TRIP.

Differential protection.

DB9 RS – 232 communication connector.

SUB-D signals connector of 62 pins.

ON / OFF switch.

Connection terminals.

Characteristics:

It protects two terminals transformers, generators, reactances and other power devices using a combination of differential, instantaneous and “inverse-time” overcurrent elements. The safety of the differential diagram is obtained by the following actions:

Dual-slope percentage reduction.

Second and forth harmonic blocking.

Fifth harmonic blocking for transformer overexcitation.

CT and transformers connection compensation.

• N-PSM. Power Switch Module (2 units).

Supply voltage: single-phase 230 VAC.

Power terminals:

Four power input terminals (3PH + N).

Four power output terminals (3PH + N).

Auxiliary contacts:

One “NO” contact.

One “NC” contact.

Two push-buttons to open/close the power terminals and auxiliary contacts.

Two control contacts of 24 VDC.

Two voltage supply outputs of 24 VDC.

• N-ERP-MF01. Digital Fault Simulator Module.

Enables injecting one, two or three pole different faults at the desired location through fault and line terminals.

Trip time potentiometer.

Ethernet connection: two communication ports for SCADA remote control.

• N-INDT02. Three-Phase Inductances Module 2.

Nominal voltage: 400 VAC.

Inductance value: 16 mH.

Nominal current: 10 A.

Manual commutator to switch on/off the inductances.

Fuses: 3 x 2 A.

Terminals:

Four input terminals (3PH + N).

Four output terminals (3PH + N).

• N-REP. Rotor Earth-Fault Protection Module.

Nominal voltage: 400 VAC.

Relay voltage: 24 VDC.

Contacts: 1 NO, 1 NC.

Contact nomnal current: 10 A.

Test push-button.

Reset push-button.

Two lamps for fault to earth signalling.

• N-THF01. Three-Phase Harmonic Filter Module 1.

Passive filter designed for the third harmonic blockage up to the 90 %, reducing the currents through the neutral in three-phase installations.

Phase to neutral voltage: Up to 750 V.

Frequency: 50 / 60 Hz.

Nominal neutral current: 10 A.

IP00 and IP22 protection.

• N-REVT/1K. 1 kW Three-Phase Variable Resistors Module.

Three banks with three-phase variable resistances of 150 – 500 Ohm.

Supply voltage: 230 VAC.

Fuses: 3 x 2 A.

Fan forced cooling.

• N-MED11. AC Ammeter (0 - 10 A) Module. (Required to do the generator tests).

Measurement range: 0 – 10 A.

Terminals:

Measurement terminals.

• N-MED22. AC Voltmeter (0 - 400 V) Module. (Required to do the generator tests).

Measurement range: 0 – 400 VAC.

Terminals:

Measurement terminals.

• EMG-SCADA. Data Acquisition and Energy Management Software.

The SCADA stands for the operation and control center of the power plant. From this software, the user will control the power genereation resources. On the basis of this predefined planning, the power generation will injecte the available energy into the grid in order to meet the demand requirements. Meanwhile, the user will be able to visualize the power curves resulting from the power plant. All this energy production will keep a delicate balance with the energy consumption determined by the loads.

• TPEL/1800W. Single-Phase AC/CC Electronic Load of 1800W (3 units).

The TPEL/1800W electrical load stands out for its versatility in operating in both AC and DC. With a wide power range from 0 to 1800VA and an adjustable frequency from 45 Hz to 450 Hz, this electronic source offers exceptional capabilities.

With its advanced measurement capabilities, 7-inch LCD display, and features such as the built-in oscilloscope, the TPEL/1800W is the ideal solution for laboratory testing of most of EDIBON’s unit such as UPS, inverters, AC power supplies, electrical generators and AC electronic components. In addition, its connectivity via LAN and USB interfaces ensures reliable and fast control, adding an additional level of efficiency to its capabilities.

Technical Features:

Frequency Range: 45 Hz~450 Hz.

Power Range: 0~1.8 kVA

Voltage Range: 50-420 Vrms, 15-260 Vrms

Parallel Connection/Three Phase Control: Capacity can be extended up to 43.2 kVA

7 Inch LCD Display: Provides a clear and accessible interface.

Oscilloscope Function: Allows display of voltage and current waveforms.

High Speed AD Sampling: Captures real-time waveforms.

Measurement Modes: Includes Vrms, Vpk, Vdc, Irms, Ipk, Idc, W, VA, VAR, CF, PF and FREQ.

Harmonic Measurement: Up to harmonic 50 (THD-V).

AC Electronic Load: DC/CR/CP modes.

DC Electronic Load: DC/CR/CP/CV modes.

External Analog Control: 0~10 V analog input, with voltage and current monitoring functions.

Protection Functions: OTP, OCP, OVP, UVP and OPP.

Communication Interfaces: LAN and USB, plus external interface for USB flash disk.

Additional recommended elements (at least one) (Not included):

• TPEL/1800W. Single-Phase AC/CC Electronic Load of 1800W (3 units).

The TPEL/1800W electrical load stands out for its versatility in operating in both AC and DC. With a wide power range from 0 to 1800VA and an adjustable frequency from 45 Hz to 450 Hz, this electronic source offers exceptional capabilities.

With its advanced measurement capabilities, 7-inch LCD display, and features such as the built-in oscilloscope, the TPEL/1800W is the ideal solution for laboratory testing of most of EDIBON’s unit such as UPS, inverters, AC power supplies, electrical generators and AC electronic components. In addition, its connectivity via LAN and USB interfaces ensures reliable and fast control, adding an additional level of efficiency to its capabilities.

Technical Features:

Frequency Range: 45 Hz~450 Hz.

Power Range: 0~1.8 kVA

Voltage Range: 50-420 Vrms, 15-260 Vrms

Parallel Connection/Three Phase Control: Capacity can be extended up to 43.2 kVA

7 Inch LCD Display: Provides a clear and accessible interface.

Oscilloscope Function: Allows display of voltage and current waveforms.

High Speed AD Sampling: Captures real-time waveforms.

Measurement Modes: Includes Vrms, Vpk, Vdc, Irms, Ipk, Idc, W, VA, VAR, CF, PF and FREQ.

Harmonic Measurement: Up to harmonic 50 (THD-V).

AC Electronic Load: DC/CR/CP modes.

DC Electronic Load: DC/CR/CP/CV modes.

External Analog Control: 0~10 V analog input, with voltage and current monitoring functions.

Protection Functions: OTP, OCP, OVP, UVP and OPP.

Communication Interfaces: LAN and USB, plus external interface for USB flash disk.

• SVC-K3. Dynamic Load Kit 1.

• N-DLC01. Dynamic Load Controller 1 Module.

Nominal voltage: 230 VAC.

Includes brake with 5 Nm nominal torque.

LCD display.

Daily load curve configuration by means of 5 potentiometers:

Test time from 30 to 300 s.

Definition of maximums and minimums of the load curve.

• EMT7B/1K-E. 3PH Squirrel-Cage Industrial Motor, 1 kW, 4 Poles.

Nominal power: 1000 W.

Nominal voltage: 230 VAC / 400 VAC.

Frequency: 50/60 Hz.

Nominal load speed: 1400 rpm.

Nominal current: 4.57 A /2.64 A.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 2000 x 320 x 920 mm approx. (78.73x12.59x36.22 inches approx.) - Weight: 150 Kg approx. (330 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Practical possibilities with the required element for the manual control of the turbine-generator group, “HPSG-K1”:

1.- Manual control of the generator-motor group speed and frequency.

2.- PWM signal generation and duty cycle manual regulation.

3.- Assembly of the self-excited static excitation circuit.

4.- Manual control of the synchronous generator voltage by means of the excitation current control through PWM signals.

5.- Startup of the synchronous generator under no load.

6.- Manual synchronization of the synchronous generator with the grid.

7.- Synchronization through the dark lamps method.

8.- Synchronization through the bright lamps method.

9.- Synchronization through two dark and one bright lamps method.

10.- Monitoring the power flows injected into the grid.

11.- Active power (P) and reactive power (Q) generation control in synchronism with the grid, by means of the generator frequency and excitation current control respectively.

12.- No load test of the synchronous generator.

13.- Empirical obtaining of the synchronous generator no load characteristic, E= f (Iex). Lineal and saturation zone.

14.- Short-circuit test of the synchronous generator.

15.- Empirical obtaining of the synchronous generator short-circuit characteristic.

16.- Calculation of the Poitier's diagram.

17.- Calculation of the synchronous generator impedance.

18.- Determining the equivalent circuit of the synchronous generator.

19.- Calculation of the synchronous generator capacity limits.

20.- Control logic circuit for the protection of the synchronous generator in synchronism with the grid.

21.- Measurement of the harmonic generation caused by the power electronic.

Practical possibilities with the required element for the automatic control of the turbine-generator group, “N-ERP-PGC01”:

22.- Automatic control of the generator-motor group frequency and speed.

23.- Assembly of the self-excited static excitation circuit.

24.- Adjustable automatic control of the synchronous generator output voltage.

25.- Automatic synchronization of the synchronous generator with the grid.

26.- Monitoring the power flows injected into the grid.

27.- Adjustable automatic control of the active power injected into the grid.

28.- Control logic circuit for the protection of the synchronous generator in synchronism with the grid.

Practical possibilities with the additional recommended element for the island operation of the generator, “RL-KIT-1”:

29.- Island operation with manual/automatic control of the generator-motor group.

30.- Study of the generation and demand in isolated systems.

31.- Study of the votlage drops in sycnhronous generators and compensation through manual/automatic regulation of the generator excitation current.

32.- Test for obtaining the external characteristic of the synchronous generator, V = f (I).

Practical possibilities with the additional recommended element for the study of faults in sycnhronous generators, “GFS-KIT-1”:

33.- Overcurrent protection test with the generator in synchronism with the grid.

34.- Overcurrent protection test with the generator in island operation.

35.- Differential protection relay setting.

36.- Study of the scheme for the differential protection of the synchronous generator in case of fault.

37.- Study of single-phase, two-phase, three-phase, to earth, with and without impedance faults.

38.- Study of the differential protection against instantaneous overcurrent (50P1H).

39.- Study of the differential protection against definite time overcurrent (50P1).

40.- Study of the differential protection against inverse time overcurrent (51P1).

41.- Study of the differential protection against definite time negative sequence current (50Q1).

42.- Study of the differential protection against inverse time negative sequence current (51Q1).

43.- Study of the differential protection against instantaneous residual current (50N1H).

44.- Study of the differential protection against definite time residual current (50N1).

45.- Study of the differential protection against inverse time residual current (51N1).

46.- Transient analysis of fault injection by means of the differential relay manufacturer's software.

Practical possibilities with the additional recommended element Rotor Earth-fault Protection Module, “N-REP”:

47.- Study of the scheme for the rotor protection of a synhcronous generator in case of fault.

48.- Rotor earth-fault protection test.

Practical possibilities with the additional recommended element Three-phase harmonic Filter Module 1, “N-THF01”:

49.- Study and monitoring the harmonics generation.

50.- Harmonic reduction by means of a three-phase passive filter.

Practical possibilities with the recommended additional element Single-Phase DC/AC Electronic Load of 1800 W, “TPEL/1800W”:

51.- Study of different dynamic load curves developed by means of the electronic load.

52.- Island operation with manual/automatic control of the motor-generator group with electronic load.

53.- Study of generation and demand in isolated systems with electronic load.

54.- Study of voltage drops in synchronous generators and compensation by manual/automatic regulation of the generator excitation current under electronic load.

Practical possibilities with the recommended additional element, Dynamic Load Kit 1 “SVC-K3”:

55.- Modulation of a load curve consisting of two peaks, a valley and a base load by means of a magnetic dust brake.

56.- Island operation with manual/automatic control of the motor-generator set with dynamic load.

57.- Study of generation and demand in isolated systems with dynamic load.

58.- Study of voltage drops in synchronous generators and compensation by manual/automatic regulation of the generator excitation current under dynamic load.

Practical possibilities with the additional recommended element Data Acquisition and Energy Management Software, “EMG-SCADA”:

59.- Remote manual control of voltage and frequency of the synchronous generator.

60.- Remote automatic control of voltage and frequency of the synchronous generator.

61.- Remote automatic synchronization operation of the synchronous generator and the grid.

62.- Real time monitoring of the induction generator frequency, current and voltage values and waveforms.

63.- Real time monitoring of the generated active, reactive and apparent powers.

64.- Visualization of the phasor diagrams of the system electrical parameters.

65.- Data storage and results obtained.

66.- Comparison of the obtained results.

- Several other exercises can be done and designed by the user.

Optional

AEL-HPSG/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.