TENDER SPECIFICATIONS

The “AEL-KNX1” application includes the following elements:

• N-KNX34. KNX/EIB USB Programming Interface Module.

Power supply: KNX line, 21…30 VDC.

Current consumption (max.): < 5 mA.

USB connection: USB2.0 connector.

• N-KNX18. KNX/EIB Power Supply Module.

Power supply: 230 VAC.

Maximum load: 640 mA.

Output voltage: 24 VDC.

• N-KNX14. KNX/EIB Light Sensor Module.

KNX bus voltage: 24 VDC.

Bus inputs/outputs:  2 mm safety sockets.

Photosensitive cell.

• N-KNX24. KNX/EIB Shutter/Blind Actuator Module.

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

Mains inputs/outputs: 4 mm safety sockets.

• N-KNX20. KNX/EIB Push-Button Module.

KNX bus voltage: 24 VDC.

Switch ON / OFF.

Mains inputs/outputs: 4 mm safety sockets.

Bus inputs/outputs: 2 mm safety sockets.

Additional recommended elements (Not included):

• N-KNX23. KNX/EIB Shutter Motor Module.

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

Nominal power: 100 W.

• N-KNX21. KNX/EIB Room Controller Module.

KNX bus voltage: 24 VDC.

Bus inputs/outputs: 2 mm safety sockets.

• N-KNX30. KNX/EIB Touch Panel Module.

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

KNX bus voltage: 24 VDC.

Mains inputs/outputs: 4 mm safety sockets.

Bus inputs/outputs: 2 mm safety sockets.

• N-KNX25. KNX/EIB Single Line Extending Bus Module.

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

KNX bus voltage: 24 VDC.

Mains inputs/outputs: 4 mm safety sockets.

Bus inputs/outputs: 2 mm safety sockets.

Interconnection with other KNX/EIB systems.

• N-KNX2. KNX/EIB Additional Power Supply Module.

KNX bus voltage: 28 VDC.

Bus inputs/outputs: 2 mm safety sockets.

• N-KNX32. KNX/EIB Weather Station Module.

Nominal voltage: 230 VAC.

Frequency: 50 Hz.

KNX bus voltage: 24 VDC.

Mains inputs/outputs: 4 mm safety sockets.

Bus inputs/outputs: 2 mm safety sockets.

Different configurations:

Day/night.

Winter/summer.

This module allows measure:

Outside temperature.

Wind speed.

Level of external brightness.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.)  - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the fundamentals of a KNX/EIB installation bus.

2.- Programming of ON / OFF smart switches to demonstrate KNX/EIB switching operations.

3.- Programming of switching for lights control.

4.- Study of preparation and installation methods of a KNX/EIB project.

5.- Utilization of conventional switches in a KNX/EIB project.

6.- Programming of the control of shutter and curtains.

7.- Programming the three way switching equivalent circuit.

Some practical possibilities with additional recommended elements:

8.- Programming the touch panel according to different scenes.

9.- Checking all parameters in the touch panel.

10.- Centralized control of the main configurations in the touch panel.

11.- Extending a bus system through a single-line topology.

12.- Manual control over the main actuators.

13.- Study of the weather conditions control against disturbances variables.

14.- Programming of lighting regulation control.

Optional

AEL-KNX1/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.