TENDER SPECIFICATIONS

The “PHIP-UB” unit includes the following elements:

• N-ALI02. Domestic Main Power Supply Module.

Supply voltage (single-phase): 230 VAC, 1PH + N.

ON / OFF removable key.

Output voltage connections:

Two single-phase: 230 VAC.

Single-phase supply hose connecting plug.

Differential magnetothermal 2 poles, 25 A, 30 mA CA 6 KA.

Emergency stop push-button.

Required elements to operate with “PHIB-UB” base unit (at least one) (Not included):

• PHIP-K1. Analysis of Series and Parallel Photovoltaic Modules Kit.

• PESS. Solar Photovoltaic Unit with Solar Path Emulation.

Photovoltaic panel:

Nominal power: 20 W.

Voltage at maximum power: 17.8 V.

Open circuit voltage: 22.25 V.

Nominal current: 1.12 A.

These values may vary according to manufacturer.

Halogen lamp: 500 W.

• N-EAL-DC. DC Network Analyzer Unit.

DC analyzer.

DC voltage measurement: 0 – 100 VDC.

DC current measurement: 0 – 10 A.

ON / OFF switch.

Connection terminals.

• N-REV01. Single-Phase Variable Resistor Module 0-700 Ohm.

Variable resistor of 0 – 700 Ohm.

Maximum power: 500 W.

Potentiometer.

Fuses: 2 x 6.3 A.

Terminals:

Three terminals to choose maximum resistance or variable resistance.

• N-SSM. Solar Simulation Module.

3 photovoltaic panel simulators.

No load voltage: 24 V.

3 potentiometers for the adjustment of the simulated irradiance.

3 DC ammeters.

3 DC voltmeters.

Bypass diodes.

Short circuit protection.

• PHIP-K2. Off-grid Single-Phase Photovoltaic Systems Kit.

• BAT6. Lead-Acid Battery 6.

Battery voltage: 12 V.

Charge and discharge current: 23 A.

Battery capacity: 7 Ah.

• N-MED85. DC Ammeter Module (-6+6 A).

Measurement range: - 6 A +6 A.

Terminals:

Measuring terminals.

• N-MED86. DC Voltmeter Module (0-15 V).

Measurement range: 0 – 15 V.

Terminals:

Measuring terminals.

• N-REG02. Current Electronic Regulator Module 2.

Automatic detection of operating voltage of 12 V or 24 V.

Monitoring of parameters:

Voltage.

Current.

Charge level of the battery.

Charging current.

State.

Devices with self-protection, battery and loads.

• N-SWIN. Sine Wave Inverter.

Nominal power: 375 VA.

Input voltage: 12 VDC.

Output voltage range: 210 – 245 VAC.

• N-EALD. Electrical Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-LAM34. 3 AC Lamps Module.

Lamps voltage: 230 VAC (PH + N).

Conventional lamp: 25 W.

Low consumption lamp: 4 W.

LED lamp: 2 W.

Three switches.

• N-LAM35. 2 DC Lamps Module.

Lamps voltage: 12 VDC.

Halogen lamp: 25 W.

LED lamp: 2 W.

Two switches.

• PHIP-K3. Single-Phase Photovoltaic Systems for Grid-Parallel Operation Kit.

• N-EALD. Electrical Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-INV04. Single-Phase Grid Inverter Module 4.

Input voltage: 40 – 80 V.

Power: 350 W.

Output voltage: 230 VAC/50 Hz.

Maximum Input current : 9 A.

Efficiency: 95 %.

Emergency disconnection for grid protection.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 640 x 320 x 670 mm aprox. (25,19 x 12,59 x 26,37 inches approx). - Weight: 35 Kg aprox. (77,16 pounds approx).

EXERCISES AND PRACTICAL POSSIBILITIES

Practical possibilities with the PHIP-K1 kit:

1.- Daily and yearly curves display.

2.- Calculation of the photovoltaic modules optimal position.

3.- Display of the photovoltaic modules characteristic curves.

4.- Working of bypass diodes.

5.- Series and parallel configuration of the photovoltaic modules.

6.-  Analysis of the simulated generation with different configurations of the photovoltaic modules and different solar irradiation conditions.

Practical possibilities with the PHIP-K2 kit:

7.- Installation of photovoltaic plants.

8.- Measuring the electrical parameters of the photovoltaic generation system.

9.- Designing and testing a photovoltaic system isolated from the grid in direct operation.

10.- Designing and testing a photovoltaic system isolated from the grid in storage operation.

11.- Designing and testing a photovoltaic system isolated from the grid with AC voltage supply and energy storage.

Practical possibilities with the PHIP-K3 kit:

12.- Installation of photovoltaic plants.

13.- Measuring the electrical parameters of the photovoltaic generation system.

14.- Designing and testing grid-connected (or parallel) photovoltaic systems.

15.- Determining the industrial inverter efficiency.

16.- Analysis of the photovoltaic system response to a grid black-out.

- Several other exercises can be done and designed by the user.

Optional

AEL-PHIP/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.