TENDER SPECIFICATIONS

The “AEL-SPTT” application includes the following elements:

• N-VPS01. 3PH AC Power Supply Module.

ON / OFF removable key.

Supply voltage: 3 x 400 VAC + N.

Emergency stop push-button.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Fuses: 3 x 6 A.

Motorized variable autotransformer:

Nominal power: 3000 VA.

Input voltage: 400 VAC.

Fixed output voltage: 3 x 400 VAC+N.

Variable output voltage:

0 – 450 VAC (3PH + N).

0 – 240 VDC.

Nominal current: 2 A.

Three-position switch with automatic return:

Position I: Voltage reduction.

Position 0: Constant voltage.

Position II: Voltage increase.

DC ammeter: 0 – 5 A.

AC ammeter: 0 – 5 A.

DC voltmeter: 0 – 300 V.

AC voltmeter: 0 – 500 V.

Autotransformer.

Nominal power: 3000 VA.

• N-MED22. AC Voltmeter  Module (0-400 V).

Measurement range: 0 – 400 VAC.

Terminals:

Measurement terminals.

• MED65. Digital Multimeter (2 units).

This module has a digital multimeter of 3 ½ digits, with double-jack ending cables of 4 mm to facilitate interconnections.

With this digital multimeter we will be able to measure:

Voltage.

Current.

Resistance.

Capacitors capacity.

Temperature.

• TRA31. 1 kVA Single-Phase Transformer.

Nominal supply voltage: 230 VAC.

Nominal output voltage: 230 VAC.

Nominal current: 4 A.

Nominal power: 1 kVA.

• N-REV01. Single-Phase Variable Resistor Module 0-700 Ohm.

0 – 700 Ohm variable resistor.

Maximum power: 500 W.

Potentiometer.

Terminals:

Three terminals for taking the maximum resistance or a variable value.

• N-CON. Variable Capacitor Module.

Capacitance: 7 µF.

Nominal voltage: 230 VAC.

Switch:

Position 0: Circuit opened.

Position 1: Circuit closed.

• N-IND. Single-Phase Inductance Module.

Inductance: 1.4 H.

Switch:

Position 0: Circuit opened.

Position 1: Circuit closed.

Fuse: 2 A.

• N-MED09. AC Ammeter (0-2.5 A) Module (2 units).

Measurement range: 0 - 2.5 A.

Terminals:

Measurement terminals.

• N-MED06. Single-Phase Wattmeter Module ( 0-1000 W, 230 VAC, 5 A).

Nominal voltage: 230 VAC.

Terminals:

Measuring terminals.

• N-MED07. Single-Phase Wattmeter Module (0-80 W, 230 VAC, 5 A).

Nominal voltage: 230 VAC.

Terminals:

Measuring terminals.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 1500 x 300 x 1400 mm approx. (59,05 x 11,81 x 55,11 inches approx.). - Weight: 50 Kg approx. (110 pounds approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Calculation of transformer ratio.

2.- Carrying out the open circuit test.

3.- Carrying out the short-circuit test.

4.- Determination of equivalent circuit.

5.- Determination of the voltage drop due to a resistive load.

6.- Determination of the voltage drop due to an inductive load.

7.- Determination of the voltage drop due to a capacitive load.

8.- Study the efficiency of the transformer.

9.- Determination of load which allows the maximum efficiency of the transformer.

- Several other exercises can be done and designed by the user.

Optional

AEL-SPTT/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.