TENDER SPECIFICATIONS (for main items)

1 AEL-TI-07. Application:

• EMT6/1K. Independent Excitation 3PH Synchronous Motor-Generator, 1 kW.

Nominal power: 1000 VA.

Power factor: 0.8.

Nominal output voltage: 3 x 400 VAC.

Frequency: 50 / 60 Hz.

Speed: 3000 rpm.

Nominal output current: 0.8 A.

Nominal excitation current: 5 A.

Shaft height: 71 mm.

• N-EALD. Electrical Network Analyzer Module with Oscilloscope and Data Acquisition.

The network analyzer module allows fulfilling measurements, displaying and analyzing all the parameters of the AC electrical networks. It has an LCD screen and push-buttons for the navigation through the different menus. It includes specific software for monitoring current and voltage curves, harmonics display, tariffs programming, alarms programming and electrical parameters storage.

Features:

Multifunctional three-phase power meter:

Three-phase and single-phase voltage. Up to 690 VAC L-L.

Line and neutral nominal current: 10 A.

Active, reactive and apparent power.

Suitable frequencies: 25 Hz, 50 Hz, 60 Hz and 400 Hz.

Display of the V-I vector diagram.

Supply voltage: 85 – 265 VAC.

Energy quality control:

Current and voltage individual harmonics measurement. Up to the 40th harmonic.

Voltage and current THD, TDD and K-Factor.

Maximums and minimums display.

Waveforms display, 128 samples/sec.

Events and data storage:

Harmonics analyzer:

Voltage and current THD, current TDD and K-Factor, up to the 40th harmonic.

Current and voltage harmonic spectrum and angles.

Tariff programming:

Class 0.5S IEC 62053 – 22, active and reactive power in four quadrants.

Measurement of the total and per phase three-phase active, reactive and apparent powers.

Usage time, four energy/demand records of total tariffs.

Eight tariffs, four seasons and four types of days.

Automatic daily report of energy consumption maximums and minimums.

Communications:

RS – 485 communication port.

• N-PSUB2. Power Generation Substation Module 2.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF switch.

Power and signals connection:

Auxiliary connection.

Three-phase turbine supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Three-phase generator supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Three switches to choose between the different modes:

Local and remote control mode.

Manual and automatic speed control.

Manual and automatic excitation control

Three control switches to:

Start and stop the turbine.

Give permission to synchronize the generator with the grid.

Give permission to close 52NET circuit breaker.

Two potentiometers to regulate:

Turbine speed.

Generator excitation.

Emergency stop button.

Two circuit breakers for synchronization and stand-alone operations.

Different terminals to measure.

Ethernet connection: RJ45 communication port for SCADA remote control.

Automatic speed and voltage controller (easygen):

Enables to connect up to 16 electric generators in parallel-island with distribution of active and reactive load and start/stop in function of the load demand.

Enables to connect a generator in parallel with the grid.

Enables different switches control modes, such as opening, closing and synchronization.

Analogical outputs to control voltage and frequency regulators.

Three-phase measurement of the grid and the generator voltage.

Three-phase measurement of the generator intensity and power.

Single-phase measurement of the grid intensity.

Protection system:

Generator:

Maximum/minimum voltage (59/27), maximum/minimum frequency (81O/U), voltage asymmetry, detection of dead busbars, overvoltage (32), load unbalance (46), negative sequence power/reduced power (32R/F), overcurrent by defined curve (50/51), inverse time overcurrent (IEC255), measured ground fault (50N/51N), phase rotation, switches faults.

Network:

Maximum/minimum voltage (59/27), maximum/minimum frequency (81O/U), vector jump, phase rotation.

Six alarms:

Alarm 1: Reverse power.

Alarm 2: Overcurrent.

Alarm 3: Over/undervoltage.

Alarm 4: Inverse time overcurrent.

Alarm 5: Over/under frequency.

Alarm 6: Shutdown alarms.

Four operation signals:

Ready for operation.

Start request.

Synchronization conditions.

Permission for synchronization.

• SERV01. AC Servomotor 1.

Nominal power: 2000 W.

Nominal voltage: 400 VAC.

Nominal speed: 3000 rpm.

Nominal current: 5 A.

• N-VPS01. 3PH AC Power Supply Module.

ON / OFF removable key.

Supply voltage: 3 x 400 VAC + N.

Emergency stop push-button.

Three-phase supply hose with IP44 3PN + E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Fuses: 3 x 6 A.

Motorized variable autotransformer:

Nominal power: 3000 VA.

Input voltage: 400 VAC.

Fixed output voltage: 3 x 400 VAC+N.

Variable output voltage:

0 – 450 VAC (3PH + N).

0 – 240 VDC.

Nominal current: 2 A.

Three position switch with automatic return:

Position I: Voltage reduction.

Position 0: Constant voltage.

Position II: Voltage increase.

DC ammeter: 0 – 5 A.

AC ammeter: 0 – 5 A.

DC voltmeter: 0 – 300 V.

AC voltmeter: 0 – 500 V.

Autotransformer.

Nominal power: 3000 VA.

Elements required (only one) (Not included):

• TPEL/1800W. Single-phase AC/DC Electronic Load of 1800W (3 units).

The TPEL/1800W electrical load stands out for its versatility in operating in both AC and DC. With a wide power range from 0 to 1800VA and an adjustable frequency from 45 Hz to 450 Hz, this electronic source offers exceptional capabilities.

With its advanced measurement capabilities, 7-inch LCD display, and features such as the built-in oscilloscope, the TPEL/1800W is the ideal solution for laboratory testing of most of EDIBON’s unit such as UPS, inverters, AC power supplies, electrical generators and AC electronic components. In addition, its connectivity via LAN and USB interfaces ensures reliable and fast control, adding an additional level of efficiency to its capabilities.

Technical Features:

Frequency Range: 45 Hz~450 Hz.

Power Range: 0~1.8 kVA

Voltage Range: 50-420 Vrms, 15-260 Vrms

Parallel Connection/Three Phase Control: Capacity can be extended up to 43.2 kVA

7 Inch LCD Display: Provides a clear and accessible interface.

Oscilloscope Function: Allows display of voltage and current waveforms.

High Speed AD Sampling: Captures real-time waveforms.

Measurement Modes: Includes Vrms, Vpk, Vdc, Irms, Ipk, Idc, W, VA, VAR, CF, PF and FREQ.

Harmonic Measurement: Up to harmonic 50 (THD-V).

AC Electronic Load: DC/CR/CP modes.

DC Electronic Load: DC/CR/CP/CV modes.

External Analog Control: 0~10 V analog input, with voltage and current monitoring functions.

Protection Functions: OTP, OCP, OVP, UVP and OPP.

Communication Interfaces: LAN and USB, plus external interface for USB flash disk.

• N-REFT/3C. 3 x 300 W Three-phase Configurable Resistors Module.

Configurable star and delta connections.

Three banks with three three-phase resistors of 1600 Ω.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 300) W.

Required element (Not included):

• AEL-PC. Touch Screen and Computer.

Touch screen:

Energy efficiency class: A.

Screen diagonal: 68.6 cm (27 inch (s)).

Power consumption (operating): 26 watts.

Annual energy consumption: 38 kWh.

Power consumption (standby / off) 0.49 watts.

Screen resolution: 1920 x 1080 pixels.

Computer:

Processor number: Intel Core i7-6600U Processor (4M Cache, up to 3,40 GHz).

Cache: 4 MB Intel Smart Cache.

Clock speed: 2.6 GHz.

# Of cores / # of threads: 2/4.

Max. TDP / Power: 15 W.

Memory types: DDR4-2133, LPDDDR3-1866, DDR3L-1600.

Graphics: Intel HD graphics 530.

Slot for PCI Express.

Recommended additional elements (Not included):

• N-ASYB. Analog and Digital Synchronization Module.

Supply voltage: 230 VAC.

Input terminals:

Generation input terminals L1, L2, L3 and N.

Output terminals:

Network input terminals L1, L2, L3 and N.

Synchronization switch:

2 x 24 VDC Connections.

Digital synchronoscope.

Synchronization switch.

Double frequency meter.

Double voltmeter.

400 VAC Lamps.

• N-PPIM1. Instrumentation Module 1.

Three-phase wattmeter:

Measured range: 0 – 1.5 kW.

Three-phase varmeter:

Measured range: 0 – 1.5 KVar.

Voltmeter:

Measured range: 0 – 500 V.

Frequency meter:

Measured range: 45 – 55 Hz.

DC meter:

Measured range: 0 – 10 A.

• N-INDT/3C. 3 x 300 Var Three-phase Configurable Inductances Module.

Configurable star and delta connection.

Inductance: 3 x (3 x 5) H.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 100) Var.

Supply voltage: 230 VAC.

Common point of normally close/open contacts.

• N-CAR19T/3C. 3x300 Var Three-phase Configurable Capacitors Module.

Configurable star and delta connection.

Three banks with three three-phase capacitors of 2 µF.

Nominal voltage: 400 VAC.

Nominal power: 3 x (3 x 100) Var.

• PSV-BPP-SOF. Biomass Power Plants Simulator.

The PSV-BPP-SOF software has been designed by EDIBON with the aim of showing the user the basic principles of operation of Rankine cycle biomass power plants, presenting in a didactic way the elements and parameters present in the generation process, as well as the interrelations between these parameters by means of the mathematical models integrated in the simulator.

The basic principle of biomass power plants is to transform the intrinsic energy of a fuel that can be considered renewable into electrical energy that will be delivered to the grid. Thus, the application offers different levels of training that will provide the user with the essential knowledge and skills on the fundamental principles of operation and functioning of thermal power plants, where this energy transformation takes place. In this way it will be possible to carry out a previous configuration of the operating conditions of the plant, to later carry out the control and management based on these factors.

By means of this software the student will be able to understand that every thermal cycle of power generation or thermal machine can be represented as a function of the net generated power and the incoming or outgoing thermal powers, according to the fundamental equations of thermodynamics for each element and thermal process of the cycle.

• PSV-GPP-SOF. Geothermal Power Plants Simulator.

The PSV-GPP-SOF software has been designed by EDIBON with the aim of showing the user the basic principles of operation of geothermal power plants, presenting in a didactic way the elements and parameters present in the generation process, as well as the interrelations between these parameters through the mathematical models integrated in the simulator.

The basic principle of geothermal power plants is the extraction of heat from the ground in the form of electrical energy to be fed into the grid. Thus, the application offers different levels of training that will provide the user with the essential knowledge and skills on the fundamental principles of operation and functioning of geothermal power plants, where this energy transformation takes place. In this way it will be possible to carry out a previous configuration of the operating conditions of the plant, to later carry out the control and management based on these factors.

• PSV-GSPP-SOF. Gas Power Plants Simulator.

The PSV-GSPP-SOF software has been designed by EDIBON with the aim of showing the user the basic principles of operation of Brayton cycle gas turbine power plants, exposing in a didactic way the elements and parameters present in the generation process, as well as the interrelations between these parameters through the mathematical models integrated in the simulator.

The basic principle of gas turbine power plants is to transform the intrinsic energy of a fuel into electrical energy to be delivered to the grid. Likewise, the application offers different levels of training that will provide the user with the essential knowledge and skills on the fundamental principles of control, operation and functioning of thermal power plants, where this energy transformation takes place. In this way it will be possible to carry out a previous configuration of the operating conditions of the plant, to later carry out the control and management based on these parameters.

• PSV-HPPS-SOF. Hydroelectric Power Plants Simulator.

The PSV-HPPS-SOF software has been designed by EDIBON to show the user the basic principles of operation of hydraulic power plants, exposing in a didactic way the elements and parameters involved in the generation process, as well as the interrelations between these parameters thanks to the mathematical models integrated in the simulator.

The application offers various levels of training that will provide the user with the essential knowledge and skills on the fundamental principles of control, operation and functioning of hydroelectric power plants. In this way it will be possible to carry out a previous configuration of the plant’s operating conditions, to later control and manage it based on these parameters.

• PSV-HSPP-SOF. Heliothermic Solar Power Plants Simulator.

The PSV-HSPP-SOF software has been designed by EDIBON with the aim of showing the user the basic principles of operation of solar thermal power plants, presenting in a didactic way the elements and parameters present in the generation process, as well as the interrelations between these parameters through the mathematical models integrated in the simulator.

The basic principle of solar thermal power plants is the use of heat energy from solar radiation to obtain electrical energy. There are different types of plants such as tower plants and parabolic trough plants. Solar towers consist of a large glass surface in the centre of which is a very high chimney fitted with devices similar to wind turbines. In this way, the application offers different levels of training that will provide the user with the essential knowledge and skills on the fundamental principles of operation and functioning of solar thermal power plants, where this energy transformation takes place.

• All necessary cables to realize the practical exercises are included.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 AEL-TI-07/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

3 Cables and Accessories, for normal operation.

4 Manuals:

This unit is supplied with 7 manuals: Required services, Assembly and Installation, Control software, Starting-up, Safety, Maintenance, Calibration & Practices manuals.

*	References 1 to 4 are the main items: AEL-TI-07 + AEL-TI-07/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Basic concepts of smart grids.

2.- Manual control of voltage and frequency of the generator in a stand-alone grid.

3.- Automatic control of voltage and frequency of the generator in a smart stand-alone grid.

4.- Study of energy requirements and energy generation in a stand-alone grid.

5.- Smart metering of the generated and transported energy

6.- Study of synchronous generator response when a resistive load variation occurs.

7.- Study of synchronous generator response when an inductive load variation occurs.

8.- Study of synchronous generator response when a capacitive load variation occurs.

9.- Synchronization operations with the synchronous generator and the grid.

10.- Representation of power generation waves through the data acquisition system.

11.- Visualization and comparison of the reactive load curves with and without power factor compensation.

12.- Display of measured values of the synchronous generator.

13.- Representation in diagrams of the measured values vs time.

14.- Data processing, analyzing and exporting of generation, lines and load electrical parameters.

Other possibilities to be done with this unit:

15.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

16.- Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

17.- The Computer Control System with SCADA allows a real industrial simulation.

18.- This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

19.- This unit can be used for doing applied research.

20.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

21.- Visualization of all the sensors values used in the AEL-TI-07 unit process.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Technical and Vocational Education configuration

5 AEL-TI-07/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

6 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.