TENDER SPECIFICATIONS

1 EMPTA-1PH/4K. Unit:

Bench-top unit.

Diagram in the front panel with distribution of the elements similar to the real one.

Power single-phase transformer.

Nominal power: 4 kVA, 1 PH.

Primary winding voltage: 110 – 127 VAC.

Secondary winding voltage: 200 – 240 VAC.

Primary winding nominal current: 30 A.

Secondary winding nominal current: 16 A.

Fuse: 32 A.

Frequency: 50/60 Hz.

Single-phase Differential Protection 30 mA, 32 A.

2 EMPTA-UL/CSA. Unit:

Bench-top unit.

Diagram in the front panel with distribution of the elements similar to the real one.

Power single-phase transformer.

Nominal power: 4 kVA, 1 PH.

Primary winding voltage: 110 – 127 VAC.

Secondary winding voltage: 200 – 240 VAC.

Primary winding nominal current: 30 A.

Secondary winding nominal current: 16 A.

Fuse: 16 A.

Frequency: 50/60 Hz.

Single-phase Differential Protection 30 mA, 32 A.

3 EMPTA-3PH/7K. Unit:

Bench-top unit.

Diagram in the front panel with distribution of the elements similar to the real one.

Power Three-phase Transformer.

Nominal power: 7 kVA, 3 PH.

Primary winding voltage: 190 – 240 VAC.

Secondary winding voltage: 380 – 400 VAC.

Primary winding nominal current: 20 A.

Secondary winding nominal current: 10 A.

Fuses: 3 x 16 A.

Frequency: 50/60 Hz.

Three-phase Differential Protection 30 mA, 20 A.

4 EMPTA-3PH/12K. Unit:

Bench-top unit.

Diagram in the front panel with distribution of the elements similar to the real one.

Power Three-phase Transformer.

Nominal power: 12 kVA, 3 PH.

Primary winding voltage: 190 – 240 VAC.

Secondary winding voltage: 380 – 400 VAC.

Primary winding nominal current: 30 A.

Secondary winding nominal current: 20 A.

Fuses: 3 x 32 A.

Frequency: 50/60 Hz.

Three-phase Differential Protection 30 mA, 32 A.

EMPTA-1PH/4K: - Dimensions: 300 x 300 x 300 mm approx. - Weight: 10 Kg approx.

EMPTA-UL/CSA: - Dimensions: 300 x 300 x 300 mm approx. - Weight: 10 Kg approx.

EMPTA-3PH/7K: - Dimensions: 300 x 300 x 300 mm approx. - Weight: 30 Kg approx.

EMPTA-3PH/12K: - Dimensions: 300 x 300 x 300 mm approx.  - Weight: 70 Kg approx.