TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Magnetotherapy instrument:

Application of Magnetotherapy:

Frequency of treatment: 1 - 100 Hz.

Maximum induction: 100 G ± 20 %.

Available outputs: 2.

Two buttons to select the frequency and the intensity of the treahtment.

LCD display for control and visualization of the magnetic field provided.

Gauss meter:

Range: 20 - 200 ± 0.1 % µT.

Bandwidth: 30 - 300 ± 60 Hz.

Mode selector: off, Tesla and Gauss measurements.

Magnetotherapy applicator:

Two solenoid coils of 15 cm.

Solenoid coil of 30 cm.

Human bone model:

Piezoelectric sensors.

Magnetic field tester.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Required elements (Not included):

- MED87. Two Channel Digital Oscilloscope 70 MHz.

- Dimensions: 1000 x 500 x 550 mm approx.  (39,37 x 19,68 x 21.65 inches approx.) - Weight:  25 Kg approx. (55 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the shape of the applied magnetic field.

2.- Study of the polarity of the applied magnetic field.

3.- Study of the intensity of the applied magnetic field according to the position of the 15 cm coils.

4.- Study of the intensity of the applied magnetic field according to the position of the 30 cm coil.

5.- Study of the piezoelectric effect over a magnetic field.

Optional

BIMAG/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.