TENDER SPECIFICATIONS

This unit is common for the different test modules type “BS”, and can work with one or several modules.

All elements are included in a metallic box.

Signal conditioning circuits:

• Amplifiers:

DC amplifiers:

- Three amplifiers.

- Input range: +/-12 VDC.

- Input impedance: 100 KΩ.

- Adjustable gain: 1, 10,100 for the “Amplifier 1“ and “Amplifier 2“.

- Fixed gain: 100 for “x100 Amp”.

AC amplifier:

- Input range: 12 VAC.

- Adjustable gain: 10, 100,1000.

- Bandwidth: 10 - 16000 Hz.

Power amplifier:

- Input range: 12 V max.

- Output current: 1.5 A max.

- Output power: 9W max.

Current amplifier:

- Gain: 10,000.

- Output current: 1 A max.

Buffers:

- Two buffers.

- Input voltage: 12 V max.

- Input impedance: 100 KΩ.

Inverting amplifier:

- Input voltage: 12 V max.

- Input impedance: 100 KΩ.

- Gain: -1.

Differential amplifier:

- Input voltage: 12 V max.

- Inputs impedance: 100 KΩ (Input A) and 200 KΩ (Input B).

- Ad (Differential gain): 1.0.

- Ac (Common mode gain): 0.02 max.

Instrumentation amplifier:

- Input voltage: 12 V max.

- Inputs impedance: 100 KΩ.

- Ad (Differential gain): 1.0.

- Ac (Common mode gain): 0.006 max.

Summing amplifier:

- Input voltage: 12 V max.

- Three inputs.

- Gain: 1.

• Comparators:

Schmitt trigger.

• Filters:

40 kHz filter: Pass-band filter at 40 kHz.

Low-pass filter: selectable cut-off frequencies at 15 Hz, 1.44 Hz, 0.14 Hz.

• Integrator:

Selectable time constants: 100 ms, 1 s, 10 s.

• Differentiator:

Selectable time constants: 10 ms, 100 ms, 1 s.

• “Sample/Hold” circuit:

Time constant: 1ms.

• Converters circuits:

Voltage to current (V/I) converter:

- Output current: ±20 mA max.

- Transfer ratio: 10 mA/V.

Current to voltage (I/V) converter:

- Output voltage: ±2 V (6 V max.).

- Transfer ratio: 0.1 V/mA.

Frequency to voltage (F/V) converter:

- Transfer ratio: 1 V/kHz.

- Maximum input frequency: 10 kHz.

- No linearity: 0.024 %.

Voltage to frequency (V/F) converter:

- Transfer ratio: 1 kHz/V.

- Maximum input frequency: 10 kHz.

- No linearity: 0.024 %.

• Other circuits:

Full-wave rectifier.

40 kHz oscillator:

- Output frequency: 41093 Hz.

- Output amplitude: 5 Vpp.

Alarm oscillator:

- Oscillator frequency: 700 Hz.

- Switch turn off voltage: 2.3 V.

Electronic switch:

- Input voltage: 12 V max.

- Switch voltage: 2.1 V.

- Output current: 500 mA max.

Control circuits:

• PID:

Process control applications.

Independent PID parameters adjustment (Proportional, Integrative and Derivative ).

• Industrial controller:

Input: DC voltage, RTD sensor or Thermocouple.

Output 1: Relay.

Alarm output: Relay.

• Relay:

Double relay.

NO and NC terminal.

12 VDC coil excitation.

• DC dimmer:

Light dimmer or DC motor speed controller applications.

PWM generator.

Frequency and duty cycle adjustable.

Power elements:

• Power supply:

AC voltage range: 0 to 30 VAC.

DC voltage range: 0 to +15 VDC.

Output current: 4 A max.

• DC Source:

Two output for each voltage.

DC voltages:+5 VDC,-5 VDC,+12 VDC,-12 VDC,0 to 12 Var.

Output current: 500 mA max.

• Wave Generator:

Sine, square, triangular and sawtooth waveforms.

Frequency range: 100 to 10000 Hz.

Amplitude range: Adjustable +/- 10V.

• Potentiometers:

Four potentiometers.

Impedance values: 0-1 KΩ, 0-5 KΩ, 0-10 KΩ and 0-20 KΩ.

Power dissipation: 1 W max.

Measuring elements:

• L.E.D. bargraph display:

Input range: 0-5 V.

• Counter/Timer:

Temporization applications.

Counting applications.

• Moving coil meter.

Cables and Accessories, for normal operation.

Manuals: It is supplied with the following: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Additional recommended elements (Not included):

-BS1. Vibrations and/or Deformations Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Strain gauges:

Gauges of a metallic material that vary their resistance depending on the distortion to which they are going to be subjected. They are stacked in  different positions so that during the vibrant bar movement some of them suffer compression and others extensions.

There are three strain gauges modules each one in different positions.

Characteristics: Resistance at 24°C: 120 Ω. Gauge factor at 24°C: 2.120.

Wheatstone bridge:

Wheatstone bridge adapted for connect the strain gauges modules.

Heating element and thermocouple:

Heating element used to produce temperature variations in the vibrant girder and to see how this situation affects the strain gauges.

K thermocouple place near the resistance measures the girder temperature.

Characteristics: Temperature range: -50°C – 350°C.

LVDT Sensor:

Linear displacement sensor, that detects the relative displacement of a ferromagnetic core between the primary and the secondary.

Input voltage range: 10 to 24 VDC.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the Test Module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS2. Temperature Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Bimetallic switch sensor:

Bimetallic contact thermal switch. Opening temperature: 50°C. Closing temperatures: 30 °C. Configuration: NC contact.

Adjustable bimetallic thermostat:

With heater resistor that allows minimizing the differential cycles and preventing overpeaks. Temperature range: 0 °C – 30 °C. NC contact.

Magnetic Block:

Inside of an external case. Composed of a ferrite pipe, with a heater element inside of them.

Incandescent lamp.

Relay AC:

It allows to turn on and off the incandescent lamp placed over the temperature sensors. Voltage and current (nominal): 250 V – 10 A. NO contact. Switching voltage: 12 V.

Capillary thermostat:

Temperature range: 0 °C – 90 °C. Max. bulb temperature: 150 °C. Socket current: 15 A, 250 VAC. NC contact.

Thermocouples:

Three cromel-alumel thermocouples type “K”. One of them is placed near the capillary thermostat and the bimetallic sensor, another on the  adjustable bimetallic thermostat and the third one inside the magnetic collection. Each one of them is used to measure the temperature that  each one of the sensor are controlling. Temperature range: -50 °C – 250 °C.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS3. Pressure Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Linear positioning sensor (Potentiometer):

Resistor range: 500 Ω – 5 KΩ. Operation force: 200 – 750 g.

LVDT sensor:

Sensibility: 780 mV/mm. Power voltage: 5 – 12 VDC. Total path: 2.5 mm.

Differential pressure sensor:

Measurement range: 0 – 30 psi. Sensibility: 3.33 mV/psi. Overpressure: 60 psi. Power supply range: 10 – 16 VDC.

Two strain gauges mounted in a Wheatstone bridge:

Nominal resistor: 25 °C: 120 Ω. Gauge factor: 2.00 – 2.1 typical. Nominal resistor tolerance: + -0.5%.

Manometric pressure sensor:

Measurement range: 0 – 30 psi. Sensibility: 3.33 mV/psi. Overpressure: 60 psi. Power supply range: 10 – 16 VDC.

Absolute pressure sensor:

Measurement range: 2 – 30 psi. Sensibility: -11 mV/psi. Overpressure: 60 psi. Power supply range: 10 – 12 VDC. Air compressor (located inside the steel box): Air flow: 10 l/min Pressure: 1.83 Kg/cm². Power supply: 220V, 50/60Hz.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS4. Flow Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Flow switch:

Monitors the flow and closes an electric circuit. Contact form: N/O. Switching voltage AC: 240 VAC. Switching voltage DC: 120 VDC.

Optical flow sensor:

It gives an output in pulses proportional to the liquid flow. At the output of this sensor we get a pulse signal with a frequency proportional to the flow volume that crosses the sensor.

Power supply: 5 VDC. Measurement range: 0.25 – 6.5 l/min. K factor: 4600 pulses/liter.

Underwater pump: The variation in the pump power supply voltage enables to change the water volume in the test module. Power supply: 12 VDC.

Pressure level sensor:

It is a sensor that measures the pressure caused by the water in relation to the atmospheric pressure, so the liquid level in the tank can be calculated. Pressure range: 0 – 1psi. Sensitivity: 16.7 mV/psi. Power supply: 10 – 16 VDC.

Differential pressure sensor (Hole board system):

This sensor is connected to a hole-board system to measure the pressure difference caused by the volume narrowing of the conduct through which the water flows. On this way, with the measurement of the pressure difference between the hole board water output and input, it is  possible to calculate the water volume that crosses the board.

Measurement range: 0 – 16 psi. Sensitivity: 1.5 mV/psi. Power supply: 10 – 16 VDC.

Changeable flow meter:

Using a small floating buoy that is inside the tube calibrated in liter/minute, it can be read the volume measure flowing through the pipe.

Range: 0 – 2 l/min.

V narrowing:

The connection between the main and the secondary tank, a dam, includes a “V” narrowing. The altitude of the water level above the dam bottom is a very precise measure of the flow relation. The ruler fixed on the right end of the tank will show this height.

Main and secondary tanks.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS5. Ovens Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Oven chamber.

Heating element:

Oven heating element made up of two parallel resistances with a maximum dissipation power of 500 W. The heating element power supply is  of 0-30V AC. Inside the heating element there is a temperature sensor element.

Fan:

Fan with changeable speed that can be operated varying the fan energy supply voltage.

Energy supply voltage: +12 VDC (max). Maximum power: 0.96 W. Maximum air flow: 2.5 l/s.

Thermocouples:

Four thermocouples placed inside the oven, each one of them at a different height. Temperature range: -184 °C – 400 °C.

Platinum resistance thermometer:

Platinum resistance temperature detector, suitable for measuring air and gas temperatures. Temperature range: -70 °C – 600 °C. Resistance  (0 °C): 100+/-0.1Ω.

Thermistor:

NTC thermistor for temperature measurement and control, with great sensitivity and stability. Resistance at 25 °C: 5.8 KΩ. Temperature range:  -40 °C – 125 °C.

Semiconductor temperature sensor:

Reverse polarized diode. The current through the diode depends on the temperature at which balance with the surrounding environment is  achieved. Therefore it needs a conditioning circuit able to transform this current variation in voltage proportional to temperature.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS6. Liquid level Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Water tanks.

Capacitive level sensor:

Level sensor immersed in the tank. Power supply: 12-35 VDC. Output: 4-20 mAdc.

Level sensor by pressure:

It is a differential pressure sensor that measures the pressure practiced by the water compared to the atmospheric pressure to measure the  water level. Pressure range: 0-1psi. Sensibility: 16.7mV/psi.    Excitation Voltage: 10-16 VDC.

Float level switches with potentiometer:

It is a potentiometer fixed to a float arm that will vary its position depending to the water level. This system complements itself with two end and  beginning path switches respectively. Switching voltage contacts: 250 VAC/125 VDC.

Conductivity level sensor:

This sensor works with two electrodes immersed in one of the tanks. As the water level rises and covers the electrodes its resistance will decrease until it arrives to KΩ unit values; as long as the water does not touch the electrodes, the resistance between them will be very big and will behave  like an open circuit.

Magnetic float level sensor:

Sensor formed by a small float that has inside a magnetic element, the float base has a Hall effect element that detects when the float has gone up due to the effect of the water. Switching voltage: 240 VAC, 120 VDC.

Optical level sensor:

It is a photodiode and phototransistor, which in presence of water changes its refraction properties and make the output state approximately change from 3 VDC to 0 VDC. Power supply: 5VDC. Load current: 20 mA max. at 125 °C.

Two pumps:

The volume supplied by these pumps can be regulated varying the dc voltage value with which they are supplied. Power supply: 12 VDC (max. voltage). Nominal volume: 1 l/min. Nominal current: 1 A DC.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS7. Tachometer Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

DC Motor:

Nominal voltage: 12 V. Resistance: 9,7 Oh. Max. VACuum speed: 8500 rpm. Max. load speed: 3500 rpm approx.  Start voltage: 210 mV.

Inductive Sensor:

Output voltage: up to 10 Vpp. Body-housing material: Steel. Operating temp. range: -40 ºC – 60 ºC.

DC Tachometer: Voltage rating: 1.5 V (DC). Power rating: 1.21 W.

Retroreflective optical sensor:

Sensor where an infrared emitting diode and a NPN phototransistor encased side-by-side on coverging optical axes in a black thermoplastic housing. Vo in output connectors of the module: 0.0 – 400 mV for Vs=12 VDC.

Slot Optical Sensor:

Slot optical sensor where an input LED and an output phototransistor are capsulated. Vo in output connectors of the module: 0.0 – 5 V for Vs=5 VDC.

Hall Effect sensor:

Hall-effect position sensor where exist a relationship between supply voltage and the combined effects of a change in sensitivity (gain) and null voltage output at room temperature.

Supply voltage: 4 – 10 V. Supply current: 3.5 mA. Output type: Differential. Output voltage: 0.25 V – 2 V. Sensitivity: -130 – 130 gauss; 0.75 – 1.06 mV/gauss. Vo in output connectors of the module: 0.0 – 1V for Vs=5 VDC.

Encoder:

This optical encoder contains a LED source, an integrated circuit with detectors and output circuitry, and a codewheel which rotates between the emitter and the detector IC. Supply voltage: -0.5 – 7 V.

Output voltage: -0.5 – VDC. Output current per channel: -1 – 5 mA. Velocity: 30000 rpm.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices  Manuals.

Each module may operate independently of another.

-BS8. Proximity Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

DC Motor: Nominal power supply: 12 VDC.

Proximity capacitive sensor:

It can detect metallic objects.

Detection distance: 10 mm. Output: 10 – 60 V Imax = 200 mA. Power supply voltage: 10 – 60 V.

Hall effect sensor:

Proximity switch using the Hall effect, switching when there is a magnetic field. Power supply voltage: 5 VDC. Magnetic flux density: works at 22 mT (35 mT max).

Retroreflective optical sensor:

Emission narrow beam GaAs IR Emitter. Detection narrow beam IR Photodetector.

Emitter: VF(max): 1.7, VR (min): 3 V, radiation power: 4.8 mW, peak wavelength: 935 nm. Receiver: Vc (max): 12 VDC, Ic (min): 8 mA, Darkness current: 100 nA.

Transmission infrared sensor:

Emission narrow beam GaAs IR Emitter. Detection narrow beam IR Photodetector.

Emitter: VF (max): 1.7, VR (min): 3V, radiation power: 4.8 mW, peak wavelength: 935 nm. Receiver: Vc (max): 30 V, Ic (min): 8 mA, Darkness current: 100 nA.

Conduction sensor:

Proximity sensor with plate sensible to magnetic fields. Contact material: Rhode. Output: NO-NC. Breaking voltage: 400V DC or AC current (max) 0.6 Amp.

Inductive sensor:

Sensor that gives variations in the output voltage as a variation of the magnetic field, caused by the near ferromagnetic material movement.

Inductance: 12 mH. Winding Resistance: 130 Oh. Detection distance: 2 mm.

Ultrasonic sensor:

Transmitter sensibility: 106 dB. Receiver sensibility: -65 dB. Resonance frequency: 40 kHz. Operation distance: 40 cm. Output voltage: 20 V rms.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals:  It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS9. Pneumatic Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Proportional valves (1 and 2):

Nominal voltage: 24 VDC. Pressure range: 8 bar maximum, 0 – 6 bar control. Linearity: 1% full scale.

Differential pressure sensor: Measurement range: 0 – 30 psi. Sensitivity: 3.33 mV/psi. Power-supply range: 10 – 16 VDC.

Pneumatic switch: Activation: 20 – 24 VDC. Positions: 2. Maximum pressure: 6 bar.

LVDT Sensor: Power-supply voltage: 9 to 24 VDC. Sensitivity: 60 mV/mm/10 VDC.

Regulation filter: Manual drainage. Maximum input pressure: 8 bar. Flux: 14.5 dm³/s.

Air compressor:

Air flow: 10 l/min. Pressure: 1.83 Kg/cm².

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

-BS10.Light Test Module.

Painted steel box.

Connection diagrams for each transducer are represented graphically.

Lamp:

Voltage: 12 V. Power 5 W.

Photodiode:

Power: 250 mW max. Sensibility: 0.34 A/N. This sensor converts light into either current or voltage, depending upon the mode of operation.

Phototransistor:

Current collector: 20 mA max. Peak wavelength: 570 nm. It also consists of a photodiode with internal gain.

Light Dependent Resistor:

Power dissipation: 250 mW max. Peak wavelength: 550 nm. A LDR is a resistor whose resistance decreases with increasing incident light intensity.

Photovoltaic Cell:

Power: 250 mW max. Peak wavelength: 550 nm. A photovoltaic cell converts solar radiation into direct current electricity.

Infrared emitter-receiver:

Power: 470 mW max. Max. current: 200 mA. Peak wavelength: 880 nm. This element consists of an IR emitter LED and IR phototransistor.

Sensor connections with the base unit and with power supplies are through 2 mm terminals located in the front panel of the test module.

Manuals: It is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Each module may operate independently of another.

Dimensions: 490 x 450 x 470 mm approx. (19.29 x 17.72 x 18.50 inches approx.). Weight: 30 Kg approx. (66 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Practices to be donde with the Vibrations and/or Deformations Test Module (BS1):

1.- To measure the vibration of a vibrant girder using strain gauges.

2.- To use a heating element to rise the girder temperature in order to study the effect on the sensors. (Thermocouple and heating element).

3.- To detect the displacement of the BS1 module vibrant girder using a LVDT sensor.

4.- Effect of temperature variation on the strain gauges.

5.- Effect of deformation on the resistance of a . girder

6.- Measurement of the three deformation dimensions or deformation of spherical or cylindrical coordinates systems.

7.- Linear variable differential transformer (LVDT) for measuring displacements.

8.- Analysis of how to compensate the variation of resistance of a strain gauge due to temperature variations, using compensating gauges.

9.- Linear variable differential transformers (LVDT) as a weighing system.

10.-Effect on the vibration of a with different masses.

Practices to be donde with the Temperature Test Module (BS2):

11.- How to use the Curie effect as application of a high temperature thermostatic controller.

12.- Adjustable bimetallic thermostat. To use the bimetallic thermostat as a temperature controller, calculating its hysteresis.

13.- Adjustable bimetallic thermostat. How we can reduce the hysteresis by adding a resistor to the heating circuit.

14.- To use the thermostat based on a bimetallic sensor to control the temperature.

15.- To use the capillary thermostat.

Practices to be donde with the Pressure Test Module (BS3):

16.- Use of linear positioning sensor (potentiometer) to detect the displacement produced by a diaphragm expansion caused by the air pressure.

17.- Use of a LVDT as an element to measure the diaphragm distortion that is consequence of the pressure inside the pressure chamber.

18.- Differential pressure sensor with hole-board system. Use of a differential pressure sensor of the semiconductor type to measure the pressure fall in a hole-board system.

19.- Strain gauges. To detect objects using an infrared sensor by light beam interruption.

20.- Measure the pressure in the chamber, using two different types of sensors (manometric and absolute pressure sensor).

21.- Strain gauges for measuring deformations: their resistance changes as the diaphragm expands due to the pressure coming from the pressure chamber.

Practices to be donde with the Flow Test Module (BS4):

22.- To measure the water volume produced by an underwater pump in the module using an optical flow sensor.

23.- To use a high-resolution optical flow sensor to measure low flow volumes.

24.- Pressure level sensor. To use a differential pressure sensor to measure the liquid level in one of the tanks.

24.- Differential pressure sensor. To measure the pressure-fall in the module hole board system, as a necessary parameter to determine volume.

26.- To measure the flow volume generated by the underwater pump using a flow meter of changeable area.

27.- To obtain the flow-volume value in the secondary tank using the V narrowing weir.

Practices to be donde with the Ovens Test Module (BS5):

28.- Heating element. Raise the oven internal temperature, over the environmental temperature, using a heating element to do tests and practices related with temperature measurement.

29.- To use a fan as refrigerating element of the oven.

30.- To use thermocouples as temperature sensors elements inside the oven. Temperature measurement using a thermocouple.

31.- To measure temperature inside the oven using a platinum resistance thermometer.

32.- To measure temperature inside the oven using a thermistor temperature sensor.

33.- Temperature measurement using a thermistor, based on its negative temperature coefficient.

34.- To obtain the temperature value inside the oven using a semiconductor sensor (diode).

35.- PID control.

Practices to be donde with the Liquid level Test Module (BS6):

36.- To use a capacitive sensor to measure the liquid level in the tank.

37.- To use the differential pressure sensor as an element to determine the water level in a tank.

38.- To use a changeable resistance fixed to a float system as a liquid level measurement element.

39.- Conduction Sensor. Use of a sensor made up of to steel electrodes to measure the water level of a tank.

40.- Magnetic float level sensor. It detects a precise liquid level in the tank with a magnetic contact.

41.- Control of the module left tank liquid level using an optical level sensor.

Practices to be donde with the Tachometer Test Module (BS7):

42.- DC Motor. Provide the group of sensors of the BS7 module fixed to the central axle of the equipment with movement power.

43.- DC Tachometer. To use a DC motor as a tachometer to measure the revolutions of the BS7 module central axle.

44.- Inductive Sensor.

45.- Retroreflective optical sensor. To measure the central axle revolutions of the module using a light reflection optical sensor.

46.- To obtain the central axle speed value using a slot optical sensor through light interruption.

47.- To obtain the central axle speed value using a Hall-effect position sensor.

48.- To measure the central axle revolutions of the module using the encoder.

Practices to be donde with the Proximity Test Module (BS8):

49.- How to use a capacitive sensor to detect metal objects as the pass in front of the sensor.

50.- To use a Hall effect sensor as an element to detect the presence of magnetic objects.

51.- Retroreflective optical sensor. To use an optical sensor that works through infrared light reflection.

52.- Infrared sensor by transmission. To detect objects using an infrared sensor by light beam interruption.

53.- Conduction sensor. To detect magnetic objects using a REED switch sensor.

54.- To detect the presence of ferrous object using an inductive sensor.

55.- Ultrasonic sensor. To detect metallic and non-metallic object using high frequency sounds.

Practices to be donde with the Pneumatic Test Module (BS9):

56.- Proportional valves. To control electronically the vertical displacement of a double effect pneumatic piston using proportional valves.

57.- Differential pressure sensor. To use a pressure sensor for measuring the pressure difference between both pneumatic piston air inlets.

58.- Pneumatic switch. To deflect the air flow using a pneumatic switch.

59.- LVDT Linear Displacement Sensor. To measure pneumatic piston displacement using an excitation and DC output LVDT.

Practices to be donde with the Light Test Module (BS10):

60.- Study of the equivalent electrical circuit of a photodiode. Study the V-I characteristic of a photodiode.

61.- Study of the normal operation mode of a photodiode. Study the “ON/OFF“ operation (light switch) of a phototransistor.

62.- Measurement of light intensity using a solar cell.

63.- Study of the properties of light dependent resistors (LDR).

64.- Study of the operation of IR sensors.

65.- Study of a real application for controlling the light intensity using PID control elements.

Optional

BSUB/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.