TENDER SPECIFICATIONS

CE00 Pilot Plants:

Four complete production lines for the elaboration of the main products derivated from wheat.

Real units used in flour, bread and pasta industry.

Sensors actuators which allows the study and understanding of the different process of the cereal industry.

Modular layout to allow adaptation to different spaces.

1 CE00/MF. Pilot Plant for Milling and Flour Production:

GM/CTS. Computer Controlled and Touch Screen Grain Milling Machine.

Mill with stone crusher

Hourly production: 50 kg.

Power: 1.8 kW.

Grinding wheel diameter: 250 mm.

Feeding hopper with 15 kg capacity.

Manually adjustable degree of fineness.

Oven-painted steel support structure.

Adjustable height.

Flour cone for visibility and touch.

FT/CTS. Computer Controlled and Touch Screen Pneumatic Transport System.

Decanting cyclone.

Cast iron star valve.

Helical gear motor.

Power: 0.18 kW.

Centrifugal fan.

Power: 0.75 kW.

Exhaust air filter sleeves.

FS/CTS. Computer Controlled and Touch Screen Industrial Sifter.

Magnetic pulse rotor.

Power: 0.15 kW.

Anti-dust ball bearings.

Interchangeable sieves.

Three ejection ramps.

Diameter: 150 mm.

Magnetic grid for the separation of metallic impurities.

2 CE00/B. Pilot Plant for the Production of Bread:

AWD/CTS. Computer Controlled and Touch Screen Automatic Water Dosing Device.

It can work automatically or manually.

Connection to network or water cooler.

Electronic selector and luminous screen.

A siphon system increases its precision.

DK/CTS. Computer Controlled and Touch Screen Dough Kneader Unit.

Blades and dough-holder pan in stainless steel, the rest of the elements in high quality steel.

Provides up to 30 equal divisions of portions up to 150 g.

Tabletop unit.

WCO. Water Cooler.

Temperature range: 3 - 5 °C.

Capacity: 175 l.

Production: 70 l/h.

Power: 0.9 kW.

Injected polystyrene tank and AISI 430 stainless steel frame.

Electric agitator.

Injected polyurethane insulation.

Stainless steel coil.

Water level control.

DD/CTS. Computer Controlled and Touch Screen Dough Division Unit.

Arm kneader for bread production.

Capacity: 40 kg of dough.

Power: 1.2 kW.

Solid construction.

Painted cast iron chassis.

Trough and arms in stainless steel.

Kneading by means of arms that imitate the manual work of the baker.

Plastic hood that avoids the flour in suspension.

Aluminum legs with silentblock to avoid vibrations and give stability to the machine.

Capacity: 40 kg of dough.

Power: 1.2 kW.

Masses admitted with a minimum of 55% water.

Machine mounted on wheels with stabilizing feet.

2 speeds: a first for mixing and a second for kneading.

BM. Baguette Maker Unit.

Power: 0.55 kW.

Production up to 2700 pieces/hour.

Outer covers made of stainless steel.

Bands in non-hygroscopic fabric.

Solid acetal rollers.

Belt carrying rollers with conical ends that ensure the centering of the belts.

Exit of baguettes on a fixed table of 200 mm.

Manufacture of bars up to 700 mm long.

CBO/CTS. Computer Controlled and Touch Screen Convection Bread Oven.

Electric oven for baking bread by forced air.

Reversible turning turbine.

Electronic control of temperature and baking time.

Adjustable steam time at the beginning of the baking.

Motorized steam output.

Built entirely in stainless steel.

COOL TOUCH cold door.

Capacity for five trays of 600 x 400 mm.

Separation between trays. 76 mm.

Power: 7.5 kW.

Additional recommended elements (Not Included):

For CE00/B:

AWD/CTS. Computer Controlled and Touch Screen Automatic Water Dosing Device.

It can work automatically or manually.

Connection to network or water cooler.

Electronic selector and luminous screen.

A siphon system increases its precision.

TTO. Trolley and Set of Trays for Oven.

Four wheels of double turn for easy movement.

Capacity for 22 trays of 600 x 400 mm.

20 perforated and corrugated aluminium trays:

With non-stick bi-layer silicone rubber.

Four longitudinal channels.

Dimensions: 600 x 400 mm.

CV. Vapour Condenser.

Prepared for use in places where there is no possibility of evacuating cooking vapours to the outside.

Water cooling.

Condensation of the vapours and evacuation by drainage.

Power: 30 W.

3 CE00/P. Pilot Plant for the Production of Pasta:

PEK/CTS. Computer Controlled and Touch Screen Extruder and Kneader.

Production: 25 - 30 kg/h.

12 kg feeding hopper.

Worm screw feeder in stainless steel.

Removable mixing arm.

Pre-drying fan.

Water cooling device.

Electronic knife for cutting short pasta.

Four standard matrixes included.

SPD/CTS. Computer Controlled and Touch Screen Static Drying Unit.

Capacity: 100 kg.

Trolley with 25 wooden and aluminium trays.

Electrical resistors.

Polyurethane foam insulation.

Two motorized fans with reverse flow.

Two extractors with electronic valves.

Power: 5.3 kW.

Microprocessor-based cycle management:

Programmed cycles.

Possibility of programming new cycles.

4 CE00/MA. Pilot Plant for Cereal Malting:

GCS/CTS. Computer Controlled and Touch Screen Grain Cleaning and Steeping Unit.

Built in AISI 304 and AISI 316 stainless steel.

Truncated-conical tank with drain in the lower part that allows the tank to be completely emptied.

Volume: 800 l.

Water jacket for internal temperature control.

External insulation.

Gate and grid system in the lower part that allows draining the water inside the tank and retaining the grain.

This allows wet and dry periods during drainage.

Aeration system by means of compressor:

Power: 300 W.

CO2 aspiration system in dry periods by means of centrifugal fan:

Power: 90 W.

Maximum flow: 265 m3/h.

Maximum pressure: 600 Pa.

Overflow that allows the cleaning of the grain and elimination of impurities.

Surface spray nozzles to maintain humidity during dry periods.

Temperature control system:

Heat pump (allows cooling and heating):

Electrical power: 1.5 kW.

COP: 5.32.

R32 refrigerant.

Circulation pump:

Power: 370 W.

Control by touch PLC:

Control and programming of wet and dry periods.

Temperature control.

Monitoring of process variables.

Instrumentation:

Six temperature sensors.

CO2 concentration, humidity and temperature sensor in the

suction stream.

pH sensor.

Three level switches.

Three solenoid valves.

GGER/CTS. Computer Controlled and Touch Screen Grain Germination Unit.

Built in AISI 304 and AISI 316 stainless steel.

Germination tank:

Surface area: 1.5 m2.

Perforated plate bottom that allows the homogeneous distribution of the inlet air.

Two cleaning doors.

Door for emptying system.

External insulation.

Stirring system for removing and turning the grain:

Power: 1.1 kW.

Includes frequency inverter.

Aeration and humidification system.

Centrifugal fan:

Power: 1.1 kW.

Built in stainless steel AISI 304.

Includes frequency inverter.

Maximum flow: 650 m3/h.

Maximum pressure: 3000 Pa.

Air temperature control system:

Heat pump:

Electrical power: 1.5 kW.

COP: 5.32.

R32 refrigerant.

Two water circulation pumps:

Power: 370 W.

One of them includes frequency inverter.

Heat exchangers with finned tubes.

Accumulator tank:

Built in stainless steel AISI 316.

Volume: 100 l.

Nozzle system for air humidification.

Control by touch PLC:

Control and regulation of operating temperature and humidity.

Control of the stirring speed.

Monitoring of process variables.

Instrumentation:

Six temperature sensors.

CO2 concentration, humidity and temperature sensor inside the germination tank.

pH sensor.

Level switch.

Three solenoid valves.

Slot sensor for measuring the speed of rotation of the blades system.

GDT/CTS. Computer Controlled and Touch Screen Grain Drying and Toasting Unit.

Constructed entirely of AISI 304 and AISI 316 stainless steel.

Drying cylinder:

Volume: 1000 l.

Perforated plate bottom for correct distribution of hot air.

External insulation.

Cleaning door.

Emptying door.

Centrifugal fan:

Power: 3 kW.

Includes frequency inverter.

Manufactured in AISI 304.

Allows working temperatures up to 250 °C.

Maximum flow: 3000 m3/h.

Maximum pressure: 3500 Pa.

Heating resistor block for air heating:

Power: 22.5 kW

Possibility of increasing or decreasing power.

Consult us for more information.

Air-air heat exchanger that allows to recover

part of the heat from the exhaust gas:

Made of stainless steel AISI 316.

Temperature range: 50-150 °C.

Allows brewing of all types of malts: pale,

roasted and black.

Recirculation system:

Butterfly valve for external air inlet control.

Two butterfly valves with pneumatic control to regulate the circulation rate.

Two flow meters by Pitot tube.

Paddle system for emptying the dryer:

Power: 250 W.

Control by touch PLC:

Control and regulation of temperature, air flow and recirculation ratio.

Monitoring of process variables.

Instrumentation:

Temperature sensor.

Two humidity and temperature sensors before and after the bed.

Two flow meters.

Pneumatic positioners and actuators for recirculation rate control.

DCLM/CTS. Computer Controlled and Touch Screen Deculmer.

Rotating drum built in AISI 304 stainless steel:

Volume: 400 l.

Two loading and unloading doors.

Geared motor:

Power: 1.5 kW.

Includes frequency inverter.

Germ collection tray.

Hopper for cereal unloading.

Positioner for drum blocking. Includes limit switch system.

Slot sensor for rotation speed measurement.

Control by touch PLC:

Rotation speed regulation.

Monitoring of process variables.

RCB. Rising Conveyor Belt.

Allows the transport of grain between the different stages of the malting plant.

Adjustable discharge height.

Mobile structure mounted on wheels.

Geared motor:

Power: 1.1 kW.

Start and stop system.

Emergency stop system.

Easy cleaning system.

SCH. Horizontal Screw Conveyor.

Allows the cereal to be emptied from the germinator and dryer, and feed the RCB.

Mobile structure mounted on wheels.

Geared motor:

Power: 750 W.

Start and stop system.

Emergency stop system.

The complete pilot plant includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

5 Cables and Accessories, for normal operation.

6 Manuals:

Each unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*	References 1 to 6 are always included in the minimum supply (according to choice): CE00/MF: GM/CTS,  FT/CTS and FS/CTS + CE00/B: AWD/CTS, DK/CTS, WCO, DD/CTS, BM and CBO/CTS + CE00/P: PEK/CTS and SPD/CTS + CE00/MA: GCS/CTS, GGER/CTS, GDT/CTS, DCLM/CTS, SCH and RCB + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Study of the conditions of wheat milling.

2.- Study of the relation between the final size of the flour grain and the milling stones distance.

3.- Study of the milling of different types of cereals.

4.- Study of the separation of solids in a sifter.

5.- Study of the separation of the flour according to the type of cereal.

6.- Study of the separation speed as a function of the sieve rotation speed.

7.- Study of a pneumatic system for transporting solids.

8.- Study of the kneading process.

9.- Study of the influence of the flour-water-yeast ratio in bread making.

10.- Study of the influence of kneading time.

11.- Study of the time and conditions of fermentation in the elaboration of bread.

12.- Study of the process of weighting and forming.

13.- Calculation of the error produced in the division and weighing of the dough.

14.- Study of the influence of the time of fermentation in the dough.

15.- Study of the forming process in bread doughs.

16.- Making of bread loafs of different shapes.

17.- Influence of the separation of the lamination rolls.

18.- Influence of the baking temperature on the resulting bread.

19.- Influence of the baking time on the resulting bread.

20.- Influence of the heating ramp on the resulting bread.

21.- Variation of the baking conditions according to the type of bread.

22.- Study of the wheat pasta extrusion process.

23.- Study of the pressure in the extrusion process.

24.- Elaboration of fresh pasta with different molds.

25.- Differences between the elaboration of long and short fresh pasta.

26.- Study of the drying process of pastes.

27.- Determination of the drying time.

28.- Differences in the drying process between several types of pasta.

29.- Evolution of the temperature and the humidity in the drying process.

30.- Study of the barley steeping process.

31.- Influence of temperature on water absorption.

32.- Influence of the pH in the humidification process.

33.- Study of the production of CO2 in the process of steeping and germination.

34.- Study of the influence of the aeration flow in the steeping process.

35.- Evolution of humidity during the steeping stage.

36.- Study of the operation of an auger transport system.

37.- Study of the influence of temperature on the germination process.

38.- Study the influence of humidity on the germination process.

39.- Study of the influence of the stirring in the process of germination.

40.- Study of the influence of the air flow on the germination process.

41.- Study of the operation of a heat pump.

42.- Study of the operation of a cereal dryer.

43.- Study of the drying and toasting times of malt.

44.- Study of the temperature evolution in the drying process.

45.- Study of the influence of temperature on the malt during the roasting phase. Production of pale, roasted and dark malts.

46.- Study of the influence of gas recirculation in the dryer.

47.- Study of the deculmer process.

48.- Calculation of the percentage of culm in the malt.

Additional practical possibilities:

49.- Sensors calibration.