TENDER SPECIFICATIONS

Distribution and transport case for all parts.

The "CGA" unit mainly consists of:

Combined gear:

Transmission ratio (i): 44.1.

Max. output torque (Mmax): 179 Nm a 500 rpm.

Materials:

Case: cast iron.

Shafts: steel.

Gears: alloyed case-hardened steel.

Retainers: nitrile – NBR.

Dimension for driving and driven shafts:

Driving shaft: Ø140 x 11.

Driven shaft: Ø18.

Spur gear stage:

Transmission ratio (i): 2.94.

Pinion:

Number of teeth (z): 18.

Module: 1 m.

Gear wheel:

Number of teeth (z): 53.

Module: 1 m.

Worm gear stage:

Transmission ratio (i): 15.

Worm screw:

Number of teeth (z): 4.

Number of thread starts (e): 2.

Module: 2 m.

Worm wheel:

Number of teeth (z): 30.

Module: 2 m.

Assembly and disassembly tools set:

Tool case with different compartments.

Transparent storage box for small parts.

Nylon hammer.

Lock ring extractor.

Torx screwdrivers.

Fixed keys.

Plans: including an exploded drawing and list of components, and the dimensions of the equipment.

Spare parts kit:

Seeger lock rings.

Oil seals.

O-rings.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Recommended elements (Not included):

- ADSG. Unit to Study the Alignment of Drives, Shafts and Gears.

Dimensions:  223.5 x 115 x 228 mm approx. (tool box) (8.80 x 4.53 x 8.98 inches approx.)

150 x 445 x 310 mm approx. (case) (5.90 x 17.51 x 12.20 inches approx.)

Weight: 30 kg approx. (66.1 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Study of the operation of a combined gear.

2.-  Analysis of the structure of a combined gear.

3.-  Study and analysis of the functions and construction characteristics of the components and assemblies.

4.-  Understanding of technical drawings and plans.

5.-  Gauging and dimensioning components exercises.

6.-  Study and analysis of assembly aids.

7.-  Study and performance of assemblies and complete units.

8.-  Study and determination of the work schedule, mainly the representation and planning of the assembly development.

9.-  Study and analysis of faults and damages.

10.-Repair exercises and maintenance operations for damages and faults.

11.-Understanding the criteria required to select the materials.

Additional practical possibilities:

12.-Analysis of the operation of the assembled gear ("ADSG" unit is required).

Optional

CGA/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.