TENDER SPECIFICATIONS (for main items)

1 EBDC. Unit:

Stainless steel frame and painted steel panels.

The unit includes wheels for easy mobility.

Main metallic elements made of stainless steel.

Diagram on the front panel with similar distribution to that of the elements in the real unit.

Circulation hose in specific polymer for fuels.

Cellulose filter with ten micron pore size.

Purification compartment with ion exchange resin.

Computer controlled single-phase pump specific for diesel fuels:

Stainless steel interior.

Seals in polymer resistant to chemicals and fuels.

Max. flow rate: 80 l/min.

Max. discharge height: 25 m.

Reactor tank:

Truncated cone-shaped vessel made of AISI 304 stainless steel.

Lower outlet for decanting.

Intermediate outlet to obtain biodiesel without previous decantation.

Capacity: 120 l.

Incorporates:

3 kW electric heating resistance, computer controlled.

Two level switches made of stainless steel.

“J” type temperature sensor.

Level sight glass made of stainless steel and glass, incorporates sampling valve.

Mixing tank:

Truncated cone-shaped vessel made of AISI 304 stainless steel.

Lower outlet for decanting.

Capacity: 40 l.

Oil tank:

Fuel resistant high density polyethylene container.

Capacity: 250 l.

Incorporates:

Lid to prevent the deposition of dust and other impurities.

Removable mesh filter for solid particles.

Manual shut-off valve.

Drain valve.

Graduated volumetric scale up to 250 l.

Methylate tank:

Chemical resistant high density polyethylene container.

Capacity: 25 l.

Incorporates:

Lower tap for sampling.

Graduated volumetric scale up to 25 l.

Glycerin tank:

Container made of high density polyethylene resistant to fuels.

Capacity: 60 l.

Incorporates:

Lower tap for sampling.

Graduated volumetric scale up to 60 l.

Biodiesel tank:

Container made of high density polyethylene resistant to fuels.

Capacity: 250 l.

Incorporates:

Lid to prevent the deposition of dust and other impurities.

Removable mesh filter for solid particles.

Manual shut-off valve.

Drain valve.

Graduated volumetric scale up to 250 l.

Instrumentation:

Load cell for measuring the mass of glycerin and biodiesel obtained; range: 0 - 300 kg.

pH sensor for the study of the quality of the biodiesel obtained; range: 0 - 14 pH.

Pressure sensor; range: 0 - 6 bar.

Titration kit:

Bottle suitable for methanol.

Titrant bottle.

Four 7 ml droppers.

Three 250 ml glass beakers.

Pipette with hand pump 10 ml.

Plastic wash bottle 500 ml.

The complete unit includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 EBDC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time PID control with flexibility of modifications from the computer keyboard of the PID parameters, at any moment during the process.

Real time PID control for parameters involved in the process simultaneously.

Proportional control, integral control and derivative control, based on the real PID mathematical formula, by changing the values, at any time, of the three control constants (proportional, integral and derivative constants).

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 EBDC/CCSOF. PID Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Analog and digital PID control. PID menu and set point selection required in the whole work range.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*	References 1 to 6 are the main items: EBDC + EBDC/CIB + DAB + EBDC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Study and performance of tests to determine the appropriate amount of catalyst to be used in the biodiesel generation process (titration).

2.- Familiarization with the operation of a biodiesel production unit during the products reaction, extraction and washing process.

3.- Study of the agitation and mixing level influence on the final quality of the obtained biodiesel.

4.- Study of the washing and purifying stage influence on the final quality of the obtained biodiesel.

5.- Study of the influence of temperature during the vegetable oils preheating and reaction stage on the final quality of the obtained biodiesel.

6.- Analysis and tests to determine the biodiesel quality.

Additional practical possibilities:

7.- Study of vegetable oils transesterification with an alcohol to produce biodiesel.

8.- Sensors calibration.

Other possibilities to be done with this Unit:

9.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

10.- Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

11.- The Computer Control System with SCADA and PID Control allow a real industrial simulation.

12.- This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

13.- This unit can be used for doing applied research.

14.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

15.- Control of the EBDC unit process through the control interface box without the computer.

16.- Visualization of all the sensors values used in the EBDC unit process.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Technical and Vocational Education configuration

7 EBDC/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

b) Multipost Expansions options

8 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA and PID Control integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA and PID Control, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.