TENDER SPECIFICATIONS

The EDAS/VIS-WF System includes hardware(DAIB, Data Acquisition Interface-Box) and software (EDAS/VIS-SOF, Data Acquisition and Virtual Instrumentation Software).

1 DAIB. Data Acquisition Interface-Box:

Metal box. Dimensions: 310 x 220 x 145 mm. approx.

Front panel:

6 analog inputs.

4 analog outputs.

24 digital inputs/outputs, configurable as inputs or outputs, with LED state indicators.

4 digital signal switches 0-5V.

2 analog signal potentiometers ±12V.

Main ON/OFF switch.

Inside: internal power supply of 12 and 5 V. Potentiometer.

Back panel: power supply connector. USB connector (to connect with the PC).Antenna for the WI-FI 	communication.

Connection cables.

2 EDAS/VIS-SOF. Data Acquisition and Virtual Instrumentation Software:

Compatible con los sistemas operativos Windows actuales. Entorno gráfico amigable.

Compatible with current Windows operating systems. User-friendly graphical environment.

Configurable software allowing the time/frequency representation of the inputs and outputs. 	Visualization of the circuits voltage on the computer screen.

It allows data storage in a file. Screens and reports of the signals can be printed at any time.

Measurement, analysis, visualization, representation and report of results.

Set of Virtual Instruments:

-	Oscilloscope:

Maximum input voltage: ±10V.

The channels can be scaled to compare signals with different voltage levels.

“Math Menu” with Addition, Subtraction, Multiplication and Division operations.

- 	Function Generator:

Signal generators: sine, triangular, sawtooth and square.

Maximum output voltage: ±10V.

It includes a chart where the output signal for each channel is shown.

- 	Spectrum Analyzer:

Maximum voltage: ±10V. Digital spectrum analyzer: based on the FFT.

- 	Multimeter:

Voltmeter. Maximum voltage: ±10V RMS.

Ammeter. Maximum amperes: 500 mA RMS per channel.

Manuals:	This system is supplied with the following manuals: Required Services, Assembly and Installation, Interface and Software, Starting-up, Safety, Maintenance & Practical Exercises Manuals.

This system can be used with:

- All “M” modules (M1 to M99) of Basic Electronics (LIEBA).

- All “ED-CAM, ED-CFM and EDICOM” type modules of Communications (LICOMBA).

- SAIT. Transducers and Instrumentation Trainer.

* Any other electronic board.

Dimensions: 310 x 220 x 145 mm. approx. (12,20 x 8,66 x 5,70 inches approx.).  Weight: 2 Kg. approx. (4 pounds approx.).