Tender Specifications

Bench-top unit.

PEM electrolyzer with a hydrogen generation of 3  l/h:

Stack type: Proton exchange membrane electrolysis cell.

Water input: De-ionized  or distilled water.

Water temperature: 10 – 40 °C.

Water consumption approx.: 20 ml/hr.

Hydrogen generation capacity: 0 – 3  l/hr.

Purity: 99,99 %. For information on how to achieve higher purity, please contact us (*).

Direct connection for the Computer Controlled PEM Fuel Cell Unit, “EC5C”, or the PEM Fuel Cell Unit, “EC5B”.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manual.

Dimensions: 500 x 300 x 450 mm approx. (19.68 x 11.81 x 17.71 inches approx.)  Weight: 10 Kg approx. (22.04 pounds approx.)