TENDER SPECIFICATIONS

Bench-top unit.

PEM electrolyzer with a hydrogen generation of 60 l/h:

Water input: De-ionized water (must meet ASTM type II and conductivity of less than 0.1 uS).

Maximum outlet pressure: 10 – 16 bar.

Operating temperature: 15 – 40 °C.

Purity: 99,9999% (Grade 6.0). For information on how to achieve higher purity, please contact us (*).

Direct connection for the Computer Controlled PEM Fuel Cell Advanced Unit, “EC6C”, or the PEM Fuel Cell Advanced Unit, “EC6B”.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manual.

Dimensions: 500 x 300 x 400 mm approx. (19.68 x 11.81 x 15.74 inches approx.). Weight: 30 Kg approx. (66 pounds approx.)