TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The “EEF” unit mainly consists of:

Electric motor of 0.37 kW, maximum speed: 3000 rpm.

Motor speed control by a frequency regulator.

Loading device with spring balance (0 – 30 kg) and adjustment using threaded spindle with hand wheel between 0 and 300 N at the cantilever end of the test pieces.

Test pieces with a diameter of 8 mm and a length of 380 mm:

Two AISI 304L Stainless steel cylindrical test pieces.

Six AISI 304L Stainless steel cylindrical test pieces, with different notches.

Two F-1 Carbon steel cylindrical test pieces.

Six F-1 Carbon steel cylindrical test pieces, with different notches.

Two F-212 Steel cylindrical test pieces.

Six F-212 Steel cylindrical test pieces, with different notches.

The unit includes all the safety measures required for a safe operation:

Metallic protection cages, a fixed one for the motor axis outlet and a removable one for the test piece zone; without it the unit does not work, since it has a safety system to avoid accidents.

Automatic shut down in case of test piece breakage by stop switch (control switch).

Control box, including:

ON/OFF Main switch of the unit.

ON/OFF Switch of the electric motor.

Cycle counter, with digital display.

Motor speed controller.

Motor speed digital display.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Additional recommended elements (Not included):

- EEF-SP. Set of Spare Parts of Standard Specimens.

Test pieces with a diameter of 8 mm and a length of 380 mm:

Two AISI 304L Stainless steel cylindrical test pieces.

Six AISI 304L Stainless steel cylindrical test pieces, with different notches.

Two F-1 Carbon steel cylindrical test pieces.

Six F-1 Carbon steel cylindrical test pieces, with different notches.

Two F-212 Steel cylindrical test pieces.

Six F-212 Steel cylindrical test pieces, with different notches.

- EEF-SSP. Set of Small Specimens.  (*) Check price and delivery time (*).

Twelve cylindrical test pieces with a diameter of 8 mm in the test area (length: 106 mm), 12 mm in the clamping area (length: 41 mm) and 147 mm in total length. They have three different types of roundness:

Three steel test pieces with the shaft shoolder between the test area and the clamping area of radius of 0.5 mm and surface quality of Rz 4.

Three test pieces made of steel with the shaft shoolder between the test area and the clamping area of radius of 1 mm and surface quality of Rz 4.

Three test pieces made of steel with the shaft shoolder between the test area and the clamping area of radius 2 mm and surface quality of Rz 4.

Three test pieces made of steel with the shaft shoolder between the test area and the clamping area of radius 2 mm and surface quality of Rz 25.

(*) Check price and delivery time (*).

- EEF-SSC. Software, Sensors and Control for EEF Unit.

The Software, Sensors and Control for EEF Unit, “EEF-SSC”, is a set that allows the acquisition of the measurements of fatigue tests to rotary bending with cantilevered piece in a computer.

For the acquisition a load cell is included, coupled to the load system, which will measure the force applied to the piece (0 – 50 kg). The speed measurement will also be monitored by the computer.

It can be used on both a laptop and a bench-top unit.

The system is mainly formed:

- Control Interface Box with process diagram on the front panel. It includes:

Data Acquisition Board. USB connection.

External signal amplifier/adaptor (load cell and tachometer).

- Control Software + Data Acquisition + Data Management; allows:

Recording, manipulation, comparison and storage of data.

Acquisition and storage of bending force values and specimen speed from the computer.

Representation of Wöhler diagrams.

Optional electric motor control.

- Dimensions: 1000 x 600 x 600 mm approx. (39.37 x 23.62 x 23.62 inches approx.) - Weight: 52 kg approx. (114 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the basic principles of fatigue strength testing.

2.- Influence of the material on fatigue strength.

3.- Influence of the notch effect on fatigue strength.

4.- Influence of the length of the notch on fatigue strength.

5.- Representation of Wöhler fatigue strength S-N curves.

6.- Physical analysis of the regions formed in the rupture of pieces in the fatigue test.

7.- Influence of different rounding radii and qualities of surface on fatigue strength (requires the additional recommended element "EEF-SSP").

Optional

EEF/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.