TENDER SPECIFICATIONS (for main items)

1 EESFC. Unit:

The “EESFC” unit is a computer controlled unit for the study of the conversion of solar energy into electric energy.

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The unit includes:

Two photovoltaic solar panels (polycrystalline):

Tempered glass modules with high level of transmissivity.

Encapsulating material: modified ethylene-vinyl acetate.

Output nominal power: 66 W.

Area of the panel: 0.51 m².

Maximum current: 3.76 A. Maximum voltage: 17.53 V.

Thirty six cells, 156 x 156 mm each.

Solar simulator composed of:

Aluminum frame.

Eight halogen lamps of 400 W each one, distributed into two independent voltage regulated circuits.

Electrically safe.

Ventilation system, computer controlled, that allows to analyze the temperature influence on the unit operation, formed by:

Four axial compact fans with plastic guards.

DC load and battery charger regulator:

It regulates how power generated in the photovoltaic solar panels is distributed to and from the auxiliary battery and to the load. A display informs about the state of the charge, operating parameters and fault messages. The functions of the electronic protection are:

Overvoltage disconnection, short circuit protection of load and module, overvoltage protection at module input, over-temperature and overload protection, and battery overvoltage shutdown.

Auxiliary battery charger:

It carefully assesses the battery and then delivers the optimum charge required.

Battery: Nominal voltage: 12 V. Nominal capacity (20 hours rate): 24 Ah.

DC loads module:

Metallic box with diagram on the front panel.

Two 24 V lamps.

DC motor: Voltage: 36 V, power: 5 W.

Rheostat of 500 W.

Independent connection for every load with the help of the four positions selector:

Solar panels work at open-circuit.

Rheostat and lamps are connected directly to the solar panels. These loads can be connected individually or in parallel with the aid of manual switches.

The DC motor is directly connected to the solar panels.

The DC load is disconnected and the solar panels are directly connected to the charge regulator.

Sensors:

Solar radiation sensor to study the behaviour of solar photovoltaic panels.

Three “J” type temperature sensors to measure the environmental temperature, the temperature in the solar panel nº 1 and the temperature in the solar panel nº 2.

DC current sensor and DC voltage sensor. The value of DC power can be visualized with the software.

The connection of solar panels in parallel or series, the measurement of the voltage and the current before or after the regulator and the regulation of the light intensity of lamps of the two independent circuits are computer controlled.

The unit includes four blinds to reduce a direct visual contact with the halogen lamps and to reduce the direct contact with the photovoltaic solar panels when the unit is working.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Additional recommended elements (at least one) (Not included):

- EE-KIT. Kit of Conversion and Consumption Simulation (AC):

• Single-phase inverter:

Single-phase.

25 kHz switch mode technology.

Start-up power of 200 %.

Short-circuit protection.

High temperature protection.

Overcharge protection.

Operation state indicating LED.

Rear connection/disconnection switch.

• AC Loads Module:

Metallic box.

Diagram in the front panel.

Axial compact fan of 230 V with plastic guards.

Three lamps of 220 V – 240 V, power: 11 W.

Independent connection for every load with the help of the four positions selector:

- Inverter operation with no load.

- Fan motor connected.

- One AC lamp connected.

- Two AC lamps connected in parallel.

• AC voltage and current sensor. The value of AC power can be visualized with the software.

- EE-HYB-KIT. Hybrid Grid Inverter Kit with Self-Energy Management:

This inverter can take electrical energy either from the laboratory power grid (50/60 Hz) or from a photovoltaic panel (not included) to supply power to a local load (included) or to store such energy in a battery (included). This criteria will depend on the amount of stored energy in the battery at any given time and whether or not sufficient solar energy is available to supply the energy demand at that time.

In order to study the different modes of operation of the hybrid inverter, this kit consists of a battery, a variable resistive load and several analog DC and AC current and voltage meters.

Depending on the energy stored in the battery and the energy demanded by the variable load, it will be possible to observe the inverter behavior by means of the analog meters located at different points of the electrical circuit. For example, it will be possible to measure the current flows in and out of the battery, the current consumed by the resistive load and also, the current flows coming from the grid in case there is not enough energy in the battery to supply the demand of the resistive load.

Supply voltage: 230 VAC.

Frequency: 50/60 Hz.

Nominal power: 1000 W.

Power switch.

Bidirectional DC ammeter: -50 A – 0 – 50 A.

DC voltmeter: 0 – 15 VDC.

AC ammeter for load consumption measurement: 0 – 5 A.

AC voltmeter for measuring the voltage at the load: 0 – 250 VAC.

AC ammeter for measuring input current of the network: 0 – 5 A.

AC voltmeter for mains voltage measurement: 0 – 250 VAC.

Mains disconnecting switch.

Load disconnecting switch.

Battery disconnecting switch.

Battery circuit breaker: 40 A.

Differential circuit breaker: 230 VAC, 10 A, 30 mA.

Fuses: 2 x 16 A.

Variable resistor: 0 – 1000 Ohm, 1000 W.

Hybrid inverter:

Battery voltage: 12 VDC.

MPPT regulator: 17 – 80 VDC.

Gel battery:

Nominal voltage: 12 VDC.

Capacity: 60 Ah.

2 EESFC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 EESFC/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*	References 1 to 6 are the main items: EESFC + EESFC/CIB + DAB + EESFC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.-Identification and familiarization with all components of the unit and how they are associated with its operation.

2.-Determination of the solar panel characteristic parameters.

3.-Study of the materials that make up the solar cell.

4.-Study of the p and n sides of a solar cell.

5.-Study of the I-V and P-V curves.

6.-Study of the inverse current or the saturation current.

7.-Study of V, I and W according to different loads.

8.-Measurement of the open-circuit voltage and the short-circuit current for a solar panel with load.

9.-Measurement of the maximum power for a solar panel with load.

10.-Study of the relationship between power generated and solar radiation power.

11.-Study of the solar panel maximum power.

12.-Study of the influence of temperature on the solar panel opencircuit voltage.

13.-Determination of the photo-conversion efficiency.

14.-Study of the efficiency of the solar panels connected in parallel.

15.-Study of the efficiency of the solar panels connected in series.

16.-Study of the efficiency, depending on the temperature, of the photovoltaic system connected in parallel.

17.-Study of the operation of the photovoltaic generation system supplying power to different DC loads without an auxiliary battery.

18.-Study of the photovoltaic power generation system operation with an auxiliary battery and supplying different DC/AC loads.

19.-Study of the operation of the photovoltaic system in series/parallel with connection of different loads and without the support of the storage battery.

20.-Study of the operation of the photovoltaic system in series/parallel with connection of different loads DC and with the support of the storage battery.

Additional practical possibilities:

21.-Sensors calibration.

22.-Lamps illumination profile study.

23.-Determination of the resistance of a solar cell connected in series and in parallel.

24.-Study of the parameters that define the quality of a solar cell.

25.-Study of the dependence of the voltage of open circuit (V∞) on the lumens.

Practices to be done with the additional recommended element “EE-KIT”:

26.-Study of the operation of the photovoltaic system in series/parallel with connection of different loads and without the support of the storage battery.

27.-Study of the operation of the photovoltaic system in series/parallel with connection of different AC loads and with the support of the storage battery.

28.-Study of the connection of loads to an alternating voltage of 220 V.

Practices to be done with the additional recommended element “EE-HYB-KIT”:

29.-Study of the hybrid inverter’s grid connection procedure: correct sequence of battery and grid switches.

30.-Study of the hybrid inverter configuration.

31.-Study of the hybrid inverter in grid connection mode.

32.-Study of the hybrid inverter in island mode.

33.-Study of the behavior of the hybrid inverter in the event of a blackout.

34.-Study of the charging process of the battery from the laboratory grid through the hybrid inverter.

35.-Study of the battery charging process from a renewable energy source.

36.-Study of the power flows of the battery and the grid under variations of the energy demand with the variable resistive load.

37.-Study of the response of the hybrid inverter when the critical discharge point of the battery is reached.

38.-Study of the energy balance between the battery-charge-grid by means of the analog ammeters and voltmeters incorporated in the kit.

Other possibilities to be done with this Unit:

39.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

40.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

41.-The Computer Control System with SCADA allows a real industrial simulation.

42.-This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

43.-This unit can be used for doing applied research.

44.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

45.-Control of the EESFC unit process through the control interface box without the computer.

46.-Visualization of all the sensors values used in the EESFC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Industrial configuration

7 PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs (X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs.

Touch screen.

Panasonic PLC:

High-speed scan of 0.32 µsec. Program capacity of 32 Ksteps. High-speed counter. Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-EESFC/PLC-SOF. PLC Control Software:

For this particular unit, always included with PLC supply.

Practices to be done with PLC-PI:

1.-  Control of the particular unit process through the control interface box without the computer.

2.-  Visualization of all the sensors values used in the particular unit process.

3.-  Calibration of all sensors included in the particular unit process.

4.-  Hand on of all the actuators involved in the particular unit process.

5.-  Realization of different experiments, in automatic way, without having in front the particular unit. (These experiments can be decided previously).

6.- Simulation of outside actions, in the cases do not exist hardware elements. (Example: test of complementary tanks, complementary industrialenvironment to the process to be studied, etc).

7.-  PLC hardware general use.

8.-  PLC process application for the particular unit.

9.-  PLC structure.

10.-PLC inputs and outputs configuration.

11.-PLC configuration possibilities.

12.-PLC program languages.

13.-PLC different programming standard languages (ladder diagram (LD), structured text (ST), instructions list (IL), sequential function chart (SFC), function block diagram (FBD)).

14.-New configuration and development of new process.

15.-Hand on an established process.

16.-To visualize and see the results and to make comparisons with the particular unit process.

17.-Possibility of creating new process in relation with the particular unit.

18.-PLC Programming Exercises.

19.-Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

8 EESFC/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

9 EESFC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Multipost Expansions options

10 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.