TENDER SPECIFICATIONS (for main items)

1 EOMC-UB. Base Service Unit and Waves Generator:

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The “EOMC-UB” unit mainly consists of:

CF80/5. Flow Channel (section: 80 x 300 mm), length: 5 m.

Channel of rectangular section with transparent walls. It is assembled on supports, with a system to control the inclination of the channel. Channel slope: adjustable.

Section: 80 x 300 mm.

Length: 5 m.

Inlet tank (capacity: 38  l), with flow stilling and drain valve.

Reception tank (capacity: 38  l), with drain valve.

Elevation angle: each cm in the scale corresponds to 0.18°, the maximum elevation angle is 3°.

Flow control valve.

Pipes.

Storage tank:

Capacity: 140  l, approx.

Impulsion pump, computer controlled:

Power: 0.37 kW. Max. flow: 80  l/min. Max. pressure: 20.1 m.

ON/OFF safety switch.

Manometric panel:

Formed by two PMMA pipes of 1000 mm of length mounted on an aluminum frame.

Orifice plate:

Material: PVC. Inner diameter: 80 mm.

Venturi tube:

Material: PMMA. Length: 180 mm.

Larger section: 32 mm. Smaller section: 20 mm.

CFGO-K1. Mechanical Wave Generation Elements Kit. To produce different types of waves. The waves frequency is modified by changing the blade rotating speed from the computer.

Two reservoirs to collect and measure the impelled water flow by the Modules (EOMC-1, EOMC-2, EOMC-3 and EOMC-5).

Flow sensors:

Sensor for the Base Service Unit and Waves Generator, “EOMC-UB”, to measure the flow at the pump outlet, range: 2 – 150  l/min.

Two sensors for the “Energy Generation Modules” to measure the impelled water flow by the modules. These sensors calculate the flow from the water height in the reservoirs. These water heights are measure by two pressure sensors, range: 0 – 300 mm w.c.

Differential pressure sensor for the Base Services Unit and Waves Generator, “EOMC-UB”, to measure the pressure drop in the orifice plate and the Venturi tube, range: 0 – 1 psi.

Waves sensor for the Base Services Unit and Waves Generator, “EOMC-UB”. This sensor allows to measure the frequency and the amplitude of the waves generated by the waves generator.

Speed sensor to measure the blade rotating speed of the waves generator.

Required elements (at least one) (Not included):

- EOMC-1. Floating Buoy Module.

Floating Buoy system consists of a floating structure anchored to the seabed, which is used as the support of a cylinder which freely floats with an upwards and downwards vertical movement, following the waves. This cylinder slides by a central shaft joint to the immobile structure. The relative movement of the cylinder sliding through the shaft and the fixed structured serves to activate an energy converter which can be hydraulic or electromagnetic.

The “EOMC-1” module is designed to operate with a double acting water pump that provides a variable load and measurable output.

There are several interchangeable floats: round, triangular and rectangular bottom floats, and another rectangular float of larger size. The floats are supplied with a support to which weights can be added.

This module includes:

Double acting water pump. This pump is fixed to a stainless steel frame. Pump material: PVC.

Four interchangeable floats: a round bottom float, a triangular float, a rectangular float and other rectangular float but larger size. Floats are made of medium density foam, and each float is provided with support to which weights can be added.

Set of weights: three 100 g weight and a 50 g weight.

The double acting water pump extracts a variable amount of water due to the up thrust of the wave and the down force of gravity. The outlet pipes are routed to two reservoirs mounted on a support. The amount of water collected over a number of wave cycles can be measured. They include two flow sensors to measure the impelled water flow by the module.

The module is prepared to be installed in the Base Service Unit and Waves Generator, “EOMC-UB”.

- EOMC-2. Oscillating Water Column Module.

An Oscillating Water Column system is a structure with an opening for the wave.

When the water enters in the enclosure, it displaces the existing air into the enclosure and forces the air to exit by a duct where a turbine is installed. The air makes the turbine coupled to a generator to rotate. When the wave retires, the air enters in the enclosure through the same duct, at this moment in the reverse direction.

The “EOMC-2” module allows experiments to be carried out using such a wave absorber. This module uses the oscillating air to drive a pneumatic piston connected to a special pump that provides an easily variable load and measurable output.

This module consists of three sections: a chamber to create the moving column of air, a pneumatic piston and a water pump. The chamber is mounted with its open-end submerged below the water level, in the path of the waves and creates a column of moving air. The upper edge of the chamber is attached to a pipe ducting the air to a cylinder, in which a lightweight piston is able to move. The energy in the wave is transferred to the air, which is in turn transferred to the piston. The piston is connected via a straight connecting rod to the double acting water pump.

This module includes:

Double acting water pump. This pump is fixed to a stainless steel frame. Pump material: PVC.

Air chamber.

Pneumatic piston, diameter: 54.4 mm.

The double acting water pump extracts a variable amount of water due to the up thrust of the wave and the down force of gravity. The outlet pipes are routed to two reservoirs mounted on a support. The amount of water collected over a number of wave cycles can be measured. They include two flow sensors to measure the impelled water flow by the module.

The module is prepared to be installed in the Base Service Unit and Waves Generator, “EOMC-UB”.

- EOMC-3. Pelamis Module.

The “Pelamis” is an attenuating type device. It consists of a longitudinal chain of cylinders linked each one with the others and anchored to the seabed, so that they are correspondingly aligned to the waves direction. A cylinders nodding is produced because of the wave height and this movement is used by the pistons connected between each cylinder to impulse the fluid in an hydraulic circuit to produce electric energy.

The “EOMC-3” module consists of a longitudinal chain of two cylinders linked each other. A double acting water pump is connected between each cylinder.

This module includes:

Double acting water pump. This pump is fixed to a stainless steel frame. Pump material: PVC.

Longitudinal chain of two cylinders linked each other. This device allows to simulate a “Pelamis” device.

Set of weights: three 100 g weight and a 50 g weight.

The double acting water pump extracts a variable amount of water due to the up thrust of the wave and the down force of gravity. The outlet pipes are routed to two reservoirs mounted on a support. The amount of water collected over a number of wave cycles can be measured. They include two flow sensors to measure the impelled water flow by the module.

The module is prepared to be installed in the Base Service Unit and Waves Generator, “EOMC-UB”.

- EOMC-4. Tapered Channel Module.

The Tapered Channel is a channel built on the coast in the direction of the incident wave. The channel progressively narrows to the inside of the coast. When the wave enters the channel, it goes to a every time less section, which makes to rise its height to enter in a tank placed at the end of the channel.

Thus, there is water at a higher height that can be used in a turbine. Summarizing, it concentrates the waves and guides them up a ramp into a tank, from which a turbine extracts energy.

The “EOMC-4” module allows experiments to be carried out on a variety of channel configurations.

This module studies the energy in breaking waves, and allows to alter the way the waves break.

The module includes three ramps with different slop and a reservoir to contain and measure the water.

The ramps have two walls, which can be adjusted to provide a contraction on the wave as it moves up the ramp.

This module include:

Three ramps with different slop and adjustable walls.

Reservoir to contain and measure the water.

The module is prepared to be installed in the Base Service Unit and Waves Generator, “EOMC-UB”.

- EOMC-5. Salter’s Duck Module.

Salter’s Duck is a small buoy anchored to the seabed in a particular way which seems a duck moves in a swinging way when receiving the waves. This rotation is used to move, by means of mechanisms, a system connected to a generator.

The "EOMC-5" module is designed to provide experiments to test the performance of basic flexible devices. A flexible device forms a structure from which a float can pivot.

The module captures the wave’s energy by using a floating device. The device is extended beyond the pivot point by a lever used to drive the double acting water pump.

The float is supplied with a support to which weights can be added.

This module include:

Double acting water pump. This pump is fixed to a stainless steel frame. Pump material: PVC.

Flexible float.

Set of weights: three 100 g weight and a 50 g weight.

The double acting water pump extracts a variable amount of water due to the up thrust of the wave and the down force of gravity. The outlet pipes are routed to two reservoirs mounted on a support. The amount of water collected over a number of wave cycles can be measured. They include two flow sensors to measure the impelled water flow by the module.

The module is prepared to be installed in the Base Service Unit and Waves Generator, “EOMC-UB”.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

2 EOMC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 EOMC/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required services, Assembly and Installation, Interface and Control software, Starting-up, Safety, Maintenance, Calibration & Practices manuals.

*References 1 to 6 are the main items: EOMC + EOMC/CIB + DAB + EOMC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

Practices to be done with the Base Service Unit and Waves Generator, "EOMC-UB":

1.-  Study of the characteristic parameters of waves.

2.-  Measurement of flow using Venturi tube.

3.-  Measurement of flow using orifice plate.

Practices to be done with the Floating Buoy Module, “EOMC-1”:

4.-  Determination of the extracted energy with a floating buoy module.

5.-  Study of the influence of the float shape in the extracted energy with a floating buoy module.

Practices to be done with the Oscillating Water Column Module, “EOMC-2”:

6.-  Determination of the extracted energy with an oscillating water column module.

Practices to be done with the Pelamis Module, “EOMC-3”:

7.-  Determination of the extracted energy with a Pelamis module.

Practices to be done with the Tapered Channel Module, “EOMC-4”:

8.-  Determination of the extracted energy with a Tapered Channel module.

9.-  Study of the effect of slope on the extracted energy with a Tapered Channel module.

10.-Study of the effect of width contraction on the extracted energy with a Tapered Channel module.

Practices to be done with the Salter’s Duck Module, “EOMC-5”:

11.-Determination of the extracted energy with a Salter's Duck module.

Additional practical possibilities:

12.-Study of the different wave's energy generation systems.

13.-Study of the operation and performance of the different wave's energy generation systems.

14.-Study of the energy available from a wave.

15.-Study of the Archimedes and buoyancy principles.

16.-Study of the relationship between wave form and buoyancy.

17.-Determination of the potential energy in a fluid.

18.-Study of the float shape in determining energy extraction.

19.-Study of the compression of air.

20.-Study of the pneumatic/hydraulic pistons.

21.-Study of the effect of wave shape on its breaking characteristics.

Other possibilities to be done with this unit:

22.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

23.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

24.-The Computer Control System with SCADA allows a real industrial simulation.

25.-This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

26.-This unit can be used for doing applied research.

27.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

28.-Control of the EOMC unit process through the control interface box without the computer.

29.-Visualization of all the sensors values used in the EOMC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

TENDER SPECIFICATIONS (for optional items)

a) Industrial configuration

7 PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs (X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs.

Touch screen.

Panasonic PLC:

High-speed scan of 0.32 µsec. Program capacity of 32 Ksteps. High-speed counter. Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-EOMC/PLC-SOF. PLC Control Software:

For this particular unit, always included with PLC supply.

Practices to be done with PLC-PI:

1.-  Control of the particular unit process through the control interface box without the computer.

2.-  Visualization of all the sensors values used in the particular unit process.

3.-  Calibration of all sensors included in the particular unit process.

4.-  Hand on of all the actuators involved in the particular unit process.

5.-  Realization of different experiments, in automatic way, without having in front the particular unit. (These experiments can be decided previously).

6.- Simulation of outside actions, in the cases do not exist hardware elements. (Example: test of complementary tanks, complementary industrialenvironment to the process to be studied, etc).

7.-  PLC hardware general use.

8.-  PLC process application for the particular unit.

9.-  PLC structure.

10.-PLC inputs and outputs configuration.

11.-PLC configuration possibilities.

12.-PLC program languages.

13.-PLC different programming standard languages (ladder diagram (LD), structured text (ST), instructions list (IL), sequential function chart (SFC), function block diagram (FBD)).

14.-New configuration and development of new process.

15.-Hand on an established process.

16.-To visualize and see the results and to make comparisons with the particular unit process.

17.-Possibility of creating new process in relation with the particular unit.

18.-PLC Programming Exercises.

19.-Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

8 EOMC/ICAI. Interactive Computer Aided Instruction Software.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

9 EOMC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The “FAULTS” mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Multipost Expansions options

10 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.