TENDER SPECIFICATIONS

Steel frame and painted steel panels.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Stainless steel test tank:

Area: 2 x 1 m².

Depth: 0.2 m.

Sand capacity: 0.3 m³.

Drainage and filter system to prevent loss of test material.

Twenty sample shots.

Tilt adjustment system: -2.5° – 5°.

Overflow drainage system.

Closed water circuit:

Adjustable speed pump:

Power: 0.55 kW.

Max. pressure: 2 bar.

Max. flow rate: 2000 l/h.

Two storage water tanks:

Capacity: 90 l per tank.

Two flow meters:

Flow rate measurement, flowmeter: 0 – 1700 l/h.

Drainage flow measurement, measuring weir: 0 – 1700 l/h.

Flooding devices:

Rain simulator with eight diffuser nozzles with individual valves.

Inlet tank for river simulation.

Two french wells for groundwater simulation.

Pressure gauge panel:

Twenty manometer tubes for measuring the water table, range: 0 – 300 mmH2O.

Overflows for water outlet in the test tank, with adjustable height.

Three manual regulation valves for water inlet.

Cables and accessories, for normal operation.

Manuals: This unit is supplied whit the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals

- Dimensions: 1500 x 2700 x 2500 mm approx. (59,05 x 106,29 x 98,42 inches approx.). - Weight: 950 Kg approx. (2094 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of surface dragging.

2.- Hydrogram curve, strong storm.

3.- Calculation of concentration time for a short storm.

4.- Determination of the compactness index.

5.- Drainage density determination.

6.- Reservoir filling and flooding.

7.- Gravity force water. Field capacity.

8.- Sediment transport in river models.

9.- Extracting water from adjoining wells.

Optional

ESH(2x1m)/S/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.