TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

The “HEMP” unit mainly consists of:

Bourdon manometer for measuring negative pressure (vacuum gauge), range: -1 – 0 bar.

Bourdon manometer for measuring positive pressure, range: 0 – 2.5 bar.

Vertical “U” tube manometer, with scale in mm.

Inclined “U” tube manometer, with scale in mm.

Manual pump (syringe) for pressurising and reducing the pressure in the measurement devices.

Ball valves in each manometric tube.

Three quick “Y” connections to connect two instruments to the same point.

The measuring elements can be exchanged.

Included elements:

- FME10. Dead Weight Calibrator.

Dead-weight calibrator consists of a piston, with is free to move vertically, in cylinder. Flexible hose connects the cylinder with the Bourdon pressure manometer. A set of weights are included.

Bourdon type manometer with internal mechanism clearly visible through the transparent dial, range: 0 – 2.5 bar.

Cylinder and piston:

Diameter of the piston: 18 mm.

Area of the piston: 0.000254469 m².

Weight of the piston: 0.5 Kg.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Unit: - Dimensions: 700 x 400 x 800 mm approx. (27.55 x 15.74 x 31.49 inches approx.)  - Weight: 20 Kg approx. (44 pounds approx.)

FME.10. Dead weight Calibrator: - Dimensions: 500 x 350 x 350 mm approx. (19.68 x 13.77 x 13.77 inches approx.)  - Weight: 10 Kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Familiarisation with different pressure measurement methods.

2.- Study of the function and characteristics of a Bourdon type manometer.

3.- Study of the function and characteristics of an “U” tube manometers.

4.- Pressure measurements with Bourdon type manometers. Indirect measurement.

5.- Pressure measurements with vertical or inclined “U”-tube manometers. Direct measurement.

6.- Comparison between the different methods of pressure measurement.

7.- Calibration of a pressure gauge.

8.- Determination of measurement errors.

9.- Checking the readings of a Bourdon type manometer using a calibrated standard set of masses.

10.- Determination of the hysteresis curve of a manometer.

Optional

HEMP/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.