TENDER SPECIFICATIONS

HMM-W500. U-Shape Double Manometer:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

This multimanometer has been designed for operating with Pitot’ s tube.

It allows finding the pressure between two points or two fluids.

Two U-shape glass manometers, length: 500 mm.

Millimeter precision rules, length: 500 mm.

Three points for pressure measurement.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 250 x 500 x 870 mm approx. (9.84 x 19.68 x 34.25 inches approx.). - Weight: 3 Kg approx. (6.61 pounds approx.).

HMM-U1000. U-Shape Manometer:

Designed for wall assembly.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

U-shape manometer, length: 1000 mm.

Millimeter precision rules, length: 1000 mm.

Upper collector.

Lower collector.

Drain valve.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 170 x 40 x 1400 mm approx. (6.69 x 1.57 x 55.11 inches approx.). - Weight: 2 Kg approx. (4 pounds approx.).

HMM-I1000. Inclined Multimanometer with 20 manometric tubes of 250 mm length:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Inclination: 30° approx.

Twenty manometric tubes, length: 250 mm.

Tubes inner diameter: 8 mm, to avoid bubbles.

Water tank for filling.

Twenty points for differential pressure measurement, with key.

Common collector.

Drain valve.

Millimeter precision rules, length: 250 mm.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 1400 x 1400 x 700 mm approx. (55.11 x 55.11 x 27.55 inches approx.). - Weight: 10 Kg approx. (22 pounds approx.).

HMM-V500. Multimanometer with 8 manometric tubes of 500 mm length, vertical position:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Vertical position.

Eight manometric tubes, length: 500 mm.

Tubes inner diameter: 8 mm, to avoid bubbles.

Air pump for pressurization.

Eight points for differential pressure measurement, with key.

Common collector.

Non-return valve.

Drain valve.

Millimeter precision rules, length: 500 mm.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 300 x 500 x 870 mm approx. (11.81 x 19.68 x 34.25 inches approx.). - Weight: 4 Kg approx. (8 pounds approx.).

HMM-V500-12. Multimanometer with 12 manometric tubes of 500 mm length, vertical position:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Vertical position.

Twelve Manometric tubes, length: 500 mm.

Tubes inner diameter: 8 mm, to avoid bubbles.

Air pump for pressurization.

Twelve points for differential pressure measurement, with key.

Common collector.

Non-return valve.

Drain valve.

Millimeter precision rules, length: 500 mm.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 400 x 500 x 870 mm approx. (15.74 x 19.68 x 34.25 inches approx.). - Weight: 5 Kg approx. (11 pounds approx.).

HMM-4B. 4 Bourdon type Manometers Unit:

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Vacuum-meter, range (-9800 [ mmH2O] – 0).

Vacuum-meter, range (-1000 [ mmH2O] – 0).

Manometer, range (0 – 1000 [ mmH2O] ).

Manometer, range (0 – 2.5 [ bars]).

Mobile Piston (syringe).

Eight valves.

Non-return valve.

Polyurethane tubes.

This system is supplied with atm, bar, psi, mmHg, mmH2O, conversion tables.

This system allows the calibration of six sensors (same type) simultaneously.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

- Dimensions: 720 x 300 x 570 mm approx. (28.34 x 11.81 x 22.44 inches approx.). - Weight: 15 Kg approx. (33 pounds approx.).