TENDER SPECIFICATIONS (for main items)

1 HTHS150/150C. Unit:

Hydrodynamic tunnel in closed water circuit flowing in the vertical plane.

Made of high quality material.

Steel suport frame.

Panels made of painted steel.

Main metallic elements in stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Generals dimensions:

Five meters in length between vertical tunnel parts and 1.6 m in height between horizontal parts.

Channel:

Working area of 150 x 150 mm and 3000 mm in length (consult other measures) to assemble and perform tests with the different models.

Fused silica (quartz) windows on the front, back and top face for optical tests.

Five 500 x 150 mm windows per side, which can be replaced if necessary.

Honeycomb flow straightener before the working section.

Inlet section to the working area with a shrinkage ratio of 2.3:1.

Outlet section with diffuser to avoid the generation of turbulence, which can damage the quality of the flow in the working area.

Guide vanes on the elbows to stabilize the flow.

Pumping system:

Variable speed, computer controlled axial propeller pump:  Power: 10 – 20 kW.

Instrumentation:

Temperature sensors along the work area.

Temperature sensors at the inlet and outlet of the cooling coil.

Pressure sensors at key points.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Recommended additional elements (Not included):

- HTHS1/150. Single-Bladed Propeller Model.

- HTHS2/150. Two-Bladed Propeller Model.

- HTHS3/150. Three-Bladed Propeller Model.

- HTHS4/150. Four-Bladed Propeller Model.

- HTHS5/150. Five-Bladed Propeller Model.

- HTHS6/150. Six-Bladed Propeller Model.

- HTHS150/150C-HS. Heating System.

Heating elements on the lower face with variable and independent power for each one computer controlled.

- HTHS150/150C-CS. Cooling System.

Water cooling coil before passing through the pump and the working section.

- HTHS150/150C-FCP.  Filter for the Collection of Study Particles.

- HTHS150/150C-TARC. Force Measurement Interface and Sensors.

2 HTHS150/150C /CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

The Data Acquisition board model may change at any moment, providing the same or better features than those required for the unit.

4 HTHS150/150C /CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*	References 1 to 6 are the main items: HTHS150/150C  + HTHS150/150C /CIB + DAB + HTHS150/150C /CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Extensive study of hydrodynamics and water flow studies.

2.- Study of flow visualization.

3.- Study of static pressure, dynamic pressure and total pressure using a Pitot tube.

4.- Study of the measurement of velocity using a Pitot tube.

5.- Study of turbulent and laminar flow.

Additional practical possibilities:

6.- Calibration of sensors.

Additional practical possibilities to be carried out with the recommended additional elements (Not included):

7.- Study of cavitation in different models.

8.- Thrust and torque measurement.

9.- Study of cavitation as a function of the number of propeller blades.

10.- Study of the noise production inherent to cavitation.

11.- Study of the pressure fluctuations due to cavitation.

12.- Performance losses due to cavitation.

Other possibilities to be done with this System:

13.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

14.- Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivate parameters; etc, in real time.

15.- The Computer Control System with SCADA and allow a real industrial simulation.

16- This unit is totally safe as uses mechanical, electrical and electronic, and software safety devices.

17.- This unit can be used for doing applied research.

18.- This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

19.- Control of the HTHS150/150C unit process through the control interface box without the computer.

20.- Visualization of all the sensors values used in the HTHS150/150C unit process.

- Several other exercises can be done and designed by the user.