TENDER SPECIFICATIONS (for main items)

1 LE00/MP. Pilot Plant for the Production of Milk Powder:

EPAC. Computer Controlled Rising Film Evaporator.

Evaporation double jacket column, with a heating surface of 0.122 m³, 30 mm of internal diameter, 60 mm of external diameter and 1300 mm of length.

Membrane dosing pump, computer controlled, maximum flow 15 l/h, with flow control.

Single effect vacuum pump, computer controlled, maximum flow 3 m³/h and maximum vacuum of 150 microns.

Three tanks, capacity: 10 l (for feeding, concentrated and evaporated).

Two graduated vessels for the storage of concentrated and evaporated product, capacity: 500 ml.

Computer controlled heating element, range: 300 W. Included a safety temperature sensor with internal temperature controller.

Automatic temperature control.

Coil coolant with length of 400 mm.

High safety pressure cut out for pressure control in the column.

Ten temperature sensors, type “J”.

Flow sensor, range: 0.2 - 6.5 l/min.

Three force sensors, one in each of the three tanks (for feeding, concentrated and evaporated), for measuring the amount of substance.

Two pressure sensors, range: 0 - 2 bar.

Solenoid valve, computer controlled.

Dimensions: 1000 x 800 x 2300 mm approx.

Weight: 135 kg approx.

TGV. Steam Generator (3 kW).

Stainless steel water tank (volume: 40 l). It has one water inlet, one water outlet and two vapour outlets.

Temperature PID control, set point max.: 120º C.

Temperature sensor, type “J“.

Pressure working range: 0 - 0.9 bar.

Electric heating element: 3 kW.

Polycarbonate heat-resisting protection screens.

Safety pressure switch (2 bar).

Safety level switch (the heating element will be switched off if there is not enough level).

Electronic console.

Dimensions: 680 x 430 x 750 mm approx.

Weight: 50 kg approx.

SSPC. Computer Controlled Spray Drier.

Bench mounted spray drier for processing aqueous emulsions, solutions, suspensions and colloids. This unit is suitable for aqueous solutions only.

Downward co-current operation (a fine jet of the product is brought into contact with a hot air stream).

Characteristics:

Maximum drying capacity: 1000 ml/h approx.

Temperature range: 40 - 200ºC (temperature at inlet).

Dry air volume range: 0.2 - 0.65 m³/min.

Spray air pressure range: 0.5 - 2.5 kg/cm².

Feed pump volume range: 102 - 1800 ml/h approx.

Maximum air pressure: 70 mbar.

The chemically resistant powder coated housing includes the fan and heating element (resistance).

All clamps and fittings are designed to allow assembly and removal of the glass elements rapidly and easily.

Fan, computer controlled:

Power: 0.4 kW.

Velocity: 3000 rpm.

Drying air throughflow: 70 m³/h (fixed).

Heating element (resistance) of 3 kW, computer controlled. Temperature sensor type “J”, located at the inlet of the drying chamber, works with the PID controller to maintain the desired air temperature at the inlet of the drying chamber.

Drying chamber:

Material: borosilicate glass.

It includes a spray nozzle, diameter: 0.5 mm.

The spray assembly incorporates a manual de-blocking device that prevents the jet nozzle from becoming blocked.

Feed pump: peristaltic pump, with variable speed, computer controlled.

Sample collection bottle:

Material: hard glass.

Volume: 500 ml.

Exhaust tube:

Outer diameter: 50 mm.

It includes a diaphragm with an orifice plate.

Filter/air regulator located between the compressor (not included) and the unit to ensure that the drying air does not include contaminants.

Temperature sensor type “J” to measure:

Environmental temperature.

Air inlet temperature in the drying chamber.

Exhaust air temperature.

Feeding temperature.

Differential pressure sensor to measure, together with the diaphragm with orifice plate, the flow of exhaust air, range: 0 - 1 psi (0 - 100 m³/h).

Pressure sensor at the compressed air inlet, range: 0 - 6 bar.

Glass vessel, volume: 1 l.

Dimensions: 500 x 500 x 1200 mm approx.

Weight: 80 kg approx.

The complete pilot plant includes as well:

Advanced Real-Time SCADA and PID Control.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Additional recommended elements (Not included):

WTD. Work Table with Wheels for Draining.

AISI-304 stainless steel construction.

Double tray and holes for drainage with high inclination to avoid the accumulation of any type of substance.

Mounted on four wheels, two of them with brakes.

Dimensions: 1800 x 650 x 900 mm.

CRCA. Clean Room Cabinet.

AISI 304 stainless steel sheet metal cabinet.

Low-level return louvres in anodised extruded aluminium sheeting.

Curtain made of 2 mm thick transparent PVC slats, overlapping each other, at the access.

Filters and filter holders on the low-level return line.

High efficiency filters located on the discharge (minimum 4 filtration units).

Flow rate of the filtration unit: 4800 m3/h.

High efficiency centrifugal fans.

Air diffusion in unidirectional flow by means of a diffuser sheet (veil).

LED bar.

Particle size: ≥0.5 μm.

Dimensions: 2500 x 4200 x 2200 mm.

ASCD. Air Shower Cabin for Decontamination.

Compact and self-supporting module.

AISI 304 stainless steel.

Doors with perimeter gasket to improve the tightness of the whole unit.

Sight glass with integral vision.

Status signalling and emergency push button.

Hygienic and bacteriostatic PVC floor, suitable for clean areas.

Integrated stainless steel nozzles with fixed orientation arranged in vertical columns (minimum 12 nozzles).

Large air return surface, which increases the speed and efficiency of the cycle.

Flow rate: 30 m/s.

Air filtered through H14 filters.

Cycle control by means of a small PLC, which allows programming modifications.

Dimensions: 1200 x 1000 x 2000 mm.

2 PLCHMI. IIoT Local/Remote Control and Monitoring with HMI (included):

The expansion for PLC and HMI, “PLCHMI”, is a system composed of an interface that includes PLC modules such as CPU, digital I/O module, analog I/O module, communications module, etc. and a control box with HMI display.

PLC interface:

PLC controller:

Panasonic FP7 CPS31E CPU.

Digital I/O modules:

16 digital inputs; Input range 0 V to 24 V.

16 digital outputs; Relay output.

Analogue I/O modules:

16 analog inputs; 16-bit resolution. Input range -10 V to +10 V.

4 analog outputs; 16-bit resolution. Output range -10 V to +10 V.

Connectors and Communication Ports:

2-Port Ethernet Switch.

SCSI connector.

USB, DB-9 Series or DB-25 (if required).

HMI control box and display:

HMI display:

Touch Screen: Analog Resistive.

Size: 10” 16:9 TFT.

Resolution: 1024 x 600, WVGA.

Colors: 64 K.

Ethernet port.

3 LE00/CCSOF. Supervision Software + Control Software + Data Acquisition Software + Data Management Software:

SCADA System is composed of four Software Package with the following features:

The Supervision Software is in charge of monitoring in real time start and stop elements, unexpected conditions and process evolution. In case of being necessary, it actuates on the system and notifies the user the incorrect operations.

The Control Software allows to manage multiple process and variables in real time either a manual way or automatic way. Several type of algorithms of control such PID CONTROL are implemented depending on the field of study.

The Data Acquisition Software focus on measuring and processing signals from the process with very high accuracy getting a synchronized and fast response of the system. A calibration system is part of this software to adjust the sensor measurements.

The Data Management Software stores and represents, alarms, variables and process evolution in real time both in a graphic format and in a numeric format such time charts or process diagram. Printable reports can be generated or historian data can be loaded to study the experiments in detail.

The Software is open and flexible architecture that facilities to access different work levels both instructors and students. It is supported by current Windows operating system and industrial standards. The graphical user interface is intuitive and user- friendly.

4 Cables and Accessories, for normal operation.

5 Manuals:

Each unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*	References 1 to 5 are the main items: LE00/MP: EPAC, TGV and SSPC + PLCHMI + LE00/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.- Production of evaporated milk and condensed milk.

2.- Powdered milk production.

3.- Analysis of all phases of the product transformation, from reception to packaging.

4.- Qualitative   evaluation of the ﬁnished product as function of the productive operations performed.

5.- Experimentation  on  the  efﬁciency  of  the  different chemical products used for the preservation process.

6.- Organization of procedures for continuous quality control of the production operations.

7.- Execution of procedures for disinfection and saniﬁcation of the units.

8.- Evaluation of technical, legal and administrative aspects concerning food adulteration.

9.- Manual and computer control operation.

Additional practical possibilities:

10.- Sensors calibration.

Other possibilities to be done with this pilot plant:

11.- Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

12.- Open Control, Multicontrol and Real Time Control.

Each unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters, etc. in real time.

13.- The Computer Control System with SCADA and PID Control allow a real industrial simulation.

14.- Each unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

15.- Each unit can be used for doing applied research.

16.- Each unit can be used for giving training courses to Industries even to other Technical Education Institutions.

17.- Visualization of all the sensors values used in the LE00/MP pilot plant process.

Additional practical possibilities with the expansions:

ESN. EDIBON Scada-Net Systems:

18.- Control any unit from any post located in the laboratory.

19.- Supervise different experiments about data acquisition and representation, from the units, in real time.

20.- Visualize any experiment from any laboratory post.

21.- Supervise as many experiments as desired, performed in different units at the same time.

22.- Generate reports with the results obtained with the units.

23.- Perform different experiments at the same time.

24.- Show to the laboratory members the appropriate manual or automatic operations to perform with each laboratory unit.

25.- Create more elaborate practical exercises using more than one unit from the laboratory.

26.- Suggest multidisciplinary experiments, in other words, mix in the same experiment units from different study fields.

27.- Modify any parameter of any unit included in the system from any workstation in the laboratory.

28.- Cause an abnormal functioning in a unit for the students to practice fault finding exercises.

29.- Assess the knowledge of a student or group of students about a particular unit (any unit included in the “ESN” system).

30.- Develop guided practical exercises for a better understanding of each unit.

31.- Individual training practical exercises.

32.- Group exams or practical exercises.

33.- Perform interactive exercises (using the chat between manager-users).

34.- Exchange of obtained results among the members of the “ESN” system.

35.- Any exercise directly related to the SCADA software of each unit.

36.- Some of the practical possibilities may be done only with the “ESN” complete system.

TENDER SPECIFICATIONS (for optional items)

6 ESN. EDIBON Scada-Net Systems.

The EDIBON Scada-Net Systems, "ESN", consists on the integration of EDIBON computer controlled units into the SCADA system in a local network.

The main feature of this system is the remote control of any EDIBON unit belonging to it from any control station included in the local network. In addition, any of these units can be visualized from any workstation.

Consequently, the efficiency of a laboratory with the "ESN" system is higher than the efficiency of a conventional laboratory.

- Higher laboratory performance since several students can work simultaneously. Several users can operate various units at the same time.

-Possibility of dividing the classroom into workgroups.

-Several experiments can be performed at the same time.

-Collaborative experiments performance.

-There are different user levels (manager, basic, intermediate and advanced) with different permissions.

-The manager has the absolute control of the system.

-The manager/teacher can supervise from his/her computer the operations every user is performing in any unit of the laboratory.

-Users and manager are connected at all times.

-Real time display and control of the whole system from an interactive whiteboard (touchscreen).

-CENTRALIZED AND SECURE SYSTEM, it can be totally controlled from the central computer (manager).

-The "ESN" System is MODULAR, OPEN and EXPANDIBLE.

-A vision system for real time monitoring of experiments is supplied.

-Visualization of the changes in a unit from any computer of the laboratory.

-All units can work simultaneously.

-The system is made up of as many units as required.

-The required infrastructure, both hardware and software is provided.

7 ECL. EDIBON Cloud Learning.

EDIBON Cloud Learning expansion, “ECL”, is a solution designed to control EDIBON Technology based laboratories remotely in a simple and easy way.

EDIBON Cloud Learning, “ECL”, is divided in two platforms:

Users Online Platform:

The main advantages of the Users Online Platform are:

The administrators have full control over their laboratories thanks to the powerful class-administrator tool that allows the users management, logs visualization and progression monitoring. It also enables to assign users permissions to let them control EDIBON units or just display them. Furthermore, the administrator can upload and download measurements, data and multimedia resources.

The users can learn interactively in a flexible environment as if they were in the laboratory, accessing through the Remote App to work with EDIBON units. Several users can work with one unit or one user with several units. The users can also upload and download measurements, data and graphs, multimedia resources and reports.

Remote App Platform:

Thanks to the Remote App Platform, the users can control EDIBON units and EDIBON SCADA software as if they were in the laboratory and share their expertise with the users community.