TENDER SPECIFICATIONS

The kit includes:

Manuals.

Set of practice wires.

Array four individual resistances 10 K.

Array eight resistances 10 K.

Array eight resistances 330 ohms.

IC 74HC14.

IC 74LS04.

IC 74LS08.

Two IC 74LS112.

IC 74LS279.

IC 74LS74.

IC 74LS86.

Ten switches 1 CIR, two positions.

Twelve LED red diodes.

Push button 1 CIR.

Resistance 100 ohms.

Seventeen resistances 10 K.

Twelve transistors.

Required elements (Not included):

- FACO. Power Supply.

- N-M15. Own Development Module.

Additional recommended elements (Not included):

- EDAS/VIS-0.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 250,000 samples/s).

or

- EDAS/VIS-1.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 1,250,000 samples/s).

Dimensions: 300 x 300 x 200 mm approx. (11.81 x 11.81 x 7.87 inches approx.). Weight: 500 g approx. (1 pound approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Bistables:

1.-  Bistables.

2.-  Bistable S-R using NAND gates.

3.-  Practical performance.

4.-  Faults study in the bistables.

5.-  Theoretical/practical exercises.

Shift registers:

6.-  Shift registers.

7.-  Faults study of the shift registers.

8.-  Theoretical/practical exercises.

Counters:

9.-  Steps to be followed for the assembly of a counter.

10.-Faults study of the counters.

11.-Theoretical/practical exercises.

Synchronous sequential circuits:

12.-Practice of the synchronised.

13.-Faults study of the synchronised sequential circuits.

14.-Theoretical/practical exercises.

Memories:

15.-Theoretical/practical exercises.

-Several other exercises can be done and designed by the user.

Optional

M13/KIT/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.