TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are mounted on a base plate to be mounted and connected to the Drive Unit for Tribological Tests, “MEMT-UB”.

Rotating friction disc:

Material: Hardened and ground stainless steel.

Adjustable operating speed, range: 0 – 0.5 m/s.

The disc is enclosed in an open tank that can be filled with different lubricants to vary the lubrication conditions.

Three friction pins:

Material: Steel, aluminum and brass.

Diameter: 4 mm.

Force sensor to measure the friction forces in the bearing point of the loading mechanism. This force is measured by a force sensor, range: 0 – 50 N.

Set of weights:

Material: Stainless steel.

3 x 20 N.

2 x 10 N.

2 x 5 N.

It includes a weight holder.

The pressure adjustment is performed using a lever and the set of weights.

Easy connection to the Drive Unit for Tribological Tests, “MEMT-UB”.

Cables and accessories, for normal operation.

Manuals: Required services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices manuals.

Required elements (Not included):

- MEMT-UB. Drive Unit for Tribological Tests (Common for all available experimental units type "MEMT").

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The drive unit consists of:

DC motor, electronically controlled:

Speed range: 0 – 3000 rpm.

Operating speed: 0 – 250 rpm.

Torque: 19 Nm approx.

Worm gear: i=14.66.

Electronic console:

Force sensor connection.

ON/OFF controller for the motor.

Motor speed controller.

Selector of the direction of rotation of the motor.

Digital display for the motor speed.

Digital display for the force sensor.

MEMT-UB. Drive Unit for Tribological Tests.

Base unit: - Dimensions: 550 x 500 x 280 mm approx. (21.65 x 19.68 x 11.02 inches approx.) - Weight: 12 Kg approx. (26.45 pounds approx.)

Electronic console: - Dimensions: 300 x 190 x 130 mm approx. (11.81 x 7.48 x 5.11 inches approx.)- Weight: 3 Kg approx. (6.61 pounds approx.)

MEMT-3: - Dimensions: 400 x 450 x 230 mm approx. (15.74 x 17.71 x 9.05 inches approx.) - Weight: 8 Kg approx. (17.63 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the friction forces between several vertical pins of different materials and a rotating friction disc.

2.- Investigation of the friction forces for several pairs of friction materials.

3.- Investigation of the friction forces for several loads.

4.- Investigation of the friction forces with different lubricants.

5.- Investigation of the friction forces at different speeds of the rotating disc.

6.- Study of the wear for different friction parameters.

Optional

MEMT-3/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.