TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

All elements of this unit are mounted on a base plate to be mounted and connected to the Drive Unit for Tribological Tests, “MEMT-UB”.

Rolling sphere:

Material: Polished and hardened stainless steel.

Diameter: 26 mm approx.

Driven glass disc:

Diameter: 160 mm approx.

Coating: BK-7 dielectric.

Microscope:

Magnification: 50 X.

It can be adjusted using a movable support, adjustable in x-y and has a focus adjustment.

Force sensor, range: 0 – 50 N.

Easy connection to the Drive Unit for Tribological Tests, “MEMT-UB”.

Cables and accessories, for normal operation.

Manuals: Required services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices manuals.

Required elements (Not included):

- MEMT-UB. Drive Unit for Tribological Tests (Common for all available experimental units type "MEMT").

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The drive unit consists of:

DC motor, electronically controlled:

Speed range: 0 – 3000 rpm.

Operating speed: 0 – 250 rpm.

Torque: 19 Nm approx.

Worm gear: i=14.66.

Electronic console:

Force sensor connection.

ON/OFF controller for the motor.

Motor speed controller.

Selector of the direction of rotation of the motor.

Digital display for the motor speed.

Digital display for the force sensor.

MEMT-UB. Drive Unit for Tribological Tests.

Base unit: - Dimensions: 550 x 500 x 280 mm approx. (21.65 x 19.68 x 11.02 inches approx.) - Weight: 12 Kg approx. (26.45 pounds approx.)

Electronic console: - Dimensions: 300 x 190 x 130 mm approx. (11.81 x 7.48 x 5.11 inches approx.)- Weight: 3 Kg approx. (6.61 pounds approx.)

MEMT-5: - Dimensions: 350 x 250 x 600 mm approx. (13.78 x 9.84 x 23.62 inches approx.) - Weight: 12 Kg approx. (26.45 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the elastohydrodynamic behavior of a lubricant film between a rolling sphere and a disc.

2.- Determination of the lubricant film thickness at the point of contact between a sphere and a disc.

3.- Comparison of the experimental lubricant film thickness with theoretical values.

4.- Study of the influence of load on the lubricant film thickness.

5.- Study of the influence of speed on the lubricant film thickness.

Optional

MEMT-5/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.