TENDER SPECIFICATIONS

Bench-top unit with adjustable legs.

Soldered and painted steel frame.

The "MFGE" unit mainly consists of:

Bending bar:

Material: stainless steel. Dimensions: 680 x 25 x 12 mm.

The bar is supported on ball bearings at two points, allowing to apply a purely bending stress.

It includes two strain gauges on the tension side and two strain gauges on the compression side in full bridge configuration:

Strain gauge measuring point: full bridge, 120 Ω.

Mechanical load application device. It includes a spindle, a cross-arm and a handwheel.

Dial gauge with adjustable dial for direct deflection measurement, range: 0 – 20 mm, graduations: 0.01 mm.

Electronic console:

Metallic box.

Connectors of the strain gauges.

Display of the strain measurement.

Switch to tare the strain gauges measurement.

Main switch.

Cables and Accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: Unit: 720 x 350 x 400 mm approx. (28.35 x 13.77 x 15.75 inches approx.). Weight: 25 Kg approx. (55.1 pounds approx.)

Electronic console: 300 x 190 x 130 mm approx. (11.81 x 7.48 x 5.11 inches approx.). Weight: 2.5 Kg approx. (5.51 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Fundamentals of measuring with strain gauges.

2.-  Displacement measurement with a dial gauge.

3.-  Determination of the k factor of strain gauges.

4.-  Comparison of the experimental values obtained by strain gauges with theoretical values.

5.-  Determination of the reactions generated when the bar is subjected to a load.

6.-  Determination of the bending moments generated when the bar is subjected to a load.

7.-  Determination of the angle of rotation generated when the bar is subjected to a load.

Optional

MFGE/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Data Acquisition System and Sensors:

For being used with mechanical units.

BDAS is designed to monitor the measurements of each mechanical unit from a computer.