TENDER SPECIFICATIONS

Supply and consumption at 12 VDC.

Supply and consumption in alternating current (AC).

Photovoltaic module:

Solar panel (polycrystalline) mounted on an anodized aluminum structure with wheels for mobility, and with calibrated cell to measure solar irradiation.

It consists of thirty six high performance photovoltaic cells (35 x 55 mm), with a typical power of 50 Wp for a 17 VDC voltage.

Both the protections and the used materials give it water proof properties, abrasion protection, hail impact protection and several other adverse environmental factors protection.

Technical data:

Maximum nominal power: 66 W. Voltage at maximum power point (Vmpp): 17.8 V.

Current at maximum power point (Impp): 3.70 A.

Short-circuit current (Isc): 4.05 A. Open circuit voltage (Voc): 22.25 V.

Dimensions: 660 x 35.5 x 780 mm. Weight: 3 Kg approx.

Lamp module:

Composed by one lamp to simulate the sunlight. Regulated Power: 0 - 400 W.

Battery offering optimal performance with low power applications.

Set of interconnection cables.

Rack for modules allocation:

Modules:

N-ES10.  Solar charge controller with an automatic recognition for operating voltage 12 V or 24 V. It monitors several parameters such as voltage, current and charge level of the battery, load current, status, etc. Additional functions can be activated such as the settings, night light function and auto-test. The regulator is equipped with various devices to protect its electronics, battery and load.

N-ES20. 12 VDC Load Module.

Supply voltage: 12 VDC.

Two lamps.

Two switches.

N-ES30.  DC/AC inverter that outputs a sinewave shaped output of 230 V / 50 Hz ± 2% and the nominal input voltage is 12 VDC. Two different operating modes: continuous mode and ASB mode (Auto Standby) to reduce the power consumption. It is provided with a diagnosis system to indicate the user the status by different flash sequences.

N-ES40.  AC Voltage measurements module until 250 V and DC until 250 V (digital multimeter).

N-ES50.  Loads module that incorporates two lamps of 220 V, 50 W, with independent switches.

N-ES80.  Module for measurements of solar irradiation (W/m2) and measurements of current until 10 A, with digital multimeter.

N-ES90.  Module for 12 VDC battery charger.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Rack with modules:  - Dimensions: 645 x 325 x 925 mm approx. (25.39 x 12.79 x 36.41 inches approx.) - Weight: 10 Kg approx. (22.04 pounds approx.)

Photovoltaic module: - Dimensions: 730 x 510 x 1150 mm approx. (28.74 x 20.07 x 45.27 inches approx.) - Weight: 10 Kg approx. (22.04 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Determination of the constituent material of the solar cell.

2.- Determination of the I-V first quadrant curve without illuminating the solar cell.

3.- Determination of the inverse (or saturation) current of the cell without illumination.

4.- Determination of parallel and series resistance of a solar cell without illumination.

5.- Dependency of the open circuit voltage (Voc) with lumens (luminous flux).

6.- Determination of the parameters that describe the quality of a solar cell.

7.- Solar energy measurement.

8.- Measurement of the solar panel voltage with no load.

9.- Determination of the disposition of cells in a solar panel.

10.- Familiarisation with the regulator parameters.

11.- Loads connection to 12 VDC.

12.- Loads connection to 220 VAC.

13.- Battery charging.

14.- Practices of serial and parallel panel configuration (minimum two MINI-EESF/M units).

Optional

MINI-EESF/M/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.