TENDER SPECIFICATIONS

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Solar panel (thermal solar collector):

Metallic structure.

Solar panel, is made of polycarbonate, with polypropylene pipes.

Pipes (already set) to connect the panel and the tank.

Temperature sensors, “J“ type.

Tank of 30 l.

Solar simulator:

Aluminum structure with adjustable height.

Two solar spectrum lamps of 300 W each one.

Pumping system:

Pump, range: 0 – 2  l/min.

Flowmeter.

Five temperature sensors, “J“ type, at different points of the unit.

The unit includes two “Venetian” type blinds to reduce a direct visual contact with the lamps and to reduce the direct contact with the solar panel when the unit is working.

Electronic console:

Metallic box.

Connections for the temperature sensors.

Digital display for the temperature sensors.

Selector for the temperature sensors.

Pump switch.

Lamps switch.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Unit: - Dimensions: 1300 x 800 x 1500 mm approx. (51.18 x 31.49 x 59.05 inches approx.) - Weight: 70 Kg approx. (154 pounds approx.)

Electronic console: - Dimensions: 490 x 330 x 310 mm approx. (19.29 x 12.99 x 12.20 inches approx.) - Weight: 10 Kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the thermosiphon operation.

2.- Study of the luminosity profile of the lamps.

3.- Study of the efficiency of the solar panel.

4.- Study of the influence of the inclination angle of the lamp panel on the unit efficiency.

5.- Study of the relation between the flow and the temperature.

6.- Study of the energy balance of the solar collector.

7.- Experimental efficiency determination.

- Several other exercises can be done and designed by the user.

Optional

MINI-EEST/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.