TENDER SPECIFICATIONS

Bench-top unit with adjustable legs.

Anodized aluminum frame and panels made of painted steel.

Diagram in the front panel with distribution of the elements similar to the real one.

The wheel, worm and pulleys are mounted on ball bearings.

The “MTSF” unit mainly consists of:

Wheel:

Material: bronze.

Module: 4 mm.

Worm:

Material: steel.

Module: 4 mm.

Number of gears: 1.

Power transmission: 7.5.

The worm and wheel pair has a transmission ratio of 1:30.

Two pulleys:

Material: aluminum.

One pulley is located on the worm shaft (diameter: 30 mm) to help to exert the effort to raise a load and the other pulley on the wheel shaft (diameter: 120 mm) to load the system.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Required elements (Not included):

- SET C. Steel Hook and Mass Set 3 kg. (2 units)

Each “type B” set includes:

1 Weight of 1000 gr (2.20 pounds approx.)

2 Weights of 500 gr (1.10 pounds approx.)

2 Weights of 200 gr (0.44 pounds approx.)

6 Weights of 100 gr (0.22 pounds approx.)

1 Support hook of 500 gr (1.10 pounds approx.)

- Dimensions: 560 x 510 x 1000 mm approx. (22,04 x 20,07 x 39.37 inches approx.) - Weight: 20 Kg approx. (44 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of the correlations and key variables of a worm gear.

2.- Investigation of torque transmission, efficiency and friction.

3.- Determination of the gear ratio of a worm gear unit.

Additional practical possibilities:

4.- Study of the basic terminology relating to gears (number of teeth, reference circle and module).

5.- Determination of torque, efficiency and friction with load variation.

Optional

MTSF/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.