TENDER SPECIFICATIONS

Bench-top unit with adjustable legs.

Anodized aluminum frame and panels in painted steel.

The “MUP” unit is mainly composed of:

Two porticoes:

A 1 m height portico, where the test bars are placed. It is designed to accommodate bars of different height by using two pins. Besides, at the upper and lower parts there are special clamps to subject the bar to the desired methods of support.

A 50 cm height portico, where the test bar will be subjected to load by turning a hand operated spindle, which will tend to elevate the lower support, so the test bar will be subjected to compression.

Regulation nut and bubble level that will allow to equilibrate the horizontal lower supports.

Digital balance to measure the bar compression load:

Range: 0 – 50 kg.

Accuracy: 100 g.

Weight: 2 kg.

Dial gauge to check the bar flexion as the test bar is subjected to load:

Range: 0 – 20 mm.

Accuracy: 0.01 mm.

Accessory to create disturbing load over the test bar, which consists of support and pulley.

Set of test bars, composed of:

Fifteen bars of rectangular section of different lengths and different methods of support.

Fifteen bars of circular section of different lengths and different methods of support.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

Required elements (Not included):

- SET A. Brass Hook and Mass Set 300 g.

Each “A type” set includes:

5 weights of 50 gr. (0.11 pounds).

4 weights of 10 gr. (0.022 pounds).

1 support hook (hanger) of 50 gr. (0.11 pounds).

Dimensions: Unit: 800 x 500 x 1200 mm approx. (31.49 x 19.68 x 47.24 inches approx.). Weight: 65 Kg approx. (143 pounds approx.)

Case for accessories: 350 x 300 x 150 mm approx. (13.78 x 11.81 x 5.9 inches approx.).

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of Euler buckling theory.

2.- Experimental determination of the relation between the buckling load for different thickness and shapes of the test bars.

3.- Experimental determination of the relation between the buckling load and the methods of support of the test bars.

4.- Determination of the critical load differences for different lengths and different bar sections.

5.- Measurement of lateral deflection.

Optional

MUP/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Data Acquisition System and Sensors:

For being used with mechanical units.

BDAS is designed to monitor the measurements of each mechanical unit from a computer.