TENDER SPECIFICATIONS

The main structure of the unit is a rigid frame, made of steel and aluminum, with two vertical guides, an upper mounting plate for the spring and a lower mounting plate for the damper. This rigid frame supports the different elements of the unit.

The mass carriage, to which various slotted weights may be attached, is constrained by rollers which run along the vertical guides to provide a single degree of freedom with minimum uncontrolled damping. The lower end of the spring is attached to the mass carriage and the upper end is attached to the frame. This section of the frame is adjustable so that the free position of the carriage may be varied.

The system produces amplitude versus time recordings using a mechanical strip chart recorder that consists of a drum recorder and a pen holder. The drum recorder is attached to the rigid frame and consists of a drum, driven by a synchronous motor, and a roll of paper. Before being rolled around the drum, the paper passes through a tensioning device that provides enough tension to ensure that the paper speed is constant (0.02 m/s). The pen holder is attached to the mass carriage and uses a spring to maintain continuous contact between the pen tip and the paper on the drum.

An electronic console is used to switch the synchronous motor on and off during experiments.

Three springs, with varying stiffness, can be interchanged as the connection between the rigid frame and the mass carriage:

Spring 1: k = 3.30 kN/m.

Spring 2: k = 1.22 kN/m.

Spring 3: k = 0.47 kN/m.

Five weights of 1 Kg each can be added and secured to the mass carriage.

An adjustable oil damper provides controlled damping and can be attached to the carriage by means of a screw.

Oil is provided to fill the damper.

Cables and accessories, for normal operation.

Manuals: This unit is supplied with the following manuals: Required services, Assembly and Installation, Start-up, Safety, Maintenance and Practice manuals.

Unit: - Dimensions: 530 x 560 x 1030 mm approx. (20.86 x 22.04 x 40.55 inches approx.) - Weight: 25 Kg approx.  (55.11 pounds approx.)

Electronic console: - Dimensions: 300 x 190 x 130 mm approx. (11.81 x 7.48 x 5.11 inches approx.) - Weight: 2 Kg approx. (4.40 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- 	Investigation of the relationship between the mass of the body, the stiffness of the spring and the period/frequency of oscillation for a simple spring mass system with one degree of freedom.

2.- Investigation of the relationship between the applied force, the viscosity of the oil and the velocity for various settings of the adjustable oil damper.

3.- Observation of how varying the degree of damping affects the response of a second order mechanical system to a step input.

4.- Observation of the free vibrations of a system having one degree of freedom.

5.- Study of the effect of viscous damping on the free vibrations of a simple spring-mass-damper system.

6.- Determination of the damping ratio for a given spring-massdamper system.

Optional

MVL/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

BDAS. Data Acquisition System and Sensors:

For being used with mechanical units.

BDAS is designed to monitor the measurements of each mechanical unit from a computer.