TENDER SPECIFICATIONS

Circuit blocks:

Transformer. (Circuit#1).

Half wave rectifier. Full wave rectifier, center tap. (Circuit#2).

Full wave rectifier. (Circuit#3).

Filtering. (Circuit#4).

Zener limiting.(Circuit#5).

Regulation. (Circuit#6).

Overcurrent protection. (Circuit#7).

Overvoltage protection. (Circuit#8).

Voltage regulators. (Circuit#9).

LM317 adjustable regulator. (Circuit#10).

L200 adjustable regulator. (Circuit#11).

Switched source. (Circuit#12).

PWM switched source. (Circuit#13).

Boost switched source. (Circuit#14).

Load.

Channels.

Required elements (only one) (Not included):

- FACO. Power Supply.

DC Fixed outputs: + 5 V, ± 12 V, 1 A.

DC Variable outputs: ± 12 V, 0.5 A.

AC Outputs: 12 V or 24 V.

Outputs through 2 mm terminals, or connectors Sub-D of 25 pins (2 outputs).

Voltage LED indicators.

Robust construction.

Power: 110/220 VAC

Frequency: 50/60 Hz.

or

- EBC100. Base Unit, with built-in power supply.

Hardware support and power supply.

Modules supporting unit.

DC Fixed outputs: + 5 V, + 12 V, -12 V.

DC Variable outputs: ± 12 V.

AC Outputs: 12 V or 24 V.

Outputs through either 2 mm contact terminals, or through 25 pins Sub-D connector.

Voltage LED indicators.

Robust construction.

Power: 110/220 VAC.

Frequency: 50/60 Hz.

Additional recommended elements (only one) (Not included):

- EDAS/VIS-0.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 250,000 samples/s).

or

- EDAS/VIS-1.25. EDIBON Data Acquisition System and Virtual Instrumentation (speed: 1,250,000 samples/s).

Dimensions: 300 x 210 x 45 mm approx. (11.81 x 8.26 x 1.77 inches approx.). Weight: 300 g approx. (0.66 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

Rectification:

1.-  Rectification.

2.-  Bridge rectifier.

3.-  Theoretical/practical exercises.

Fixed voltage sources:

4.-  Power supply with the Zener diode.

5.-  Stabilization through Zener and Transistor.

6.-  Faults study in “Stabilization through Zener and Transistor”.

7.-  Protection against overcurrents.

8.-  Protection against overvoltages.

9.-  Faults study of “Protection against overcurrents”.

10.-Theoretical/practical exercises.

Symmetrical voltage power sources:

11.-Symmetrical source; 78XX regulator.

12.-Symmetrical source; 79XX regulator.

13.-Theoretical/practical exercises.

Voltage regulators with integrated circuits:

14.-Adjustable regulator; LM317.

15.-Faults study in adjustable LM317 regulator.

16.-Adjustable L200 regulator.

17.-Faults study in adjustable L200 regulator.

18.-Theoretical/practical exercises.

Introduction to switched power supplies:

19.-Switching technique.

20.-Switching technique. PWM.

21.-Switching technique. Boost.

22.-Theoretical/practical exercises.

Additional practical possibilities:

23.-Voltage Feedback.

24.-DC-DC converter.

-Several other exercises can be done and designed by the user.

Optional

M5/N-M5/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.