Tender Specifications (for main items)

1 PBOC. Unit:

Unit designed to demonstrate the operating characteristics of several types of pumps.

Anodized aluminum frame and panels made of painted steel.

The unit includes wheels to facilitate its mobility.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Fully instrumented self-contained unit.

The unit is mounted on a structure with a work surface covered by a plastic sheet.

Four pumps (computer controlled):

Centrifugal Pump: pedestal or of free axis type, made of brass.

Maximum flow: 80  l/min.

Maximum height: 20 m.w.c. (meters of water column).

Efficiency: 35%.

Axial Pump: with propeller, which works in an acrylic casing with thin interstices between the propeller and the casing.

Maximum flow: 50  l/min.

Maximum height: 2 m.w.c. (meters of water column).

Efficiency: 15%.

Gear Pump: of positive displacement, with casing of a melted piece and two rotors in form of a straight cylindrical gear.

Maximum flow: 35  l/min.

Maximum height: 45 m.w.c. (meters of water column).

Efficiency: 75%.

Peripheral Pump: also known as regenerator or turbine pump, with a runner of straight blades inside an annular casing and an axis of activation on two lubricated ball bearings.

Maximum flow: 40  l/min.

Maximum height: 22 m.w.c. (meters of water column).

Efficiency: 30%.

Motor for each pump, with independent operating.

Variation of speed by frequency converter, located in the control interface box, computer controlled.

There are two pressure tappings in each pump, one at the admission and other at the discharge of the pump.

Calibrated volumetric tank of 0 – 10  l for low flows and of 0 – 45  l for high flows. It includes a level sensor (capacitive, length: 300 mm) to determine the water flow and the water level in the volumetric tank, an indicating transparent tube and a graduated scale.

Water storage tank, with capacity of 160  l approx. It includes an indicating transparent tube.

Two “U” shape weirs.

Two stilling baffles.

Valves for centrifugal, peripherical and gear pumps.

Control valve for axial pump.

Eight admission pressure sensor and discharge pressure sensor for each pump.

Reading of torque (Nm) of the pump.

The software can determine the pump manometric height, the hydraulic and mechanic power and the efficiency.

The complete unit includes as well:

Advanced Real-Time SCADA.

Open Control + Multicontrol + Real-Time Control.

Specialized EDIBON Control Software based on LabVIEW.

National Instruments Data Acquisition board (250 KS/s, kilo samples per second).

Calibration exercises, which are included, teach the user how to calibrate a sensor and the importance of checking the accuracy of the sensors before taking measurements.

Projector and/or electronic whiteboard compatibility allows the unit to be explained and demonstrated to an entire class at one time.

Capable of doing applied research, real industrial simulation, training courses, etc.

Remote operation and control by the user and remote control for EDIBON technical support, are always included.

Totally safe, utilizing 4 safety systems (Mechanical, Electrical, Electronic & Software).

Designed and manufactured under several quality standards.

Optional ICAI software to create, edit and carry out practical exercises, tests, exams, calculations, etc. Apart from monitoring user's knowledge and progress reached.

This unit has been designed for future expansion and integration. A common expansion is the EDIBON Scada-Net (ESN) System which enables multiple students to simultaneously operate many units in a network.

Recommended elements: (Not included)

- PBOC-2BC. Second Centrifugal Pump.

- PBOC-BIF. Flexible Impeller Pump.

- PBOC-BD. Diaphragm Pump.

- PBOC-BE. Plunger Pump.

- PBOC-VA. Vane Pump.

2 PBOC/CIB. Control Interface Box:

The Control Interface Box is part of the SCADA system.

Control interface box with process diagram in the front panel.

The unit control elements are permanently computer controlled.

Simultaneous visualization in the computer of all parameters involved in the process.

Calibration of all sensors involved in the process.

Real time curves representation about system responses.

All the actuators’ values can be changed at any time from the keyboard allowing the analysis about curves and responses of the whole process.

Shield and filtered signals to avoid external interferences.

Real time computer control with flexibility of modifications from the computer keyboard of the parameters, at any moment during the process.

Real time computer control for parameters involved in the process simultaneously.

Open control allowing modifications, at any moment and in real time, of parameters involved in the process simultaneously.

Three safety levels, one mechanical in the unit, another electronic in the control interface and the third one in the control software.

3 DAB. Data Acquisition Board:

The Data Acquisition board is part of the SCADA system.

PCI Express Data acquisition board (National Instruments) to be placed in a computer slot.

Analog input: Channels= 16 single-ended or 8 differential. Resolution=16 bits, 1 in 65536. Sampling rate up to: 250 KS/s (kilo samples per second).

Analog output: Channels=2. Resolution=16 bits, 1 in 65536.

Digital Input/Output: Channels=24 inputs/outputs.

4 PBOC/CCSOF. Computer Control +Data Acquisition+Data Management Software:

The three softwares are part of the SCADA system.

Compatible with the industry standards.

Flexible, open and multicontrol software, developed with actual windows graphic systems, acting simultaneously on all process parameters.

Management, processing, comparison and storage of data.

Sampling velocity up to 250 KS/s (kilo samples per second).

Calibration system for the sensors involved in the process.

It allows the registration of the alarms state and the graphic representation in real time.

Open software, allowing the teacher to modify texts, instructions. Teacher’s and student’s passwords to facilitate the teacher’s control on the student, and allowing the access to different work levels.

This unit allows the 30 students of the classroom to visualize simultaneously all the results and the manipulation of the unit, during the process, by using a projector or an electronic whiteboard.

5 Cables and Accessories, for normal operation.

6 Manuals:

This unit is supplied with 8 manuals: Required Services, Assembly and Installation, Interface and Control Software, Starting-up, Safety, Maintenance, Calibration & Practices Manuals.

*References 1 to 6 are the main items: PBOC + PBOC/CIB + DAB + PBOC/CCSOF + Cables and Accessories + Manuals are included in the minimum supply for enabling normal and full operation.

EXERCISES AND PRACTICAL POSSIBILITIES TO BE DONE WITH THE MAIN ITEMS

1.-  Determination of the flow by a weir of thin wall in U-shape.

2.-  Determination of discharge coefficient of a weir of thin wall in U-shape.

3.-  Determination of the curve Q vs r.p.m. of the centrifugal pump.

4.-  Determination of the curve Q vs r.p.m. of the peripherical pump.

5.-  Determination of the curve Q vs r.p.m. of the gear pump.

6.-  Determination of the curve Q vs r.p.m. of the axial pump.

7.-  Determination of the curve H vs Q for different r.p.m. of the centrifugal pump.

8.-  Determination of the curve H vs Q for different r.p.m. of the peripherical pump.

9.-  Determination of the curve H vs Q for different r.p.m. of the gear pump.

10.-Determination of the curve H vs Q for different r.p.m. of the axial pump.

11.-Determination of the mechanical power vs flow for different r.p.m. of the centrifugal pump.

12.-Determination of the mechanical power vs flow for different r.p.m. of the peripherical pump.

13.-Determination of the mechanical power vs flow for different r.p.m. of the gear pump.

14.-Determination of the mechanical power vs flow for different r.p.m. of the axial pump.

15.-Determination of the curve η vs the flow for different r.p.m. of the centrifugal pump.

16.-Determination of the curve η vs the flow for different r.p.m. of the peripherical pump.

17.-Determination of the curve η vs the flow for different r.p.m. of the gear pump.

18.-Determination of the curve η vs the flow for different r.p.m. of the axial pump.

19.-Determination of the map of a centrifugal pump.

20.-Determination of the map of a peripherical pump.

21.-Determination of the map of a gear pump.

22.-Determination of the map of an axial pump.

23.-Determination of the adimensional characteristic curves for different types of pumps.

24.-Determination of the specific speed of different types of pumps.

25.-Verification of the similarity rules for pumps of different geometry.

Additional practical possibilities:

26.-Sensors calibration.

Additional practical possibilities to be done with the Optional Models and Accessories: (Not included)

Second Centrifugal Pump (PBOC-2BC):

27.-Coupling in series of two centrifugal pumps of different characteristics.

28.-Coupling in series of two centrifugal pumps with the same characteristics.

29.-Parallel coupling of two centrifugal pumps with similar characteristics.

30.-Parallel coupling of two centrifugal pumps of different characteristics.

Flexible Impeller Pump (PBOC-BIF):

31.-Determination of the curve Q vs r.p.m. of the flexible impeller pump.

32.-Determination of the curve H vs Q for different r.p.m. of the flexible impeller pump.

33.-Determination of the mechanical power vs flow for different r.p.m. of the flexible impeller pump.

34.-Determination of the curve η vs the flow for different r.p.m. of the flexible impeller pump.

35.-Determination of the map of a flexible impeller pump.

Diaphragm Pump (PBOC-BD):

36.-Determination of the curve Q vs r.p.m. of the diaphragm pump.

37.-Determination of the curve H vs Q for different r.p.m. of the diaphragm pump.

38.-Determination of the mechanical power vs flow for different r.p.m. of the diaphragm pump.

39.-Determination of the curve η vs the flow for different r.p.m. of  the diaphragm pump.

40.-Determination of the map of a diaphragm pump.

Plunger Pump (PBOC-BE):

41.-Determination of the curve Q vs r.p.m. of the plunger pump.

42.-Determination of the curve H vs Q for different r.p.m. of the plunger pump.

43.-Determination of the mechanical power vs flow for different r.p.m. of the plunger pump.

44.-Determination of the curve η vs the flow for different r.p.m. of the plunger pump.

45.-Determination of the map of a plunger pump.

Vane Pump (PBOC-VA):

46.-Determination of the curve Q vs r.p.m. of the vane pump.

47.-Determination of the curve H vs Q for different r.p.m. of the vane pump.

48.-Determination of the mechanical power vs flow for different r.p.m. of the vane pump.

49.-Determination of the curve η vs the flow for different r.p.m. of the vane pump.

50.-Determination of the map of a vane pump.

Other possibilities to be done with this Unit:

51.-Many students view results simultaneously.

To view all results in real time in the classroom by means of a projector or an electronic whiteboard.

52.-Open Control, Multicontrol and Real Time Control.

This unit allows intrinsically and/or extrinsically to change the span, gains; proportional, integral, derivative parameters; etc, in real time.

53.-The Computer Control System with SCADA allows a real industrial simulation.

54.-This unit is totally safe as uses mechanical, electrical/electronic, and software safety devices.

55.-This unit can be used for doing applied research.

56.-This unit can be used for giving training courses to Industries even to other Technical Education Institutions.

57.-Control of the PBOC unit process through the control interface box without the computer.

58.-Visualization of all the sensors values used in the PBOC unit process.

- By using PLC-PI additional 19 more exercises can be done.

- Several other exercises can be done and designed by the user.

Tender Specifications (for optional items)

a) Industrial configuration

7 PLC. Industrial Control using PLC (it includes PLC-PI Module plus PLC-SOF Control Software):

-PLC-PI. PLC Module:

Metallic box.

Circuit diagram in the module front panel.

Digital inputs (X) and Digital outputs (Y) block: 16 Digital inputs. 14 Digital outputs.

Analog inputs block: 16 Analog inputs.

Analog outputs block: 4 Analog outputs.

Touch screen.

Panasonic PLC:

High-speed scan of 0.32 µsec. Program capacity of 32 Ksteps. High-speed counter. Multi-point PID control.

Digital inputs/outputs and analog inputs/outputs Panasonic modules.

-PBOC/PLC-SOF. PLC Control Software:

For this particular unit, always included with PLC supply.

Practices to be done with PLC-PI:

1.-  Control of the particular unit process through the control interface box without the computer.

2.-  Visualization of all the sensors values used in the particular unit process.

3.-  Calibration of all sensors included in the particular unit process.

4.-  Hand on of all the actuators involved in the particular unit process.

5.-  Realization of different experiments, in automatic way, without having in front the particular unit. (These experiments can be decided previously).

6.- Simulation of outside actions, in the cases do not exist hardware elements. (Example: test of complementary tanks, complementary industrialenvironment to the process to be studied, etc).

7.-  PLC hardware general use.

8.-  PLC process application for the particular unit.

9.-  PLC structure.

10.-PLC inputs and outputs configuration.

11.-PLC configuration possibilities.

12.-PLC program languages.

13.-PLC different programming standard languages (ladder diagram (LD), structured text (ST), instructions list (IL), sequential function chart (SFC), function block diagram (FBD)).

14.-New configuration and development of new process.

15.-Hand on an established process.

16.-To visualize and see the results and to make comparisons with the particular unit process.

17.-Possibility of creating new process in relation with the particular unit.

18.-PLC Programming Exercises.

19.-Own PLC applications in accordance with teacher and student requirements.

b) Technical and Vocational Education configuration

8 PBOC/ICAI. Interactive Computer Aided Instruction Software System.

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc...so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.

9 PBOC/FSS. Faults Simulation System.

Faults Simulation System (FSS) is a Software package that simulates several faults in any EDIBON Computer Controlled Unit.

The "FAULTS" mode consists in causing several faults in the unit normal operation. The student must find them and solve them.

There are several kinds of faults that can be grouped in the following sections:

Faults affecting the sensors measurement:

- An incorrect calibration is applied to them.

- Non-linearity.

Faults affecting the actuators:

- Actuators channels interchange at any time during the program execution.

- Response reduction of an actuator.

Faults in the controls execution:

- Inversion of the performance in ON/OFF controls.

- Reduction or increase of the calculated total response.

- The action of some controls is annulled.

On/off faults:

- Several on/off faults can be included.

c) Multipost Expansions options

10 MINI ESN. EDIBON Mini Scada-Net System for being used with EDIBON Teaching Units.

MINI ESN. EDIBON Mini Scada-Net System allows up to 30 students to work with a Teaching Unit in any laboratory, simultaneously.

The MINI ESN system consists of the adaptation of any EDIBON Computer Controlled Unit with SCADA integrated in a local network.

This system allows to view/control the unit remotely, from any computer integrated in the local net (in the classroom), through the main computer connected to the unit.

Main characteristics:

- It allows up to 30 students to work simultaneously with the EDIBON Computer Controlled Unit with SCADA, connected in a local net.

- Open Control + Multicontrol + Real Time Control + Multi Student Post.

- Instructor controls and explains to all students at the same time.

- Any user/student can work doing "real time" control/multicontrol and visualisation.

- Instructor can see in the computer what any user/student is doing in the unit.

- Continuous communication between the instructor and all the users/students connected.

Main advantages:

- It allows an easier and quicker understanding.

- This system allows you can save time and cost.

- Future expansions with more EDIBON Units.

The system basically will consist of:

This system is used with a Computer Controlled Unit.

- Instructor’s computer.

- Students’ computers.

- Local Network.

- Unit-Control Interface adaptation.

- Unit Software adaptation.

- Webcam.

- MINI ESN Software to control the whole system.

- Cables and accessories required for a normal operation.