TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame and panels made of painted steel.

Main metallic elements made of stainless steel.

Diagram in the front panel with distribution of the elements similar to the real one.

Three transparent graduated cylinders:

External diameter:  100 mm.

Height: 600 mm.

With a permeable perforated plate at the bottom, where a filter to disable the soil introduced to be swept by the water is placed.

Tank to collect the water and the smaller soil particles under study:

Capacity: 25  l.

It has a drain valve.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions:  500 x 400 x 900 mm approx. (19.68 x 15.75 x 35.43 inches approx.). Weight: 30 Kg approx. (66 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Determination of the principles of the relationship between the type of soil and infiltration and penetration rates.

2.-  Comparison of the cumulative infiltration, infiltration rate and penetration depth as a function of time in different types of soil.

3.-  Study of the empirical equations as an approximation of infiltration rate.

4.-  Visualization of the effect of crusting on infiltration.

5.-  Visualization of the effect of soil particle size on infiltration.

Additional practical possibilities:

6.-  Study of the effect of organic matter content on the infiltration and penetration rate.

7.-  Study of the effect of non-homogeneous soil strata on infiltration and penetration rates.

8.-  Study of the effect of moisture content on the infiltration and penetration rate.

9.-  Study of the effect of straw mulch on infiltration rate.

10.-Study of the effect of soil texture and structure on infiltration.

11.-Study of the effect of surface on infiltration.

Optional

PEDI/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.