TENDER SPECIFICATIONS

Portable flue gas analyzer with metal carrying case for heating systems.

It can work either with rechargeable batteries or connected to the mains. High performance rechargeable battery for 6 hours non-stop operation.

Backlit display for good visibility in poor light applications.

Probe for flue gases with soot filter.

Condensate trap and particles filter.

Eight preprogrammed fuels and five programmable fuels.

Measurements:

Combustion temperature.

Flue gases temperature.

Residual oxygen. Range: 0 - 20.95 Vol. %.

Carbon monoxide. Range: 0 - 2000 ppm.

Carbon dioxide. Range: depends on fuel.

Nitrogen oxides. Range: 0 - 2000 ppm.

Draft pressure. Range: ± 60 hPa.

Calculation of flue gas losses. Range: 0 - 99.9%.

Calculation of the combustion efficiency. Range: 0 - 99.9%.

Excess air calculation. Range: 1.0 - 99.9%.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

Dimensions: 420 x 190 x 213 mm approx.  (16.53 x 7.48 x 8.38 inches approx.)   Weight: 7 Kg approx.  (15 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Measurement of the residual oxygen in flue gases.

2.- Measurement of the carbon monoxide and carbon dioxide.

3.- Measurement of nitrogen oxides.

4.- Measurement of the flue gases temperature and the combustion temperature.

5.- Combustion efficiency analysis.

6.- Excess air analysis.