TENDER SPECIFICATIONS

Bench-top unit.

Anodized aluminum frame.

Main metallic elements made of stainless steel.

Loading device with spindle and dynamometer for compressive and tensile load. Max load force: 250 N.

Polarization filter and quarter-wave filter properly protected.

Filter for monochromatic light.

Possibility of linear or circular polarization.

Test specimens:

Bar test tube without notches.

Bar test tube with hole.

Bar specimen with notches on one side.

Bar specimen with notches on both sides.

Rectangular bar specimen without recesses.

Rectangular bar specimen with recesses.

Fork probe.

Crane hook test tube.

Manuals: This unit is supplied with the following manuals: Required Services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices Manuals.

- Dimensions: 500 x 200 x 150 mm approx. (19.68 x 7.87 x 5.90 inches approx.) - Weight: 10 Kg approx. (22 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.- Study of different plane stress states generated in different models subjected to compressive load.

2.- Study of different plane stress states generated in different models subjected to tensile loading.

3.- Study of stress distributions with linear or circularly polarized light.

4.- Investigation of the stress distribution by means of images obtained by photoelasticity.

5.- Investigation of stress concentration by means of images obtained by photoelasticity.

Optional

PSD/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.